Description of the L2STX by OE5DXL with DAMA                 10.12.1994

============================================

Translation by G0KIU @ GB7MAM

-----------------------------

Warning: The TNC-Command V will be used for diverse Installations on the

L2STX. Please observe Information further below!

Short History:. Chris OE5DXL known through the OE5DXL-Boxes with

Gateway, then later the Baycom Boxes were built and now, naturally

have made their own individual development including a new Digipeater-

Hardware. Also, because of faster Hardware it required a corresponding

Software, naturally something similar to that has been created. Up to

now all on the PC, only the Node Computer does not stand still. However

it occurs to me, that it lies already with the 68000 from Motorola.

Now, and presumably because it had stood around, a very old Atari ST

also still unused, meant that an Interface could be constructed to

which faster Hardware could be connected to a Module-Bus of the ST.

With long discussions the Idea was realised to make an Emulator, which

the faster HDLC-Card emulates by Software for a Port.

Now here were the two most important parts of the L2STX already firmly

finished. Now always appeared again Questions in the Boxes about the

usage of Baycom and Nurmodem on the ST. Now, because certain matters

which are specially concerned with the ST were conveyed, the question,

was thrown open, "Would it not be possible, to allow the L2 and the

Emulator to make a TFST??", now Chris took the time and programmed this

interface. With agreed advice from Joachim DL8HBS, how the DP works

with TFST, now arose the Software solution for Nurmodems. Because Chris

is one, that it does not remain dribbling at 1200 Baud, but here it

must run already at least at 9k6. Certainly and because Chris is also

an economical person and the outlay for a 9K6-Modem produces a 3 digit

amount, this has been inbuilt as a Software Solution.

So and similarly those to deter here, to say why are not all TNC-

Commands contained. The L2 in L2STX has to work with no known TNC-

Software, neither in the Construction nor has been engraved anywhere.

For this reason it is not possible to incorporate many Commands within

it, because here L2 occupies no counterpart for that purpose.

L2STX programs the MFP of the Atari ST and TT correspondingly. After

the start of the L2STX no more Programs should access on the serial

Port. L2STX runs in the Processor Interrupts. On the ST approximately

17 % of the Computer time will be used. (With 9k6 scrambled)

L2STX.TOS can be redesignated in L2STX.TTP, if the currently installed

Buffers appear too few or too little Memory Space is available. The

required Buffer Size is conveyed simply as a Number in the Command-

Line...

L2STX.PRG also runs in the Auto Order.

L2STX installs a XBRA-Recognition TFST and with that, can also work

together with SP, which claims a XBRA-Recognition.

L2STX has no individual CTEXT.

L2STX has a digital Squelch, for this reason a squelch must always be

open.

L2STX must be started before the Hostmode-Terminal-Program.

It runs with  DP and now also with SP.

Configuration in DP or SP as if TFST is allowed to run.

Therefore CFG=PORT0:5

CFG=TYPE0:TFST

TNC-Commands contain the following:

C        Connect Call(via Call) Port 0 UI-Path

D        Disconnect

F        FRAC 1-127

I        Mycall

L        Linkstatus

N        Retries 1-32000 ! therefore Caution!

M        0 or 1 off/on

O        Maxframes 1-7

R        0 or 1 Digipeat on/off.

T        Txdel value x 10 msec caution also goes 5 Minutes

W        Chance Time sends from 0 - W x 10 msec. see below!

Y        Maxlinks 0-30

@ B      Number of free Buffers

@ T      T3 with TF but in Seconds

@ Ipoll  Ipoll...

All other cases are not to be changed. L2  does that dynamically.

Naturally the L2STX works with a Frame Collector, as all Developments

by Chris.

Now still an important Parameter:

V=1200     1200 Baud normal

V=9600     9600 Baud normal V=9601 9k6 scrambled as G3RUH

V= baud    300-19200 All possible Values unscrambled Baud Rates.

Scrambled simply Baud

+ 1

V=1       Test Operation

V=2            "

V=3            "

V=4            "

V=5       Pin 20 on minus

V=6       Pin 20 is plus

!!! Also set the Command CFG=V2 in the PRCONF.DP through CFG=VBaud !!!!

Further Entries are possible, but they should not mutually exclude.

These 1-4 will be used in order to send high or low, with V=3 a

010101010101010 Bit Test will be sent with FSK. Therefore Ideal in

order to install Frequency and the amplitude with DC-Coupling.

V5 and V6 are for the Modems, which the Power Supply obtains from the

RS232. V5 switches the Pin 20 DTR of the RS232 on -12 Volt, V6 switches

on +12 Volt. The second Pin which will be used for Power Supply is Pin

4 RTS. For the AFSK-Modem both Pins will be supplied with +12 Volt and

both together are adequate for the current. With the FSK-Modem the 4-

section OP TL064 requires one + (Pin 4) and once again - Voltage (Pin

11 of the TL064). For this purpose Pin 20 of the RS232 will switched

from L2STX on the Command V5.

By using other faster OPs an external Voltage Supply can be chosen.

L2STX employs 8 different MyCalls, also 8 different SSIDs. But in order

to be able to be connected from an external source, at least 2 Ports

must be occupied, but the sequence is not defaulted. The basis lies in

dynamic management, therefore every Connect has no Port-Number as with

TNCs, but internal Addresses. With that can be 1000 Connections. Only

dependent on the  Buffer-Size.

So now for the required Hardware.

Because the ST has no free conductors from MFP to the Serial Port which

can be used for PTT, and the Programming of the Sound-Chips is not

possible in the Interrupts without the danger of a crash, another path

for the PTT will be made. In any case a Watchdog is required for a

Modem, in order to switch off the TX after a certain time with an error

Function of the Software. A 74HC04 will not be used as with the BayCom

Modem but a 74HC123.

These are 2 Edge-Triggering Mono-Flops with a Reset. As soon as Data

arrives from the COMPort switches the Mono-Flop through on to the 2nd.

and the time determining RC switches the PTT once, however after the

run out of the Watchdog Time the PTT is gone again. At the End of

the Data switches the PTT again onto receive.

On these Grounds the connection allocations are NOT identical with

BayCom Modems. Also the Data lines are otherwise occupied with Baycom.

The Voltage Supply will be placed by the RS232 for selection.

The circuit for both Modems here is here as ASCII-Drawing for essay and

airs 1st Class. I am working on a design construction.

A combined construction is planned, also AFSK and FSK-Modem on one PCB,

because the PTT- and Watchdog-Switching are identical and the type of

Current-Supply, it places a switching from the Software in expectation.

Component Costs with the FSK-Modem lie under 10 DM, with the AFSK-

Version somewhat higher, because the 3105 has its Price.

L2STX must run on all ST, STE and TT in all TOS-Versions,

Commentary from Chris OE5DXL:

Der W-Timer bei P=0:

===================

By means of a simple Statistical method it can be displayed, that in a

System without Time-Slots, then the Slot-Width has its Optimum, if it

goes against Null. With that arises the greatest possible Strewing of

the Send-Starts after DCD or maximum average Time-Distance, and with

that, least Collision-Likelihood. It should also equalise the Send-

Start from 0 up to a similar definable optimal Maximum-Value to strive

for best Data-Throughput with a given allocation situation. The optimum

Maximum-Value is expensive to generate and dynamic from the User-

Numbers and whose Dead-Time between no longer dependent on DCD reaction

and still no DCD generation. An appreciating Value in 10ms units can be

installed. W 100, also Send-Start from DCD equalised from 0 up to 1s is

an Example.

For DAMA-Tests:

===============

Start DAMA Mode: With the first Link with a DAMA-Master.

End DAMA Mode:  After 1 Min. if no Frame with DAMA-Bit has been

received. (Note: Who gets 1 Min no Send-Release, should then control

Receive or change the Digi)

Protocol alteration in the DAMA-Mode with the Slave:

On Send will be gone immediately to Receiving of an optional Frame

with DAMA-Bit, which is addressed to the individual TNC, with the

exception of I-Frames without Command+Poll-Bit.

Then all will be sent, which according to WB4JFI-Protocol is permitted

to Send, especially also on all other Links, also those to not-DAMA-

Masters and SABM for new Links.

Immediately after an associated Sending, the Send will be barred, up to

-see above.

With that Timer 1 will be controlled through the Send-Release, also

clocked from the Master.

The Poll/Final-Procedure of AX.25 Version 2 from WB4JFI is valid only

more for SABM/UA. The Send-Sequence-Number will be opposite to the

normal AX.25-Precept, returned not only with Receipt of R(N)R with

Response+Final-Bit or REJ, but with the receipt of every Frame, which

contains an Acknowledgement-Number. That increases the efficiency with

Frame-Repeat; because it gives no chronological overlap between Send

and Receive, it should arrive at no drawback in the Form of unrequired

Repeats. Also with that, the Frame Collector from DL4MEA-Procedure

works impeccably.

Then Info-Frames will be repeated only if in between an Acknowledgement

Number for this Link has been received, which the Send-Sequence-Number

has returned.

The Slave-Implementation also understands the (for the same in the L2

containing DAMA-Master is appropriate) Send-Release-Method from OE5DXL

by way of an RR-Command without an Idle-Answer from the Slave and with

that close upon double quick Idle-Link-Continue-Switching. As a near

effect multiple Collisions with SABM will be prevented by new Slaves

to the Master, the Stream-Account of the User decreases and increased

through fall out of the "nervous Polls" by the Acceptance of DAMA.

Much Fun and 55 & 73 de Sepp DH5RAE

Translation by G0KIU @ GB7MAM