Page  1.

TheNetNode

Created by Nord><Link

Issued 10th. September 1997

User Handbook for the Packet-Radio Node Software

TheNetNode (PC)/(GNU) V 1.74

Alterations compared to the Description of the Version TheNetNode 1.73

(Issued for the Annual General Meeting of the NORD><LINK 1997 in

Hannover)

The Files TNN170.xxx have been set against TNN174.xxx.

Translation by Denis G0KIU @ GB7MAM

(Please note that this translation has not been Proof Read. If you

discover any errors either Technical or Otherwise please drop a message

to me at GB7MAM. Thanks.

Page  2.

Now in TNN174.PAS ';' is used in addition to '#', ';', otherwise ALIAS

cannot be incorporated.

ESC I 1...6 is removed.

The Installation of the Node Calls SSID etc. is only possible via

TNN174.pas.

In TNN174.TNB <CALL> only able to be altered with ESC I.

RUNBATCH filename.tnb>. Allows e.g. TNB to be executed directly.

With the Interlink Setup, the input of the direct Neighbour will be

routed to the Interlink Setup. In the Connect-Text allows the actual

Date of the AKTUELL.TXT to be annexed and displayed. For this purpose a

%A is attached in the Connect Text following the Topic.

Version displays with which combination TNN.EXE was compiled. From this

Version, a 10 times HOST Interface is inbuilt. (Commands as with TF27)

Extracts for the PREVIOUS ESC E1/E0 have been used, now ESC @E1/ @E0

must be used. B and B+ for the MAILBOX have been removed. Mailbox will

be incorporated with L or L+.

Previously the Text Marking in TNN174.PAS has been marked with a #. Now

they will be set, additionally with a Semicolono. Basis: Previously it

was no longer possible to set #ALIAS. Now, independentally of a MAILBOX

at the individual Node, it allows a LOCAL Mailbox to be incorporated

with TNN174.TNB M <call>. M - erases the Entry. Certainly the largest

reorganisation is: TNN now works with Run times (except to old

Neighbours). Then the conversion to Qualities always follows, when no

Neighbour new Type has been recognised.

The Flexnet-Interface has been improved so much, that not only the

Destinations will be undertaken, but also forwarded other Flexnet-

Neighbours    .

SET TNNCFG=<path> is removed.

SET TNNCFG=<1.Number>,<2.Number>  :

1. Number is the size of the Node - / Destination List

2. Number is the Length of the Link list (Parameter 1 removed).

ALL <ESC>  - Commands now allow input from remote.

e.g. <ESC> T will be entered as ESC T from remote.  Now it must be

incorporated in TNN174.TNB.

Entry N=. Is recognised under Links also when, the Neighbour supports

the new Router of TheNetNode. Now N- is for the Neighbour which only

supports the NetRom - Protocol (old Router).

The Function AutoDigi is removed. They have all been combined under

L2Digi.

N <  <Neighbour - Call> shows all Nodes/Destinations which are

reported from these Neighbours as faster paths.

Page  3.

User Handbook for the Packet-Radio Node Software

------------------------------------------------

TheNetNode (PC) V 1.74

----------------------

Reprint with Source Output allowed for non-Commercial Purposes.

--------------------------------------------------------------

INDEX  "3, 4 & 5"

-----------------

ALAS General Licence for  Amateur Radio Software ...... 127

<ALT> X The Special Combination ........................ 36

APPENDIX A: ............................................ 82

APPENDIX B: ............................................ 83

APPENDIX C: ............................................ 87

APPENDIX D: ............................................ 97

APPENDIX E: ........................................... 101

APPENDIX F: ........................................... 105

APPENDIX G: ........................................... 112

APPENDIX H: ........................................... 117

APPENDIX I: ........................................... 124

(ARP) .................................................. 25

AUTOEXEC.BAT ........................................... 17

(BE)ACON ............................................... 45

(BE)ACON ............................................... 22

(BE)ACON Sysop ......................................... 21

(CL)EAR ................................................ 22

Commands for all Users ................................. 45

Common Paramaters for all Ports ........................ 59

Computer Configuration ................................. 15

(C)ONNECT .............................................. 45

Construction of TNN174.PAS-File ........................ 19

(CONV)ers .............................................. 22

(CONV)ers and its Commands ............................. 46

(CPERS) ................................................ 41

(CQ) ................................................... 53

(CWER) ................................................. 41

DAMA - a new procedure for Packet-Radio ! ............. 117

DAMA ...Traffic control through the Nodes .............. 13

(D)EST ................................................. 54

(DOS) <Command> ........................................ 22

DPMI or usage of the most empty lying Memory above 1MB . 11

(E)DIT ................................................. 24

(Ethernet) ............................................. 97

<ESC> The various Keys ................................. 36

Ethernet-Port .......................................... 11

EXTERNAL Commands for all Users ........................ 76

External Driver ......................................... 8

EXTERNAL Programs for all Users ........................ 76

EXTERNAL PROGRAM/COMMANDS for the SYSOP: ............... 40

Forward ................................................. 5

Frames Layer 3/4 Construction of ...................... 105

GNU-Version of TNN .................................... 112

(H)ELP ................................................. 54

Page  4.

(H)elp ................................................. 81

Introduction............................................. 2

(IPA)dress ............................................. 24

(IPB)roadcast .......................................... 24

(IPR)oute .............................................. 24

(K)ill ................................................. 27

Kisslink ................................................ 7

KISS-Software for the TNC .............................. 84

LICENCE: .............................................. 127

(L)INKS ................................................ 27

(L)INKS ................................................ 55

(LOA)D <filename.ext> .................................. 28

(M)ailbox .............................................. 55

(MAKEDAT.EXE) .......................................... 44

(MH)EARD ............................................... 29

(MH)EARD ............................................... 55

(MSG) .................................................. 77

(MSY) .................................................. 43

(N)ODES ................................................ 56

(O)utput ............................................... 40

PACSAT-BROADCAST-SERVER of TNN ........................ 101

(P)ARAMETER ............................................ 59

Parameters for the respective Port ..................... 62

(P)ARMS ................................................ 29

(PI)NG ................................................. 63

(PO)RT ................................................. 29

(PO)RT ................................................. 63

(PR)OMPT ............................................... 32

(QTH) .................................................. 78

(Q)UIT ................................................. 65

(RE)AD ................................................. 33

(READB)IN .............................................. 33

(RES)ET SYSTEM ......................................... 33

(RES)ET <port> ......................................... 33

(R)OUTES ............................................... 65

(R)OUTES (V)ersion ..................................... 67

(RU)BATCH .............................................. 33

(SAT) .................................................. 78

(SAV)ecall ............................................. 81

(SETCALL) .............................................. 42

(SH)owcall ............................................. 81

(SH)ow_SYS ............................................. 43

(SP)arm or better Save Parameter ....................... 33

START.BAT .............................................. 17

(STAR)T <program> ...................................... 33

(S)TAT ................................................. 33

(S)TAT ................................................. 67

(SUS)PEND .............................................. 34

(SYSH)elp .............................................. 44

(SY)SOP ................................................ 35

SYSOP-Commands ......................................... 21

SYSOP Help Programs for the SYSOP at Home: ............. 44

SYSOP-Work at the Nodes ................................ 14

(SYST)ext .............................................. 42

(TA)LK ................................................. 71

Page  5.

TCP/IP and TNN ........................................ 124

Text Files various and their meaning ................... 82

(TE)ST <port> .......................................... 35

TheNetNode-Program ..................................... 12

TheNetNode The Program Package .......................... 5

(TI)ME ................................................. 35

(TI)ME ................................................. 71

TNC2c and TheNetNode ................................... 83

(TNNSET.EXE) ........................................... 45

Tokenring ............................................... 6

(TOP) .................................................. 78

(TR)ace ................................................ 36

UMB Installation of .................................... 16

(U)SER ................................................. 71

Vanessa ................................................ 87

Vanessa Card ............................................ 8

(V)ERSION .............................................. 76

Watchdog ............................................... 85

(########.TNB) FILES ................................... 39

Forward

-------

TheNetNode has been written by Radio Amateurs. This has been explained,

already and means that no little financial and time consuming

inconvenience has been taken to construct a Packet-Node, to operate and

to interlink. TheNetNode raises no claim that it is Error Free. But the

Software writers are concerned to eliminate known Errors that might

occur afterwards. For this reason, there are issued small Updates

during the year. Anything further please take from the ALAS at the end

of this Document.

The Program Version for the Atari (ST) will be distributed now by Odo,

DL1XAO. But unfortunately future enhancements of TNN will no longer be

implemented there. (e.g.: Extension of Vanessa Cards or others.)

TheNetNode, an Update will be issued in the foreseeable future for the

TNC3 with the 68302 CPU (developed by DK9SJ).

TheNet, once developed for the TNC2, will no longer be distributed.

The present day Performance required from it, is no longer attainable.

Consequently TheNetNode is at home on the most varied Hardware

Platforms. With it, incorporated Flexibility may be expected by every

Operator of a Node.

The Program Package TheNetNode

------------------------------

TheNetNode is a Software Package, from which the entire Node Protocol

will be worked, e.g.:

- Up - and Down links to Users with or without DAMA-Protocol,

- Interlink-Protocol with Routing,

- User Interface (Help and Information),

- Includes various interfaces,

- Access to external Programs such as QTH- and SAT-Calculations, PR-

Call book, to name but a few.

Page  6.

The Program is provided for Communication with the choice of various

exterior Ports:

- Serial IO with Tokenring Protocol (9k6 up to the best case 115kBit)

- Maximum of 4 Serial IO Ports with Kisslink-Protocol (resembles

Tokenring)

- Parallel Ports to the Frontend Computer Vanessa (up to 140 Kbit each

Interlink)

- Parallel Port to Ethernet (10 Mbit/s)

- Baycom USCC via External Driver.

Tokenring

---------

Ports Information :

-------------------

The Tokenring has been imported in order to enable the servicing of

more HF-Ports via a Computer Port, which offers for usage, an

economical and adequate Port over a few years.

A TNC with a special Kiss-Soft serves for transfer to the Radio side,

PTT keying, Data sending, Data receiving to and from the Computer.

Additionally, the Packets will continue to be released from no longer

required Bytes.

A Tokenring is installed for forwarding. The Computer sends a so called

Token (a definite Byte sequence) onto the Ring. Because all TNCs are

ordered in a sequential circuit one behind the other, the Token arrives

at the first TNC in the Ring. Now if anything has been received on the

Radio side, the TNC retains the Token, and instead of this sends, the

Data received a moment ago to its Ring Neighbour and attaches the

Token to this Data. Because every TNC always waits for the Token before

placing Data on the Ring, it allows every Neighbouring-TNC to pass

the Data unaltered from the previous TNC. But as soon as the Token is

recognised, it retains it and also places its individual Data on the

Ring and then once again the Token. Some time or other, after each of

the TNCs, the Data arrives at the Computer.

Here the TheNet-Protocol will be executed and the Data provided with a

Port-Address, a Token will be attached again and sent on the Ring from

the Computer. The TNC also checks, whether with the received Data,

it is the one for which it is intended (Port Number). This will be

detached from the Token and output on the Radio Side with the

corresponding Parameters. At first sight, the entire method appears to

be somewhat slower, or not faster than the network of TNCs mutually

connected with a Diode matrix. But that is not so! A distinct

considerable increase of speed can be ascertained! That is because the

Tokenring is employed always, to approximately 100%, collisions between

the TNCs are not possible and no longer must the TNC Wait procedure be

executed in order to avoid Collisions. But the main reason for the

increase of speed lies within the fact, that with the help of the KISS-

Program, the Z80 in the TNC has much less to do than with the complete

TheNet. This Program suffers mostly from Interrupt routines (e.g.

Conversion of Synchronous- to Asynchronous-Transfer), for which the SIO

serves and distributes the Packets between V24 and Radio Ports.

Page 7.

There will be no unrequired Computer time in order to be concerned

about the Packets. For that purpose the Atari-ST / the PC, can achieve

it with ample speed. The working of the Packets and the handling of the

individual AX25-Protocol-Lives on this account, also runs somewhat

faster, because in spite of more connected TNCs it ensues once and not,

in every TNC separately for itself, as is accustomed from the TNC-

TheNet. The TNC should be equipped with a 9,8304 MHz Crystal and 10 MHz

SIO + CPU.

In other respects, the KISS-Software employed for a long time already,

with the TCP/IP Packet from KA9Q, also the Packet-Terminal Program

SUPERKISS with success on the WAMPES-Node Computers, (certainly without

the Token-Ring-expansion)

With a PC it must be defined for the Port by: SET TOKENCOM=<com No.>,

transfer rates from 9.600, 19.200, 38.400 Bits/Sec are possible. With

the FIFO-Port IC 16550 AFN from National as RS232 also 57.600 and

115.200 Bit/Sec. The Program recognises this IC itself, (Displayed

with the startup of the Computer and also evident with the Calling of

the Statistics) and applies to this Mode. Simplified workwise:

Intermediately up to 8 Bytes will be saved in the 16550 AFN with TNN

and only then issue an Interrupt to the Computer. Naturally there is a

Time internally on the Chip that watches over the Data flow and also if

necessary, passes on e.g. 3 Byte.

The advantage of the Ring is, that each TNC only sends to ONE other

Receiver or considers other incoming, every TNC receives only Data from

ONE other Sender. That means, that Level Problems, such as intrude with

the Parallel operation of more TNCs on a combined Diode Matrix, do not

occur here.

However advantages mostly have drawbacks. Failure in a Ring of only one

TNC, so if no precautions have been taken, the Ring fails and with

that, the entire Node. A further disadvantage is that the Baudrate of

the Tokenring will be distributed on the individual Radio Paths.

FIFO-ICs are OBLIGATORY in the COMPort with the employment of the GNU32

Programs !

Information for the TNC-Software and the Watchdog see Appendix B.

Kisslink

-------

Port Information :

------------------

Because wired connections to other Systems such as Mailboxes, Cluster,

or other Nodes must be achieved, TheNetNode also provides via a KISS-

Port. But this KISS-Protocol must not be confused with the beginning of

the described special TOKENRING-KISS. The Kiss-Port can be allocated

with SET KISS1=1 - SET KISS4=4 to a COM-Port. See AUTOEXEC.BAT and

(PO)rt-Command. As KISS-Drivers for other Systems are e.g. TFKISS,

TFPCR and TFPCX. Also the Connection is possible of a TNC2c under

another Call. Certainly the TNC must be manually pre-switched in the

Page  8.

KISSMODE.

---------

It allows installation of a different CRC-Method via the

(MO)de Function using the (PO)rt-Command. But here once again the

advice about the COMPort IC 16550 AFN.

By usage of the GNU32 Programs FIFO-ICs are MANDATORY in the COMPort!

Vanessa Card

------------

Port Information :

------------------

The Vanessa Cards behave as Slot-Cards, which are furnished with a fast

CPU und SCC. It requires an XT-Slot as a Port to the Computer and for

the Radio Side can be installed e.g. RS 422 Port Driver (Stream Port).

This enables the operation of a Modem over more than 100 Metres.

By the parallel Data handling on the Bus of the Computer it has given

a massive push in Data throughput. The handling speed of the Node will

be determined by its Links (Baud Rate and the transfer method (Semi -

or Full Duplex)). The internal Runtimes of the Node have become so mean

through the parallel handling of the Data, that it operates with a 64

Kbit/s Full Duplex link with no consequence. With an external Clock

(therefore synchronous Data Connection) the Vanessa Card can run up to

140 Kbit/s per Port.

2 Ports are to be found on a Vanessa Card.

For details of the Vanessa Card see Appendix C.

A kit of parts: board, CPU and SCC can be obtained From Henning, DG9FU.

A small Adapter plate is required for the Connection of a G3RUH-Modem,

which undertakes the conversion of RS422-Level onto TTL-Level.

Documents (Description and Circuit) can be obtained from Henning,

DG9FU, against reimbursement of the cost of DM 10.- (Photocopies  6.-

and Postage 4.-). Address on last page.

External Driver

---------------

In the DOS- and GNU-Version the Port Drivers for VANESSA, KISSLINK and

TOKENRING are integrated in the Software. It is otherwise with Ethernet

and Baycom USCC. For these applications the TNL1.EXE has been written

that makes the contact between TNN and the EXTERNAL-Drivers. Now, as a

Driver, the Card Driver which PC-Flexnet also uses can be used for the

operation.

From the Version V1.73 it is possible to load an external Driver

before TNN. Then this will be recognised automatically and installed.

First of all, for the usage of the external Drivers in TheNetNode, a

Program which enables the communication between TNN and Driver must be

loaded. This Program (TNL1.EXE) must always be loaded before the

Page 9.

Drivers. Exception: Driver e.g. Network Cards. These are not included

in TNL1, they are mostly Manufacturer specific.

TNL1 contains a 16Bit- and a 32Bit-Interface, i.e. a Driver which is

able to be addressed from TNL1 can be loaded with TNL1,.

More Information about TNL1.EXE input can be obtained -v and a few

Parameters.

TheNetNode Layer1 Interface V5.2 (c) 1996 by NORD><LINK (DB7KG,DG1KWA)

----------------------------------------------------------------------

Resident portion loaded, handler installed to IRQ 0x60

32-Bit Protected Mode handler installed to IRQ 0x61

Manifest is at 222A:0000

511312 bytes free memory

2432   bytes CODE

2672   bytes static DATA

528    bytes STACK

Usage: tnl1 [[-?] [Options]]

-h -?       diese Hilfe anzeigen / gives this help

-32         32bit Interface abschalten / disables 32bit interface

-u          TNL1 und Treiber entladen / unloads TNL1

-v          ausfuehrliche Meldungen / verbose reports

The execution for the Starting of a TNN with external Drivers could

e.g. appear:

; here a possible. Packet Driver for loading Ethernet-Card

; NE2000 0x65 10 0x300

; TNL1 load TNL1.EXE

; USCC-Driver Load USCC.EXE

; AXIP-Driver load

; ein NE2000-Driver must be loaded

IPPD -i:0x65 -m:44.130.88.1 -p:44.130.88.2

; TheNetNode TNN173.EXE

; unload all TNL1.EXE -u

The driver for VANESSA, KISSLINK (up to 4 parts) and TOKENRING are

included, fixed in TNN and should be preferred for the external Drivers

(only VANESSA and KISSLINK), because the internal Drivers are

substantially faster and few offer improvement (e.g. KISS-SMACK or

external Clock with the VANESSA). After the Start, TheNetNode

automatically recognises the Driver, then they must be configured only

on the corresponding Baudrate and Mode.

Problem Port-Numbering

Because the Driver of Port 0 will be placed ascending, it can give

Problems with other Hardware e.g. VANESSA. For this reason a Dummy-

Driver can be loaded as Space Holder. With this, it is possible, to

hold free an indicated number of Ports.

Syntax : DUMMY <Port-Number>

Page 10.

These two instructions

DUMMY 6

IPPD -i:0x65 -m:44.130.88.1 -p:44.130.88.2

means, that 6 Ports will be held free and then the AXIP-Driver will be

loaded.

Which Driver will be employed?

------------------------------

Currently, the Driver of the PC/FlexNet will be employed (actual

Version).

WARNING: The Driver of the PC/Flex 3.3c does not work with TNN !

GNU32-TNN

---------

Meanwhile, the external Driver functions and  very well with the

GNU32Bit-Version of TheNetNode, certainly it gives an object to be

observed. Equipment that generates many IRQs, are poison for a GNU-

Version, because it forces the CPU, to switch between Real-Mode and

Protected-Mode, running here and there. But this toggling costs a

certain Computer Power.

Drivers such as USCC, SER12 or PAR96 certainly do not run without

Problems. With a more pure DOS-Version of TheNetNode it should give no

Problems.

With the Network Driver the matter appears somewhat better, this should

run without Problems. What is more, in a Test the GNU32-Version

delivered a faster (!) result than a DOS-Version !!!

At the moment the following Drivers are available, but the List lays

no claim to be exhaustive !

IPPD AXIP-Driver;

With this a LINUX-Computer (WAMPES or DP-BOX) can be combined with TNN.

Runs with GNU- and DOS-TNN

IPXPD Network Driver with IPX-Protocol, places on a loaded Packet

Driver. TESTED ....goes UFB !

IPXN IPX in a Novell-Network

NOT TESTED !

USCC Driver BAYCOM-USCC

Limitations as described above, runs impeccably with the DOS-Version.

KISS KISS-Driver, but the internal TNN-Driver should be preferred.

Runs with DOS and GNU

VANESSA Vanessa Driver; but the internal TNN-Driver should be

preferred, because it is somewhat faster!

Runs with DOS and GNU

6PACK Tokenring appropriate, is faster and more mature !

Page 11.

SER12 Driver for BAYCOM-Modem ..NOT TESTED !

--------------------------------------------

PAR96 Driver for BAYCOM-9k6-Modem ..NOT TESTED !

Valid for all Drivers, before using, read the included DOC !!!  When

anyone uses TheNetNode with a Driver, we would very much appreciate

some Feedback !

Ethernet-Port

-------------

From Version 156pG64 TheNetNode(PC) is offered the possibility, to

couple more TNNs or Mailboxes one with another per 10 Mbit/s.

Exclusively economical ETHERNET-Cards and an appropriate Diver are

necessary for the TCP/IP PKTDRV-Specification.

Now only a meager RG58 Cable is required to be installed and 2 50 Ohm

termination Resistances or a Twisted-Pair and it can be connected to

arbitrarily many Computers. In addition to further TNN-Computers e.g.

also a Mailbox or a DX Cluster (Drive in use) etc..

For Mailbox or User Connection:

Certainly the Program "TFNETR.COM" MUST behave itself as a TFKISS-

Driver, which concerns the Port to the Terminal Program. TFNETR will be

installed simply as resident, after which the PKTDRV-Driver will be.

Consequently e.g. Terminal Programs such as TOP, GP, THP can be

operated and what is more WinGT and the Node Computer connected via the

Ethernet-Network!! So is somewhat e.g. first class for laid down Sysop-

Terminals in the QRL.. Naturally this works, with the DIEBOX-Mailbox

and other Programs which TFPCR-Software Port supports. Because the

AX25 Data will be "packed" simply in Ethernet-Frames and broadcast in

the Cable, all participants can be connected with each other also.

The advantages lying at Hand:

- Ethernet Cards (mostly NE2000 compatible) are very economical and

cost at the most 1/3 or less than elementary TNCs.

- Double-TNCs or VANESSA/TNC-Combinations can be eliminated, through

that Finances and effort are saved.

- The KISSLINK with all of the Problems of the PC-RS232-COMPort can be

eliminated.

- it offers a "meager" RG58 or Twisted-Pair across all extensions where

there is space.

- galvanic decoupling.

More about this in APPENDIX D.

"DPMI or availability of the most broken continuous Memory over 1MB"

--------------------------------------------------------------------

Previously under MS-DOS we have always had to fight, that the available

Memory of 1 MB (+ HMA) was limited. This Memory must contain all

Programs. With that Programs such as TNN were squeezed in a Memory

area. With the DOS-Protected-Mode-Interface available is only up to 16

MB respectively 4 GB theoretical Memory for disposal. But why MS-DOS is

still supported, when there are so many good Operating Systems such as

Page 12.

OS/2, LINUX or WIN 95 >? All of these Operating Systems have a very

high Memory demand, furthermore for our purposes, mostly one desires no

unrequired frills (Exception LINUX and OS/2). Furthermore every Sysop

cannot become immediately friendly with LINUX. For this reason the

further support of MS-DOS.

Because the GNU-Version is an expansion of the "normal" DOS-Version, I

have included the explanation in APPENDIX G.

TheNetNode-Program

------------------

With that, we arrive at the Program. Mentioned should be, that the

Computer also checks the Port by a Watchdog and in an emergency will

be restarted, also the best Program can go astray....

On the Diskette are to be found the Programs TNN174.EXE. Password,

IDENT (Call Sign), ALIAS etc. They are included in the TNN174.PAS, that

in future should no longer be altered. With (S)tat can be requested

statistical values will be available for adjustment of the Parameter

11 in X times 10 Minute delays. Therefore with the installation '6'

once every hour, will be saved on the Diskette. Consequently a maximum

of one hour after failing a new Start. In the Statistics: How long the

Program has been without a New Start in Operation can be seen from the

first Line of the Statistics. TNN174.CFG no longer enables it. Now the

Parameter will be installed via TNN174.TNB. All required pre-installed

Parameter are contained within. With the Program Start TNN....EXE

searches for the Files TNN174.TNB, TNN174.PAS and TNN174.STA and loads

from them, its operating Parameters into the Computer. If the files are

not available or defective, on the Monitor will be displayed :

WARNING: Error in TNN174.PAS ***

*** TNN174.STA - Open-Error ***

WARNING: Error in TNN174.STA ***

WARNING: Packet-Driver (PKTDRV) fails !!

Login: TNN174.TNB not found !

Then the Program employs the Default-Parameters and create new Files.

If the Files are in order, no special Advice follows.

WARNING: Packet-Driver fails, is ONLY the advice, that an Ethernet-

Port CANNOT be addressed. But for the operation WITHOUT Ethernet has

no further drawback !

After the alteration of Parameters TNN174.PAS and TNN174.STA, from time

to time after leaving the Sysop Mode through Quit, Disconnect or

Connect in each case will be rewritten on the Diskette. With that it is

ensured, that with a Program restart the Parameters will be installed

correctly again.

Page 13.

In future the Files TNN174.PAS and TNN174.STA will have the Version

number in the File Name. Then, with that it is also possible, to place

different Program Versions in the Nodes.

TheNetNode can manage 100 Level-2 (Up - Down links) and 100 Level-4

(Circuit) Connections. That might be sufficient for most Nodes in the

immediate future. The Software from DB0EAM has been expanded to 250

Level-2 Links and 150 Level-4 Links. In the peak times of Autumn -

Winter 1995 up to 240 Level-2 and over 120 Level-4 Connections were

observed. The many Level-2 Connections established themselves from the

Connections to the Mailbox Kassel DB0EAM-3 (28) and to the Cluster

Kassel DB0EAM-4 (60) as well as at the same time, from two RMNC-

Neighbours DB0NHM and DB0BID.  Since 1994, on a normal Weekend it

rises to 200 Level-2 Connections.

DAMA ... Traffic regulation through the Node

--------------------------------------------

If it is required to install DAMA on a Node, it should be understood,

that with Nodes, definite Parameters must be installed.

The Parameter T2 under DAMA should be installed on a value from 150 ms

(1k2) to 50 ms (9k6). The DAMA-Operation now regulates the Time point

of the acknowledgement and at which place in the queue the User

receives Data again.

The Persist Value must be installed on 255, and with that the Node

sends without further delay.

The following Parameters are basic values:

DAMA Tout   = 100

DAMA MaxPri =  10

DAMA MaxPol =   5

DAMA Speedf =   0

A further point is the DAMA-Bit, that must be set in the Eprom of the

Port-TNC, on which the DAMA is to run. With a set DAMA-Bit, the TNC

sends the Information back to the Node-Compute when a Packet has been

sent to a User. This Information will be required for the control of

the DAMA-Function. (Not necessary with the Vanessa Card.)

DAMA-Execution control:

-----------------------

Stations that do not employ DAMA-Capable User Software and send through

the DAMA-Master WITHOUT invitation, will be cautioned IN THE QSO. If

more warnings are required, as given under Parameter 24 DAMA-MaxPol, so

the User will be disconnected. Also they may NOT operate more TNCs

under the same Call but different SSIDs, because the Nodes in the DAMA-

Modes invite all existing Links for Sending. Two TNCs with the same

Call but different SSID will send simultaneously!

For the TNC2 there is a DAMA-Capable Eprom, it stands in the Mailbox

under the Rubrik 'TNC2'. Also usually will be offered there will be a

Page 14.

Cost free Eprom-Service. Those who wish to use the advantages of DAMA,

should certainly install The Firmware 2.7b! Since The Firmware 2.7b

beefing Reports no longer arrive Multi-connect. The Software error

that after polling of a DAMA-Connection has sent the Data of all

existing Data, is eliminated.

Without question, all other Users should FRACK on the Maximum Value.

*** Important ***

On the DAMA-Entry direct QSOs are defaulted under bypass of the Node,

because as they do not demand being sent from the Master, leads to the

disturbance of the entire activity!!! If the occasion arises the Sysop

should switch off the Level-2 Digipeating on the pertaining Entry. Then

in the (PO)RT-List NO L2-Digi may stand behind DAMA. AUTODIGI, against

that nothing disturbs, because here will be  executed a clean Entry

in the Lists.

This is also valid for operation via another Station as "Digipeater",

in order to reach the Node on the same Frequency.

USER with Multi-Connect does not arrive in the queue very often against

USERS with only one Connection to the Node. But now "Empty Polls" will

be noticed also.

SYSOP-Work at the Node

----------------------

The Program can be serviced from the Console (Keyboard) or via Radio

also. In both cases an Identification as SYSOP is necessary. From the

Console the operator will be asked to input the "Password". As Default-

Password "Geheim" (Secret) is fixed in the Program. If it is input

correctly (Observe Upper- and Lower Case), the following greeting

arrives:

Welcome to TheNetNode (PC), Version 1.70 (Date). With <ESC> c <RETURN>

it is now possible to connect as HOST with the Node. As HOST one

automatically becomes SYSOP and can undertake all alterations to the

Parameters.

After the end of the work, close again with <ESC> logout <RETURN>.

Instead an automatics Log out occurs before every passing of an hour.

First on the Radio side the Node is to be connected and then request

the Sysop-String with at least "sy". On the strength of this the

following Text is received:

"KS:DB0EAM> 62 36 65 13 34".

This numbers give respectively, the Position of a Character in the

Password. The character belonging to the Number must be entered now.

In the Password NO empty characters may occur. A receipt, whether the

return sending is correct or is wrong, nothing returns. The sending of

five Characters (and should) be performed more than once, whereby a

Page 15.

group of five Characters only needs to be correct on one occasion.

Then whether the 1st. 2nd. or 3rd or whatever entry is the correct

Password, makes no difference. The multiple Identification serves only

for the confusion of the "Genius", who would also like to have the

Password, in order to seize Sysop "auxiliary" furtively. A genuine

Sport that has developed in many places already, hihi. If that method

is too burdensome for the Sysop (as I), so I recommend, to allow a

small Program to get afloat.

In order to assist "Genius" to extract the last tooth, so at least with

me, when I remotely load the Program as a Binary File to the Node,

instead of the Password on the Default-Parameters "12345678901234

etc.". Now with that, the copying of the Binary File is pointless.

On the one hand everyone can obtain the Program, on the other hand the

Password cannot be obtained from this path. the actual Password is

placed in TNN174.PAS already.

Additionally in der File SYSOP.PRO, a Log Book will be created, so that

every Input of the SYSOP-Command will be registered. Call Sign, Date

and Time will be saved. This File can be read by the SYSOP and by

choice also erased, when it is too long.

Now at least the Sysop the "powerful" Dominant stands open via the

Node.

Still a very important point of TheNetNode :

On the Node an MH-List will be created. It will be saved as MHEARD.TAB

on the current Diskette Drive and updated in the RAM with every User-

Packet. If a Floppy Drive is used as the current Disk Drive, so

Problems are pre-Programmed.

Escape: Always in the future more Memory Space will be required for

the intermediate saving of Data , e.g. : Creation of a Name List and

possibly a QRV-List. The readout of the `U +` List will be saved

intermediately as a  .TMP-File. Also this should occur according to

possibilities without Time delay. Therefore it offers advantage to

install a small Hard-Disk with the Node Computer. If at the Moment

nothing can be done, it must be done by recourse to a virtual Disk

Drive (RAM-DISC). Then all .TXT-Files should be copied therein and also

possibly COMMAND.COM. It must still offer the required Space in order

to execute the MHEARD.TAB File (Size adjusts itself from the

Installation MH = ...) and still be able to include the List `U +` If

the Space is not available, so a Program Crash is pre-Programmed !

Computer Configuration

----------------------

(Here as an Example for a 386 or 486 with HD)

Page 16.

CONFIG.SYS

**********

DEVICE=c:\HIMEM.SYS

DEVICE=c:\emm386.exe noems x=d000-dfff i=e000-f3ff

DOS=HIGH,umb

COUNTRY=049,850,C:\DOS\COUNTRY.SYS

FILES=30

BUFFERS=40

STACKS=9,256

Comments:

TheNetNode can be operated on a 286 (TNN1xx_2.EXE). In the case of the

Computer which has only 640K instead of 1MB, then a smaller RAMDISK

should be established.

EMM386 / QEMM for the allocation of UMB:

Upper Memory Support [UMB] 2.0 for TheNetNode 1.70 by DB7KG

First once again, what is UMB?

UMB is the name of the Free-Memory above 0A0000 to a maximum of 1 Mb,

it will be positioned by various 386 Memory Managers according to

choice. It serves for the purpose, to include Drivers, with that this

does not load the Memory under 640 KByte.

Here are the Features of the new Version:

1.)Auto Configuring:

Simply install only QEMM or EMM386 and start TNN.

2.) Usage OF ALL free UMB-Blocks, not only of ONE Block. That comes

especially good for EMM386-Users, because EMM386 dismembers the UMB-

Memory

Installation of UMB

-------------------

At first, to install a UMB-Driver such as MM386 or better QEMM. With

EMM386 an extensive Configuration is required, QEMM is far reaching in

self Configuration.

Tips for EMM386:

----------------

1.) For every employed Graphic Card (Monochrome or VGA) various domains

can be freely issued with I=xxxx-xxxx by the Calling of EMM386

A000-AFFF; B800-BFFF; F000-F7FF  /. F000-FAFF. The domain C800-CFFF

can be freely given, when no HD is used. Each one must be self

tested here on its Computer!

2.) E000-EFFF mostly usable, investigate. C000-C7FF well nigh always

usable.

3.) D000-DFFF However, MUST be barred with the insertion of Vanessa

Cards. (Domain always for Addressing of the Vanessa Card!)

Page 17.

As Help: here are a few Domains, which possibly can be used:

A000-AFFF : Graphic Memory in the Graphic Mode,

B000-B7FF : Graphic Memory (monochrome),

B800-BFFF : Graphic Memory (NOT monochrome),

C000-C7FF : ROM of the VGA-Card,

F000-F3FF : ROM-Bios.

(Source: PC-Welt 3/94)

An overview of the employed Memory is obtained with the Diagnostic-

Program MSD.EXE. It shows the occupied and available Memory Areas.

Also the Program MEMMAKER.EXE from MSDOS 6.2 self supporting gives the

free usable Memory Areas.

II. Support of TNN

------------------

Simply start TNN, all UMB-Blocks will be recognised and used

automatically.

The reporting of an Area with x=D000-DFFF by EMM386 is only necessary

with the Usage of the Vanessa Cards. Areas each for Addressing of

the Vanessa Cards.

The Memory Management can be requested by the Sysop using the DOS-

Program MEM. (See DOS-Documentation).

For clarity, I have divided up the AUTOEXEC.BAT and installed for

the TNN-specific Installation of a START.BAT. The AUTOEXEC.BAT contains

the Computer specific Installation, the START.BAT the Installation of

TheNetNode.

AUTOEXEC.BAT

------------

************

@ECHO OFF

prompt=$p$g

lh KEYB GR,,a:\KEYBOARD.SYS

path c:\dos; c:\tnn

START.BAT

START.BAT

---------

*********

set tnncfg=1009,32

rem set tnnbuffers=1500 (for a 286)

rem set tnnbuffers=2000,UMB (for 386 and 486)

rem set tnn32buffers=5000 (for the GNU-Version)

set conversd=P:0

set TZ=UTC0

set msgpath=C:\TNN\MSG\

set QTH=<coordinates> or <locator>

set TOKENCOM=2

set KISS1=1

Page 18.

set copycmd=/y

b-log.exe

cd TNN

TNN174.exe

Explanation of the START.BAT:

-----------------------------

SET TNNCFG=<1.Number>,<2.Number>

The 1st. Number inputs the size of the Nodes / Destinations List.

Default Installation :1009.

The 2nd. Number is the length of the Link List. Default : 32.

The management of the Routing-Tables of TheNetNode follows with a

so called Hash-Algorithm. This works very efficiently, so long as the

Node List is filled less than 50%. If the percentage is higher, it

raises the Access Time of the Table disproportionately high. On Systems

with little Memory, the Nodes-Table should be installed on at least 30%

more than the awaiting Nodes-Number. On Systems with plenty of Memory,

the List can be over dimensioned by a Factor of 2-3.

SET TNNBUFFERS=

---------------

ONLY FOR THE DOS-VERSION !

Via the Environmental Variable TNNBUFFERS TNN can be informed, how

much Buffer of the conventional Memory (640KByte) should be used. It

will be in the START.BAT with e.g.

"SET NNBUFFERS=2000" Additional to the Buffers in the conventional

Memory, with the operation with a "386 or 486"

Memory via the added UMB the access to the upper

Memory between 640 KByte to 1 MByte can be

enabled..

In every case TNN allows 64KB Working Memory for

external Programs.

The effect on the Buffer-Management after the

alteration of this Installation, after the start

of TNN (In the connected condition), it allows

read out, with the Command DOS-MEM /c.

SET TNN32BUFFERS=

ONLY FOR THE GNU-VERSION ! Otherwise similar operation.

SET CONVERSD=P:0

P determines whether will be displayed the Protocol on Channel

32767. The Protocol displays detailed and outgoing Host

Commands and can be input for Analysis purposes.

P:0  (default): Protocol off

P:1  Protocol on

Page 19.

SET  TZ:

The Variable TZ will be required for the Convers Mode also and

the SAT-Calculation. With that the Times attune, each to

installed Computer Time, to place on UTC0, MEZ-1, EST-2 or CET-

1 SET MSGPATH:

Path for MSG-Files SET QTH=<coordinates> or <locator>:

Variable QTH will input the Coordinates or Locator of the Nodes

for the Programs QTH.EXE and SAT.EXE in the DIR USEREXE.

Naturally in the nearby area the Coordinates are better suited.

See: external Programs for all Users.

SET TOKENCOM=<com No.>,<Address>,<irq>

The Port for the TOKENRING can be placed on COM 1 or on COM 2.

The declared <Address> and <irq> are required only for exotic

Addresses and Interrupts.

SET KISS1=<com No.>,<Address>,<irq>

Now there are 4 KISS-Ports with KISS1..4=1..4 able to be

activated. Also, as with the TOKENCOM-Port here the

Installation of the <Address> and of the irq> are only

necessary with a departure from Standard. If the KISS-Port is

not required, it can be deactivated with SET KISS1=0.

B-LOG.EXE:

Included on the Diskette is a Program B-LOG.EXE. It creates a

File LOG.DAT. If B-LOG.EXE is involved from the AUTOEXEC.BAT

before the Program Start TNN.EXE, so arrives an unbroken

overview via the Boot procedure of the Node Computer.

Construction of the TNN174.PAS-File

-----------------------------------

Through the external Commands, additional Entries in the PAS-File have

been required. In order to improve the readability of the PAS,

automatic Commentary from TNN will be written inside.

The File now has the following Construction:

; TheNetNode Configuration File

;

; DO NOT CHANGE THE ORDER OF THE CONFIGURATION LINES !

; DO NOT ERASE ANY LINES !

;

; NET/ROM-Sysop-Password, 80 Characters (01234567890123...)

12345678901234567890123456734567890 etc. up to 80 Characters...

;

; Console Login (thenet)

unused

;

; Console Password (Geheim)

Geheim

;

; Node Ident (Test)

Test

;

; Node MyCall (XX0XX);

XX0XX

Page 20.

Convers Callsign (DB0XYZ-1)

DB0XYZ-1

;

; Work path, Path to the Help-Files (\TNN\)

; TNN should be started from this path.

C:\TNN\

;

; Path to the executable Text-Files (\TNN\TEXTCMD\)

C:\TNN\TEXTCMD\

;

; Path to the external Programs for User (\TNN\USEREXE\)

C:\TNN\USEREXE\

;

; Path to the external Programs only for Sysop (\TNN\SYSEXE\)

C:\TNN\SYSEXE\

;

; Path to the PACSAT-Files (\TNN\PACSAT\)

C:\TNN\PACSAT\

Directory structure:

--------------------

The WORK PATH contains all Files used by TNN ( .TNB, .XHF), .TXT,

insofar as it handles System Reports (HELP.TXT, SUSPEND.TXT, AX25V1.TXT,

QUIT.TXT, CTEXT.*).

After the Start, TNN branches automatically to the Directory entered

under WORKPATH.

StartPath/ConfigPath

--------------------

Here stands the achievable TNNxxx.EXE and the Configuration Files

(TNNxxx.PAS TNNxxx.STA MHEARD.TAB TNN174.TNB). This Path is not

installable. It adjusts itself always from the place, at which

TNNxxx.EXE stands. If TNN is to be started with mit " A:\TNNxxx.EXE ",

also will be sought and written in the same Directory the

Configuration. In this Example is WORKPATH = StartPath.

TEXTCMD:

--------

In here belongs the Node specific Texts, which are input under

HELP.TXT. (MAP.TXT, INFO.TXT, AKTUELL.TXT etc.) USEREXE. Here stands

the external Commands, which may be executed by all Users.

SYSEXE:

-------

Here stands all external Commands, which can be executed only with

SYSOP-Privilege.

PACSAT:

-------

Sub-Directory for BROADCAST-Operation. See APPENDIX E:

The following Directory Structure works problem free for Systems with

Hard Disk and with RAMDISK:

Page 21.

C:\TNN\

C:\TNN\MSG\

C:\TNN\TEXTCMD\

C:\TNN\USEREXE\

C:\TNN\SYSEXE\

C:\TNN\PACSAT\

For Diskettes-/RAMDISK-Operation stands TNNxxx.EXE then e.g. in A:\,

for Hard Disk Operation stands TNNxxx.EXE e.g. in a Directory "C:\TNN"

The arranged queue of the Commands is: INTERN - TEXTCMD - USEREXE -

SYSEXE.

Construction of the TNN174.TNB-File

-----------------------------------

The basic installation of the Node will be undertaken with the

TNN174.TNB .

........................List from here includes ! ....................

SYSOP-Command

-------------

Now the Sysop should transfer the following Commands into position,

placing the Nodes for the local importance.

(BE)ACON

--------

Sets / Alters the Beacon Text for the individual Ports.

(BE)ACON <port> =

Removes the Beacon Text.

(BE)ACON <port> <mins> <tlm> <call or alias via call>

Switches on the Beacon.

Port 0..16     Port Number.

mins 0, n      0 =  Beacon off.

n =            Interval in Minutes. Default is n=10 Minutes.

tlm 0, 1, 2

0 =  No Telemetry.

1 =  Telemetry.

2 =  Extra LSTAT.

call or alias QST = UI-Frame from Node to Call 'QST'.

If the QST-Beacon is to be sent, normal Beacon Text will NO longer be

sent. It informs the Terminal Programs, which of the QST- and/or LSTAT-

Beacon can evaluate and display directly. Specially to be mentioned at

this point: THP (Turbo Host Packet) and TOP (The Other Packet). At the

Node Terminal this Beacon can be copied or also saved in a File. Input:

<ESC> MU + QST LSTAT or <ESC> MCU + QST LSTAT. With a new Start the

Monitor will be switched off.

The Beacon has this Information with tlm=1:

Page 22.

920102 018040         Actual Date and Clock Time

UP=0/15:45            Operation Time in Hours and Minutes

Fmem=217840           Free DOS Ram (Core left, Bytes)

Free=1623             Free TNN-Buffer (Blocks)

RPS=36                Main loop through running/Seconds

R=0                   Number of the Token-Recoveries

LNK=15                Number of the active L2-Links

CIR=20                Number of the active Circuits (L4-Links)

SUM=229961            Counter setting of the Statistic-Total-Counters

Additionally with tlm=2 a UI-Packet of the Node to LSTAT will be

radiated with every received L3-RTT measurement Frame:

DB0FC                 Call Sign of the Neighbour Node

(9)                   Receive Port of the Measurement Frame

5370717               Processor Time in 10ms steps

L3rtt= 47900          A moment ago measured Level-3 RTT

(31410)               Last Level-3 RTT

L3srtt= 10180         Last Level-3 SRTT

L2srtt= 2580          Level-2 SRTT.

With a FlexNet-Neighbour the Beacon appears:

DB0NHM                Call Sign of the Neighbour Node

(10)                  Receive Port of the Measured

5370717               Processor time in 10ms Steps

L3srtt= 3400          A moment ago measured Level-3 SRTT

L3srtt_n= 2580        From the Neighbour measured Level-3 SRTT.

LZ= 2990              Value taken over. (L3srtt + L3srtt_n /2)

(BE)ACON <port> 0 0 ID   Switches of the Beacon.

(CL)EAR

-------

Erases all Statistical-Entries.

-------------------------------

(CONV)ers

---------

Displays the List and the Status of the Neighbour-Convers- Host as well

as the Runtime to all Convers-Hosts and the Version Numbers. ONLY in

the Sysop Mode will be displayed additionally the incorporated Path

(whether L4 or L2 with Port and Via-Entry).

(CONV)ers (C)stat <call>

...or in the Convers Mode with: /Link <call>

Includes a Neighbour Node for Convers-Operation. Under <call> is to be

installed the Call Sign of the Neighbour Node. Then the Neighbour Node

must stand in the Node List.

(CONV)ers (C)stat <call> <port> <via list>

...or in the Convers Mode with: /Link <call> <port> <via list>

Includes a Neigbour Node for Convers-Operation.

Page 23.

Under <call> is to be installed the Call Sign of the Neighbour Node.

Then the Neighbour Node must stand in the Node List.

(CONV)ers (C)stat <call> <port> <via list>

Under <port> the Port Number is input and by Choice a via-List. ...or

in the Convers Mode with: /Link <call> <port> <via list>

Includes a Neighbour Node for Convers-Operation via a Level2

Connection. Under <call> the Call Sign of the  Neighbour Node is set.

Under <port> the Port Number is input and by choice a via List.

UNQUESTIONABLY LOOPS MUST BE PREVENTED. ON THIS ACCOUNT: PLEASE DENY !

(CONV)ers (C)stat - <call>

or in Convers Mode with

/Link <call>

Erases the Entry.

(CONV)ers - intern -

Ping Pong Convers out of Sysop View:

Links:

------

Links will be incorporated with the "/links" Command, which can be

selected outside of the Convers by way of "conv c". Incorporated Links

are permanent und will be connected in definite Intervals

(9s,150s,300s,600s,...,3h). There are also non-permanent Links, but

this Option can be installed by Configuration. These Links will be

delivered automatically with a Link synopsis, the Node is not connected

to the new Partners afterwards. (The entries occur by the Input of the

"/.host" Command)

Syntax is:

/l [-][call [port [via]]]. outside of Convers:

conv c [-][call [port [via]]]

- Serves to erase an Entry, (<call> is adequate as Input).

<call>  Is the Call Sign of the Neighbour Node.

<port>  Down Link Port (will be required only with L2 Connections).

<via>   Via-List (optional for L2 Connections).

For L4-Connections the Input of the Call of the required Nodes is

sufficient, but it will NOT be attempted to construct, an L2-

Connection with a missing Node Entry. With this Command under every

Link Entry the Sysop will displayed the Connect Path in Brackets,

e.g. :

Page 24.

In the Case of L4  (DB0AGM),

In the case of L2  (DB0XYZ on port 2 via DB0ZXY).

Help:

-----

Is in a File named CONVERSD.XHF. This contains all the Texts, separated

by marks. It is permitted, to alter the Texts, sequence plays no role

up to the uppermost extract. The Markings are defaulted by Convers,

insertions of new marks is for this reason useless. The File contains

Umlauts from ISO, please think about that. If Help is required, the

Convers converts the Umlauts itself into the Form that is required by

the User.

Miscellaneous Files:

--------------------

CONVERS.PRS contains the personal Data of all that has been entered to

the individual Node. This File is also in the Text Directory, but will

be secured in the Config Directory with the ending of TNN / with the

periodical securing of the Configuration. Under that falls the Personal

Text, the Character Set choice and the Line length.

Loop detected:

--------------

Now Loop will be checked and the corresponding Link for an hour "taken

out of use". The appearance and the occurrence will be displayed in the

CONV-C-Table.

(DCD)

-----

The DCD-Command displays that in the Software an identified DCD

condition.

P00 P01 P02 P03 P04 P05 P06 P07 P08 P09 P10 P11 P12 P13 P14 P15

t           rt  rt  r           OFF OFF OFF OFF OFF OFF OFF OFF

t = ????

r = ????

(DOS) <Command>

---------------

Access to COMMAND.COM.

With this, it allows execution of many Operations. Naturally for this,

there must be access to to the corresponding expansion Program. However

Caution is absolutely recommended! With an Error I have driven a

minimum of 50km and eventually won a heavy march. Therefore prevent

interactive Input!

Example:    DOS DIR     lists the actual Directory.

DOS DIR A:  lists Disk Drive A.

(E)DIT

------

Creates the File-Name in the Work Path with the name behind Edit. With

(E)dit however no existing File can be called for alteration. The

existing File will be replaced!.

Page 25.

(E)DIT <path\filename.ext>

--------------------------

The Input of a Path is possible, but this will not actually be used by

TNN. the File always stands in the actual Directory (WORKPATH) and

finally with DOS COPY, must be copied into the required Directory. If

the Edit Mode is already called by another Sysop so arrives the MSG:

Edit-Mode in use by other Sysop

(IPA)dress

----------

Sets the IP-Address of the Node.

(IPB)roadcast

-------------

Sets the IP-Broadcast-Address of the Node.

Not yet used, please leave on 0.0.0.0 !

(IPR)oute

--------

<destip>/<maskbits> +/- <interface> <gateip> <metric>

Includes <destip>/<maskbits> on <interface> <port> <gateip> in the IP-

Route List.

DestIP

------

Is the IP-Number of the (IP-) Neighbour on this Port

Maskbits

--------

Here will be incorporated either a Number (Number of Bits) or a type of

Pseudo IP Number. Number of Bits means: An IP Number has 4 Bytes with 8

Bit = 32 Bits. Here I enter 16, so only the first 2 Bytes will be

evaluated (corresponds for AFu: 44.130; 44=AFu 130=DL). If I enter 24,

the Region will be observed already, also 3 Bytes (44=AFU 130=DL 27=KS)

with a (Example) Number 44.130.27.80. So one can hierarchical

route/sort out. With the other possibility a 0 entered, will be for the

Byte, what remains free. Example:

44.130.27.0 = Kassel.

Also with Broadcast can be so views together:

255.255.255.0 or 44.130.27.255..arrives completely upon it

+/-

For the In or Output Interface

Here the Port will be named, where the Packets fo <DESTIP> should be

emitted. This can be the NAME of a Port (Therefore User_1k2, User_9k6).

Page 26.

"NETROM"

--------

Can also be input. In this case the Packet will be

transferred to a NETROM-Target (naturally that must also be a Gateway

[TNN] or a TCP/IP-Host [KA9Q]).

GateIP

------

Will name a Gateway, here will be incorporated the IP-Number of the

Gateway. The via it all (IP-)Packets will be handled.

Metric

------

This Input will have meaning in a later Version, at the moment please

ignore.

Example:

IPROUTE 44.130.27.0/24 + User_9k6

(ARP)

-----

<DestIP> +/- <publ.> <Port> DG/VC <call[-*]> <[digi1> <digi2>]

Includes <destip> with <publish> <port> as DC/VC as <call> in the Link-

List. The Digipeaters <digi1> and <digi2> can be input.

ARP-Entries make TNN automatically. They must be input only for Paths

via-Digipeater and NET/ROM-Connections, because ARP is not possible

here.

ARP-Entries will be made for Gateways, /. for / directly attainable

Stations.

DestIP

------

Is the IP-Number of the (IP-) Neighbour on this Port

+/-

For the In or Output

Publ.

-----

Publish

Should this Port/Neighbour and all others also known be input? A P for

stands for "ARPs answer", ARP-Requests to this Target will be ignored.

Port

----

Port Name of the Node via which the IP-Address is attainable. Here

NETROM can be input again.

Page 27.

DG/VC

-----

Datagram/Virtual Circuit. Therefore the entire Connection release will

be constructed (Datagram) or should a Connection (Virtual Circuit).

From Digi to Digi DATAGRAM should be installed, then it gives less

overheads. Then instead, certainly no Error Correction is found on the

Link, but this will be made (admittedly somewhat slower from the End

Stations (TCP/IP). On DAMA-Entry UNQUESTIONABLY VC is to be adjusted.

With NETROMs this Parameter has no meaning.

Call

----

AX-25-Call, under which the Computer with the IP-Number <destip> is

attainable.

Example:

ARP 44.130.82.41 + P Sysop_9k6 VC DB7KG DB7KG-5

What works and what does not work:

- The IP-Router of TNN can transfer Frames of optional Length by

Fragmentation. The Reassembly will be undertaken by the End Station

- On Compatibility Grounds TNN to Flexnet only Packets up to 256 Bytes

Info. This will alter between two TNNs in future.

- ARP/REVARP-Requests will be answered, as long as allowed.

(K)ill

------

Interrupts all Level-2 Connections on <port> after Sending the <msg>.

(K)ill <call> <msg>

Interrupts all Level-2 Connections to <call> after Sending the <msg>.

(K)ill * <msg>

Separates all Level 2 QSOs on the Digipeater after Sending the <msg>.

(L)INKS

-------

+/- <typ> <port> <alias:call[-*]> [<digi1> <digi2>]

Includes <typ> with <alias:call> via <port> in the Link-List. For the

combining of Digipeaters without TheNetNode-Protocol. The Digipeater

<digi1> and <digi2> can be input. Only Nodes will be assumed which are

incorporated under Links. Also the incorporated Port will be observed.

+/-

+ Entries and - removals of the Link.

Types:

-----

L   =  Local

Page 28.

Call and Alias will be undertaken with a Runtime of 4000 ms in the

NODES-List, but it will be executed no Runtime Checking. In the Route

List will be executed without Entry in Status.

L+ = Local

As above, however the ability to attain will be checked and the Runtime

measured. If the Runtime can be ascertained, so this Target will be

executed for the improved prospect as Connect in the Status.

F   = FlexNet

Port works with FlexNet Protocol.

N   = NetRom

Actual TheNet-Neighbour. Undertakes in the Nodes List with the

ascertained Runtime

(LOA)D<filename.ext>

--------------------

Loads <filename.ext> on the Diskette or HD with the Node. The

<filename.ext> may be a maximum length of 8 Characters + 3 ext. and

must have the following Format:

LOAD TNN152.EXE

---------------

The File will always be placed in the Sub-Dir, in which TNNxxx.EXE is

to be found and must, in the case where necessary, be copied into the

required Sub-Dir later by DOS COPY C:\TNN\etc. Caution: with the choice

of File Name, otherwise the previous TNN-Program will be overwritten

accidentally. If the occasion arises a not yet used File Name is used

and after copying into the required Directory with DOS REN.....renamed.

After the input of "LOAD <filename.ext>" the Computer waits for the

sequence #BIN#Length (e.g. #BIN#82345). The number behind #BIN# stands

for the Program length in Bytes (decimal). On this sequence, it will be

reacted with an #OK#. After the SYSOP has received this #OK#, the

transfer can be commenced.

The above described Procedure corresponds to those from THP, SP or

GIPSY are known as AUTOBIN-THP. The loading of these Programs is also

very special.

After the successful loading, the Node outputs a Checksum (CRC). This

Checksum MUST correspond to that which will be ascertained from this,

or other Programs. On that TNN will have been accommodated.

(With SP the User Level for Upload must be different to 3 !)

(M)AILBOX <call>

----------------

Sets the Call of the Local Mailbox. Now the Mailbox must not be more

than a Component of the individual Node, but also it can be integrated

as an arbitrary Mailbox.

Page 29.

(M)AILBOX-

----------

Erases the set Call.

(MH)EARD

--------

-Sets the Counter position of the USER-Statistics on 0, but does not

erase the MH-List, which will be required with more User Ports.

(MH)EARD = <number>

Sets the MH-List on the <number> Call Signs. but it must be remembered,

that also it must be saved. Erasure of the MH-List always occurs, when

its size will be decreased. But then with that, the USER-Data of which

User is QRV on which Port will be lost.

(P)ARMS

-------

Displays the installed value of this Parameter as well as the

installable range.

(P)ARMS <number> <value>

Example: p 1 100

Sets Parameter 1 (Maximum Nodes) on 100. Quit for the alteration will

be with:

KS:DB0EAM> Parms:

01 Max-Nodes 100 (50...1000)

(PO)RT

------

<Port No.> <Command 1> <Command 2> .....

Valid as Port-Commands are:

(OF)f :

(TO)kenring

(VA)nessa

(SCC)- Baycom Card

(KISS1)..bis (KISS4)

(MO)de=<Speed und Flags>

3.) (US)er : (LI)nk : (SY)sop

4.)	(CT)ext=0 : (CT)ext=1

5.) (DA)ma=0 : (DA)ma=1

6.) (L2)digi=0 : (L2)digi=1

7.)	(TX)port=<Port No.>

8.) (NA)me=<name>

(OF)f :

Page 30.

Port switched off.

(TO)kenring :

-------------

Frames at this Port will be placed on the Tokenring-

Port. The Tokenring-Port lies default on COM 1, if it

is not defined otherwise in the START.BAT.

(MO)de:

------

No effect !

(VA)nessa:

----------

TNN is suitable both for the Tokenring and also for the

simultaneous Usage of Vanessa Cards. Frames to this Port

will be addressed, via that in the TNN-Soft implemented

Vanessa Driver, the corresponding Vanessa Card via the

Computer Bus and under (PO)rts the corresponding Port will

be recognised with "Vanessa" (However ONLY, if a VANESSA is

inbuilt also).

...(MO)de: <speed>d :

---------------------

...Full Duplex.

---------------

With the Vanessa Card the TX still remains keyed for

approx. 1 Minute after the last Sending. Further Frames

will be sent without TX-Delay. With a TNC in the Tokenring

this time  can be fixed in the Eprom.

...(MO)de: <speed>e : External Clock.

...(MO)de: <speed>m :

Dual - Speed - Port (Only possible with and on both Ports of

the corresponding Vanessa Card).

(SCC) :

------      Now the Baycom USCC-Card will be supported, if at first

only for TESTS, however only with SPECIAL Program Versions.

The DOS-Variable SCC = Base Address, IRQ must be set.

...(MO)de: <speed>c :

Software DCD for AFSK-Modems.

...(MO)de: <speed>e :

External Clock. (For DF9IC-Modem).

...(MO)de: <speed>z :

NRZ instead of NRZI. (For DF9IC-Modem).

Advice:

If a DF9IC-Modem should be connected to a Vanessa, so it

requires a NRZ-GAL ! With the NRZI-GAL the external Clock

should not function.

Page 31.

(KISS1)..(KISS4) :

-----------------

The Frames to this Port will be conducted via the often

implemented Kiss-Driver in the TNN-Soft, to that COMPort

ia defined in the START.BAT, via the Computer Bus. Under

(PO)rts, the corresponding Port will be recognised with

"Kiss1 .. Kiss4" and the Speed with (MO)de as well as the

CRC-method.

(MO)de:     <speed>c :

SMACK only with KISSLINK.

(MO)de:     <speed>r

RMNC-CRC only with KISSLINK (ET)hernet:

See Description in Appendix D:

(US)er:     MH List will be executed on this Port.

(LI)nk:     No Entries will be undertaken in the MH-List. The operation

of the MH-List on the Interlinks costs Computer time,

and the MH-List, which can undertake a maximum of 500 Call

Signs, will be invisible. It will be executed in order to

contain more User Entries with more transparency vie the

Entry AND with that, the NODE knows on which Port it must

connect to a definite User.

(SY)sop:    Sets the input Port on Sysop-Mode. This function should

help the Sysop to attend to maintenance. Without a Sysop

Privilege on the Port, the Node undertakes no Commands. The

adaptations during the operation can follow without the

User being Disconnected.

Semi-Duplex Links can be set on Sysop. Through that, an

unauthorised Entry onto the Links can be prevented. The

Level3-4 Link to the Neighbour remains undisturbed. Then

certainly the Conversd MUST be forced as active Level3-4

(CT)ext :   1= Ctext on, 0= Ctext off.

Generally the CTEXT.TXT will be sent. Additionally, if a

CTEXT.n (n=Portno) is available, this will be sent on the

corresponding Port. If CTEXT=0, an actual onhand

<call>.MSG will not be sent.

(DA)ma  :   1= DAMA on, 0= DAMA off

(WARNING: BURN THE DAMA-BIT in the EPROM of the Tokenring-Software)

(L2)digi

1= DIGIPEAT permitted,  0= barred.

When L2-Digipeating is allowed, all associated PORTS must

have this Bit activated. If a higher transfer level stands

available, this will be used automatically.

Page 32.

(TX)port :

----------

Under certain circumstances the TX-Port can be a Port, other than the

RX-Port. Example: From a Node, 2 End Digipeaters have been established.

But a TX could have sent the Data Recovery alone. So the Data can be

sent on ONE common TX-Port, but the receiving direction is separated.

(NA)me :

--------

Gives this Port a special designation. It may be a maximum of 10

Characters in length and serves for distinguishing the Ports. When e.g.

more User Entries or Baud Rates are to be used, in the MH-List it is

evident, which Call is QRV and on which Entry. Furthermore the Port-

Name will be used in order to send out a Connect on a defined Port,

dependent on the Entry in the MH-List. DB0EAM has a "70cm_1200" and a

"70cm_9600" Entry as well as on a 23cm a "23cm_9600". The Port-Names

are so installed. With "c DB0XYZ 70cm_9600" a Connect can be sent on

the 9600_Bit/s Port. If with the Connect, no Name is input Port 0 will

be used, if the Call is present in the MH-List with an Entry on a Port

other than Port 0, this Port will be used.

Note:

With EXTERNAL-Drivers, the Type of the Hardware will be considered

directly and is visible already after the Start of the Software under

the respective Port.

(PR)OMPT

--------

Without Text, displays the current Installation:

Prompt: %c de %d (%t) >

(PR)OMPT

Takes over the String as Prompt. Contains the entered String of the

Characters %c, %d, %t, %0, so these will be changed in the later Prompt

as follows:

%a Into the Alias of the Node,

%c Into the Call of the User,

%C Into the Call of the User with SSID,

%d Into the IDENT of the Node,

%D Into the IDENT of the Node with SSID,

%r Into a Return,

%t Into the current Time HH:MM,

%s Into the actual Date,

%0 Prevents the Sending of the Prompt.

PR =%c de %d (%t) > outputs:

DG9FU de DB0EAM (18:30) >

The entire PT-Net has become used to the use of UTC. This saves the

resetting of the Summer-/Wintertime.

Page 33.

(RE)AD

------

Serves for the Reading of a File. The Name must be input with

Extension. e.g.:

READ CTEXT.TXT or READ AKTUELL.TXT.

----------------------------------

Here Path Inputs are permitted. These are required, when a Text from

Drive A: is to be read and the DOS-Path is on a virtual Drive. e.g.:

READ A:\TNN\AKTUELL.TXT.

(READB)IN <filename.ext>

------------------------

Allows the Binary Downloading of Files. The Method is the same as under

LOAD, only in the direction from Node to the Sysop.

(RES)ET SYSTEM

--------------

Allows the Node Computer to execute a Cold Start.

(RES)ET <port>

--------------

Executes a Reset of the Port-TNC, / the corresponding Vanessa Card.

(RU)NBATCH  <filename.tnb>

--------------------------

Executes the input TNB-File and returns the Acknowledgement OK.

(S)TAT + <call>

---------------

Includes <call> in the Statistics.

(S)TAT - <call>

---------------

Erases <call out of the Statistics.

(SP)arm or better Save Parameter.

---------------------------------

Save Parameter creates a File PARAMS.TNB, which contains all current

installed Parameters. With a New Start PARMS.TNB can be used to copy

from TNN174.TNB, for the installation of the Parameters of TNN. As an

acknowledgement arrives "PARMS.TNB save ..."

(STAR)T <program>

------------------

Starts the <program>. The Program must be input completely, e.g.

TNNDOS.EXE. If the Program is not to be found in the actual Path,

naturally this must be given with input. The execution of the Start

operation, will no longer be displayed on the Screen, but written  in

the File STARTUP.LOG.

Page 34.

(STAR)T <program>

------------------

Does NOT function with the DPMI-Versions !

(SUS)PEND + <port> <call> <links>

---------------------------------

Limits the Call Sign <call> on Port <port> to a number of <links.

Example: Generally DB2OS should be able to construct only 2

Connections, regardless of which Port. Then simply, input under <port>

the fictional Port 255.

SUS + 255 DB2OS 2

-----------------

Now USER-PORT 0 should be completely excluded, so is input:

SUS + 0 DB2OS

A Level-2 exclusion follows with: SUS + 254 DB2OS

The Connection of the Node is possible first time. However after the

first I-Frame of the User it receives the File SUSPEND.TXT  and after

that a Disconnect.

(SUS)PEND - <port> <call>

-------------------------

Releases the Call Sign <call> on Port <port> of the User on the Node.

A PR-User has complained to the BAPT, that for him, the Operator of a

Digipeater still only allows one Connect with the Digi at the same

time. Because this Problem has been catered for frequently by

Discussions on PR. Here is the letter from the BAPT to the Digi-

Operator again in full:

Reference 123-8 B 3581-29 of the 28.01.1997

Limitation of the maximum number of simultaneous Connects by the

Digipeater DB0xxx

Dear Herr xxx,

As discussed on the telephone with you on the 27.01.1997, herewith, we

communicate to you, our fundamental declarations, which relate to a

complaint which has been made against you, by reason of the limitation

of your Digipeater to the appellant.

We consider the limitation not as an exclusion in the sense of the

arrangement.

Points 2.4.2.7 of the DV-AFuG. An exclusion on grounds of an

appropriate abuse of a Radio Relay Station does not apply. Much more,

here it deals with the limitation on operational grounds. We hold that

this limitation for one, for example on Capacity Grounds, a sensible

Measure in order to give the most possible opportunity to many Radio

Amateurs, to work with this Digipeater. It must remain the object of

the operator of a Digipeater, after a technical based evaluation, to

offer from it, the selection arrangement concerning the quantity of

Users. Furthermore we see no legal basis, on which in itself, to base

Page 35.

in the Amateur Radio Service a claim to be allowed for more

simultaneous Connections via a Radio Relay Station. For further

questions we await at your pleasure.

With friendly Greetings

By order

Jeromin (for Link)

(SY)SOP

-------

Outputs a 5 position Number Group for the Login of the Sysop.

PASSWORD-Input:

--------------

The Password input for the User has been expanded, so that Passwords

such as with BAYCOM-BOX are allowed. The old 5-Position Answers also

work. The Trick exists in that, the actual Password can be placed in a

long string of Data!

BAYCOM-PASSWORD:

---------------

The most important first, the Method is compatible with the old.

Without further ado simply send the 5 Answering Letters. But

additionally it is possible, to hide the Answer in an optional chance

string of Letters. If the Solution is e.g. 12345, the following Answers

are accepted as a correct Password:

54321532412345221134254235244242415526

-----

12345

-----

12131415234132145132412345

-----

Because the listener does not know, where the solution is, he cannot

put work out the Password. It is recommended that the Password is

included with the most possible different Characters, in order to raise

the concealment level

(TE)ST <port>

-------------

Outputs a 0/1 changes of 4 KByte on the Port <port>, for equalisation.

e.g. to the Modem. Only the Sysop can release the Test-Function. If the

User calls the Test-Function, he receives back a "Port disabled".

(TI)ME

------

Displays Date and Time of the Node.

Installation of Time and Date is only possible via the DOS-Command!

Time with:  DOS TIME HH:MM

Page 36.

But Caution! Only "DOS TIME" displays the actual Time of the Node on

the Monitor awaits the new Time from the Keyboard. Then the Node stands

on that, up to a RESET. This follows automatically after 2 Minutes

(approx) through the Software-Watchdog of TNN. A little thought should

given to this.

(TR)ace <Module><Level><Module><Level> <monitor>

----------------------------------------------

This Command serves for the recognition and the finding of Errors. The

following Module is available for selection:

F = Flexnet

N = Netrom

C = Convers

L = Level2

A = All Levels

For this purpose the Level is able to be installed with a Value between

0 (Off) and 9 (All). Example: (TR)ace A9

(ALL displays, indeed all)

Example:

(TR)ace F1N9

(FlexNet only a large Error, Netrom all)

The Level (Approximate value)

9 = Pure Information

7 = Important Value

5 = Important Occurrence

3 = None-Critical Error

1 = Critical (fatal) Error

0 = Off

Nowthe Monitor allows monitioring ONLINE for a Call or Port, without it

having to be written first in a File.

The Syntax is the same as with:

<ESC> MUSIC.

------------

Example: (TR)ace MUSICH 9 +DG0XX With <ESC>-Command it allows the

calling of a few Monitor Functions. Input thre is ONLY possible from

the Keyboard. To the Category "Keyboard Commands" also belongs an ALT-

Command.

<ALT> X

-------

"The special <ALT> X Combination"

Aborts the Program and returns onto the DOS-Level

<ESC> -----"The various <ESC> Keys"

<ESC> C

-------

Connect from the Keyboard as Host in the Node.

Page 37.

<ESC> D

-------

Disconnect

<ESC> E1

--------

Opens a new File. If the File exists already, it will not be erased, but

the Output will be appended at the End. Console-Output will be branched

to a File (YYMMDDHH.PRO).

<ESC> E0

--------

Switches off the Output and closes the File.

<ESC> I

-------

Installation of the HOST Call.

<ESC> K

-------

Outputs the Date and Time

<ESC> K0

--------

Monitor will be displayed without Date and Time.

<ESC> K2

--------

Monitor will be displayed with Date and Time.

<ESC> Logout

------------

Leaves the Console and closes.

<ESC> L

-------

Displays the Status of the Channel.

<ESC> M

-------

Displays the current Installation. The adjunct means that the complete

Frames will be monitored, the adjunct Header, that the Info-Part of the

Frames will be suppressed.

<ESC> MN

--------

Monitor off (Certainly, the more the Monitor has to display, the slower

the Node will be. Therefore, after usage always switch the Monitor

"OFF" !

With the Monitor Commands and a heavily loaded Node Computer, Caution

is offered. Because the Computer will be much slower, it will be

difficult to switch off the Monitoring again, because the Keyboard will

be requested infrequently.

Page 38.

<ESC> MUSIC H/F <port nr.> +/- <call..call>

-------------------------------------------

Monitoring of of:

U = Unprotocolled Packets,

I = Info Packets

S = Control Packets,

C = Monitor an existing Connect,

H = With I-Frames only display the Header or

F = I-Frames displayed with Info-Contents,

<port no> = Only the issued Port will be monitored,

+ <call..call> = Only these Calls (up to 8) will be monitored,

- <call..call> = These Calls will be suppressed in the Monitor,

<call> can be issued with SSID.

Under <call> Inputs are also possible such as QST and LSTAT.

<ESC> P

-------

Displays the actual Password.

<ESC> P 1234567890 etc. up to 80 Characters.

--------------------------------------------

Sets the Password to "1234567890 etc. up up to 80 Characters".

<ESC> R0

--------

There follows no Advice about Token-Recoveries.

<ESC> R1

--------

With every Token-Recovery the Text will be displayed on the Screen:

*** TOKEN-Recovery (1) Wed Jan 19 13:45:10 1994 ***

The number indicates how many Recoveries have intruded one after the

other, as well as Date and Time. This is the default Installation.

<ESC> S

-------

Displays the actual Channel.

<ESC> S1

--------

Switches to the Channel 1.

<ESC> T

-------

Shows the immediate installed Baud Rate on the Tokenring.

<ESC> T 9600 / 19200 / 38400

----------------------------

Installs the corresponding Baud Rate on the Tokenring.

Page 39.

<ESC> T 57600 / 115200

----------------------

Installs the corresponding Baud rate on the Tokenring. However for

these Baud Rates a FIFO IC (16550) is necessary.

<ESC> T ?

---------

Outputs a Help.

<ESC> Y0

--------

No Connect to the HOST is possible (default).

<ESC> Y1

--------

Releases connections to the HOST on all 4 Host Channels. The Connect to

the Host is ONLY possible with C WITHOUT the addition of a <call> !

(########.TNB) FILES

--------------------

.........a Tasty Morsel.

A large daily Problem: Always after 03:00 allegedly occurs a fearful

Problem at the Node, when OM AA0BB appears. The Node must be monitored

once, or  every night at 23:00, write the Statistics on the Disk, or

from 16:00 to 24:00 other Parameters should be given on the Entry. All

examples for the Usage of TNN-Batch Files.

The object is very simple. There are two sorts of Batch Files, which

differ through the Names: YYMMDDHH.TNB and YYMMWXHH.TNB. With that YY

is the Year, MM the Month, DD the Day, HH the Hours a X the Weekday,

at which the File should be started. With that, an extra File for each

Action need not be created, "#" may be used as space holder for a

Number.

Directory:

------------

The .TNB Files belong in the Directory, in which the TNN174.CFG has

been placed under Workpath.

Example:

TNN174.TNB

----------

Will be executed ONLY with a New Start of the Program.

########.TNB

------------

Will be executed with every New Start and at every complete Hour.

######23.TNN

------------

Starts every day at 23:00.

####0100.TNB

------------

Always Starts on the 1st. of each Month at 00:00.

Page 40.

94040100.TNB

------------

Starts on the 01-APR-94 at 00:00.

####W116.TNB

------------

Always Starts on Mondays at 16:00 (Sunday is Day 0).

94##W011.TNB

------------

Starts on each Sunday in the Year 1994 at 11:00.

The construction of the Files is simple: Everything stands in there, as

it would be typed at the Console. With a single exception: When a Batch

runs, a Login occurs automatically, and after the execution of the

File, the old Login Status is available again. Now with that, not only

the Screen at the Node will be filled, but also what remains for the

future, is added to a new Commend for the Console: Now with that, the

<ESC> can be written more easily in a .TNB File, permitted here also is

the substitution of a # at the first Position. In order to create a

monthly Statistic, the following File is required:

####0100.TNB

============

This File starts on the 1st. of the Month at 0:00.

#e1         ; Link the Output in a File.

Stat        ; Read out of the Statistics.

MH 500 +    ; Read out of the MH List.

#e0         ; Switch off the Linking.

Clear       ; Statistics erase.

MH -        ; Place User Statistics on "0".

After the ";" Commentaries are allowed. DOS Commands are also possible.

RUNBATCH < filename.tnb >

-------------------------

With this allows further callings of a TNB. With that it is possible to

split up the TNN174.TNB e.g. in a PARAM.TNB and an IPNUMMER.TNB.

EXTERNAL PROGRAMS/COMMANDS for the SYSOP:

----------------------------------------

(O)utput <External Command>

---------------------------

OUTPUT.EXE, now has become an individual standing Program, which the

Sysop can Call when required. The Call follows as with an Internal

Command. OUTPUT.EXE when it is to be found in the PATH SYSEXE, IS ONLY

able to be called by the SYSOP after he has successfully obtained the

Privilege. With Input of Output without Argument, the current

condition of the Portbits will be displayed.

KS:DB0EAM> IO-Port-Status

0  1  2  3  4  5  6  7

1  1  1  1  1  1  1  1

Page 41.

The Command Output is intended for Remote Control. The Data Lines of

the  Printer Port (LPT1) of the PC can be switched Bitwise ON or OFF.

After a Program Start, the Output Condition is "all switched on". The

Base Address of the Port can be requested via the Computer-BIOS. Also

if other than LPT1 is to e used, so it must stand with only 40:08 the

appropriate Address.

(O)utput <port_bit> <on_off> <port_bit> :

----------------------------------------

To command Data Bits (0..7).<on_off>  : new Level (0 or 1).

(CWER) <external command>

-------------------------

Is a small Program, which displays the Visiting Card of the User, which

he has created himself with /P in the Convers Mode.

(CWER) a

--------

Generates a List with all Entries.

(CWER) l <number>

Displays the last <number> of Entries (always the new ones will be

appended at the end).

(CWER) <call>

-------------

Also, the Place Holders "*" and "?" can be used with Connections. With

that, "*" stands for an optional number of Characters and "?" for only

1 Character.

(CPERS) (external Command)

--------------------------

Is still used for the maintenance of the "Visiting Cards" of the File

CONVERS.PRS. Now it has, now and then more Entries, which have arisen

"accidentally", and which the Sysop requires very much to remove, or

only to improve. The simple Call of the Program removes "Empty Entries"

without Text as well as "non-sensible" Texts, which e.g. only consist

of one Character. With every Calling of the Program, a File CONVERS.ALT

will be installed, in order to be able revert once again back to the

last alteration. Because through the Calling of this Program it can

lead to Memory-Space-Problems (CONVERS.PRS to large), the Program first

checks, whether the File can be loaded in the remaining Memory.

(CPERS) /sort

-------------

Sorts the Entries alphabetically by Call.

(CPERS) + <call> <text>

-----------------------

Replaces / creates the Call with the <text> in the File CONVERS.PRS.

With this, NO Call check executed !

(CPERS) - <call>

----------------

Removes the Call and the Entry out of the File CONVERS.PRS. Here also

Page 42.

NO Call Check, with that the non-sensible Entries can be edited.

(SETCALL)  (external Command)

-----------------------------

The Fields can be filled or altered with (SETCALL). SETCALL enables the

SYSOP to be able to grasp the Data Theme. For the Programs

SHOWCALL.EXE, SAVECALL.EXE and SETCALL.EXE in the Dir SAVECALL,

respectively is a File with the Ending .SXK. If this Language File in

.SPK is renamed, so it will be used instead of the internal Texts, what

that means however, is an additional HD-Access. If anyone should

translate these Files, I would be pleased for a return of them.

(SETCALL) <call> <field recog> <text>        (External Command)

---------------------------------------------------------------

The Fields can be filled or altered with (SETCALL)

SETCALL

-------

If the Sysop should enable to be able to access on the Data Theme: To

the Programs SHOWCALL.EXE, SAVECALL.EXE and SETCALL.EXE in the

Directory SAVECALL respectively a File with the Ending .SXK. If these

Language Files are renamed, so they will be used instead of the

internal Texts, which however means additional HD-Access.

(SETCALL) <call> <field-recognition> <text>

-------------------------------------------

-> SaveCall Version [010696] de DG3AAH <-

SETCALL CALL /N .............................. 30 Chars. for the Name

SETCALL CALL /Q .............................. 30 Chars of the QTH

SETCALL CALL /L ......                          6 Chars World-Locator

SETCALL CALL /D .......                         7 Chars for DOK input

SETCALL CALL /V .........                       9 Chars QRV on ... Digi

SETCALL CALL /M ....................           20 Chars Mybbs of Mailbox

SETCALL CALL /T .....  . . . . . . . .  ...... 40 Chars for free Text

( a * as Text erases an Entry )

SETCALL CALL /*    : erases the Contents of all Entries for Call.

SETCALL CALL /B    : blocks CALL for ShowCall.

SETCALL CALL /F    : releases Call for ShowCall.

SETCALL CALL       : outputs Info about Call, also when blocked.

( a B as Call outputs all blocked Calls )

On the Diskette is a Basic population of available Calls (from DB0EAM).

There are updates only against forwarding of the  individual Data

Themes and when anyone finds a SORT ROUTINE as well as creation of the

corresponding CALL.IDX, please write.

(SYST)ext  (external command)

-----------------------------

SYSText now offers the possibility, to insert a Line of Text in the

File SYSOP.PRO. With that now can be documented for example: Parameter

Alteration for the "other" Sysops and with that a certain history. The

File SYSOP.PRO will be visible, when the Entries of other are output.

Page 43.

New Starts (at the moment documented with b-log.exe) should be entered

in the SYSOP.PRO.

(SH)ow_SYS (external command)

-----------------------------

SHOW_SYS displays the Entries in the File SYSOP.PRO in a synoptic

Form.

(SH)ow_SYS /lSHOW_SYS

---------------------

Displays the Entries in the File SYSOP.PRO in a synoptic Form and with

that erases all Privileged-Entries.

(MSY) (external command)

------------------------

MSY.EXE in SYSEXE is a Help Program for the Sysop, that belongs to

MSG.EXE in USEREXE.

(MSY) C

-------

Lifetime of all Messages will be reduced by 1. This can occur manually

or with a TNB-File.

######01.TNB

============

This File starts daily at 01:00.

MSY C

-----

Lifetime of the .MSG-File set back one Day and removed with Lifetime =

0.

(MSY) D

-------

Lists the Headers of all Messages.

(MSY) D -#  or +#x

Messages, whose Lifetime is (-) smaller or (+) larger x-Days.

(MSY) D -*x or +*x

------------------

Messages, that are (-) younger or (+) older than x-Days.

(MSY) D FROM

------------

Displays the Message List from Sender Calls.

The Sorting can be considered also according to :

BYTE, DATE, TIME, LT, RETURN.

(MSY) E ALL <call>

------------------

Erases all Messages from or to <call>.

Page 44.

(MSY) E 01.01.95/00:00:01

-------------------------

Erases all messages with this Dates/this Time.

(MSY) G <group> +/- <call>

Ordains/erases <call> from <group>. Allowed in a Group are a

maximum of 50 Calls.

(MSY) L

-------

Shows all Message Files.

(MSY) S <call> #x <text>

------------------------

Writes <text> to <call> with Lifetime #<x>. Here a Lifetime is a

maximum of a possible 99 Day.

(MSY) V

-------

Output of the Version Number and the Date.

(SYSH)elp (external command)

----------------------------

Is an external Command for a new Help Function. Functions as with

(H)elp will be accessed on the OHS.TXT (Online-Help-Sysop).

Help Program for the SYSOP at Home:

-----------------------------------

(MAKEDAT.EXE)

-------------

If you offer Sat-Data on the Digipeater, you must maintain this also!

(In the other case, you should erase the SAT.EXE in USEREXE). Instead

of the previous 22 Satellites, DB1HZ has now enabled 40 calculations in

the new SAT.EXE. Furthermore now the Data no longer stands in

individual File but now it is only the SAT.DAT in the Directory TNN to

be maintained. Consequently the Directory DATEN can be removed. Now

also a possible available CO-Processor supports the calculation, which

positively influences the Calculation Times. The previous available

Environmental Variable SET SATQTH=<Locator> or <coordinates> is

removed. Now there will be used the already available Variable QTH. The

Program MAKEDAT serves for the changing of the Packet-Radio distributed

Kepler Data in the AMSAT- or 2LINE-NASA-Format, in the Data Theme

awaited from SAT Master. More Information for MAKEDAT can be obtained

by calling the Program without Parameter.

For the Execution:

------------------

At home on the Computer open a File SAVEFILE e.g. with the Name KEPLER.

Then with <read kepler>, you can read all of the new Data Texts and

Programs that are available there. After the closing of the SAVEFILES,

now will be called MAKEDAT /l KEPLER.TXT. MAKEDAT updates only the

Entries of the SAT, which are entered in the File SAT.FLT into the File

SAT.DAT. Finally load the File SAT.DAT <load sat.dat> as a Program in

the Node .... it is ready!

Page 45.

(TNNSET.EXE)

------------

Stems from IK7NXQ and the creation of EPROMS should be eased for the

Tokenring. Simple call once .. it self illustrates.

Commands for all Users.

-----------------------

(BE)ACON

--------

Displays the installed Beacon Parameter and Beacon Text.

(C)ONNECT.

----------

If it is released with <ESC> Y 1, connect to the HOST-Interface. If Y

is set on 0, at the Terminal a CONNECT REQUEST fm <call-ssid> <date

time> will be displayed.

(C)ONNECT <call>

----------------

With the Connect-Command a Connection to another Node or another User

will be constructed. The Input CONNECT DB0FD or abbreviated/lower case:

c db0fd causes, that first of all, it will be sought from the List of

the known Targets to the Call DB0FD. It deals with a Call, that is in

the NODES-List, so the Report will be output:

Interlink setup (via call) and will be displayed in Brackets to which

direct Neighbour the Connect will be sent. If the to connecting Call

is not available in the NODES-List, the Destinations-List will be

searched. If there is an Entry to this Target, so the User receives a

Flexgate setup (Portname)..., and the Link will be constructed in this

direction. If this Call is to be conducted locally or also as BOX-Port,

it gives either a: BoxLinkSetup (Portname)... or LocalLink setup

(Portname)... If no Entry is available, so the MH-List will be

searched. If there is an Entry available there, so the Connect will be

sent on the corresponding Port. If no Entry is available there, there

only remains to Send the Connect on the Port 0. Then always, there will

be the Report Output: Downlink setup (Portname). On the Report is

recognisable already, and with thta also able to be evaluated by

Router, whether a Connection to to a TheNet / TheNetNode-Nodes can be

constructed. If a Connection has been constructed to a TheNet - or

TheNetNode-Node, the Report will be output with the ALIAS, insofar as

it is available:

BS:DB0FC> Connected to H:DB0FD  otherwise only : BS:DB0FC> Connected to

DB0FD.

A User can be connected via a Digipeater. The Input then is : CONNECT

DB3AN via DB0FD. The word via can be omitted or optionally be

shortened. Also permitted are up to a possible 8 Digipeaters. Then the

Call Signs are separated by Empty Characters. But this possibility only

functions when, L2-Digipeating is switched on by the corresponding

Digipeater. Possible Error Reports are:

Page 46.

Failure with ...   : The required Partner has not reported himself.

Busy from ...      : The Partner has declined a Connection

Construction.

Node not available : The Target admittedly, is known but it at the

moment no Connection.

The following Reports display, that the Node, to which the CONNECT-

Command has been sent, is abandoned and therefore the Connection will

not be constructed:  Node busy, Link table full, Circuit table full. A

Connect-Command can be aborted through an optional Input, or an empty

line is adequate. Also in the Connect-Command, the Port-Name can be

included. Valid Port-Names can be obtained with the (PO)rt-Command. On

the Multi-Baud Entrance of DB0EAM are incorporated, and others, the

Port-Names "70cm_1200", "70cm_9600" and "23cm_9600". If in the Connect-

Command the Port-Name is included, the Connect will sent out on

this Port without consideration of an Entry in the MH-List.

Format: C DB6XYZ 23cm_9600.

If a Connect is input without an Argument, a Connection Construction

will be sought to the HOST-Terminal. However that is only possible,

when the Sysop has released Connections to the HOST-Terminal with the

Command <ESC> y1 and that it is not occupied, otherwise it gives a :

Busy from <call>.

LOOP DETECTED is a Warning, that the selected Connection Construction

builds a Loop. Therefore, an Interlink-Connection will be occupied in

both Directions via the same Neighbour-Node and consequently not

required. Contrary to other Node-Software however, there follows only a

WARNING, nevertheless, the Connection can be constructed. WARNING:

loop detected via neighbour: DB0.. (Help Loop) But the last is not

always sensible and is disabled by one's self. Please return with 2

Quits to the penultimate Node and then the Connection constructed

directly...

(CONV)ers and its Commands.

---------------------------

Switches into the Convers-Mode. The Convers-Commands stand under -

(CONV)ERS - internal -.

(CONV)ers <Channel>

-------------------

Switches into the Convers-Mode on Channel <Channel>.

(CONV)ers C(stat)

-----------------

Displays the existing Connections, Runtimes, Data-quantities etc.

Host     State    Quality Revision  Since NextTry Tries Queue  RX  TX

(CONV)ers O(nline) [q ; l ; a]

------------------------------

Displays the User of Convers. Additional Options such as [q ; l ; a],

are possible. Now, if a Convers-LOOP is noticed, the particular Link

will be taken out of service for one hour. Nevertheless LOOPs should be

prevented. The indication of the Entries such as (DB0GOE on port 6) are

Page 47.

visible after Sysop Privilege has been obtained.

Host      State       Quality Revision  Since NextTry Tries Queue  RX    TX

db0goe    Connected    1s/1s  pp-3.06t  17:55                 0  136K  267K

(DB0GOE on port 6)

db0ii     Connected   10s/7s  pp-3.06t   7:07                 0    1K   48K

(DB0II on port 9 via DB0BID)

db0kh     Connected    1s/1s  pp-3.06t   6:13                 0   17K   65K

(DB0KH on port 11)

db0gso    Disc./locked   ---             9:22  10:22          0

(DB0GSO on port 9 via DB0BID)

1 loops detected.

DB0KV     (dw-0.84k)  2m   DB0NOE  (dw-0.82b)  5m   db0acc  (pp-3.12f) 25s

db0ais    (pp-3.12f)  3m   db0ber  (pp-3.06t)  3m   db0bhv  (pp-2.93t)  1m

db0bid    (pp-3.06t)  9s   db0bro  (pp-3.06t) 12s   db0cl   (pp-2.93t)  2m

(CONV)ers - intern -

--------------------

The CONVERS-Mode by TheNetNode.

-------------------------------

Since Conversd v2.14 the possibility exists, to Net Convers-Nodes one

with another, i.e. a Convers-User must not connect via a long

Digipeater Path up to the Convers-Node, on which its required Opposite-

Partner is to be found, but it is sufficient, if a Connect is

constructed to the next Convers-Node, which the Conversd-Protocol

supports. Unfortunately this Version tends to create Loops with the

Connect-Construction. Since TheNetNode V1.50 (PC) an essentially

improved Convers-Mode has been implemented. It is a question of

Conversd PingPong-Release 3.12, developed by DK5SG, improved further by

DC6IQ and inbuilt by DL1XAO into TheNetNode.

With the v3.12 (pp-conversd) the Networking now functions without

Problems. Furthermore a multitude will be offered additional Comfort,

as e.g. Personal Texts, Channel Themes, Channel Options and Umlaut

changer. Also older Conversd can be integrated, only practicallt this

does not understand all Commands / not completely, they act as a Filter

for the new Functions. Between the Convers-Hosts all Text, which the

various Users write, will be separated no longer for each User, but

only transferred once. This considerably unburdens the Link Stretches,

because e.g. For 10 Users one and the same Packet must be transferred

via the Inter-Links not 10 times, but only once! Furthermore, the

Convers-Entry-Area naturally has become substantially larger, and from

that it is possible to go out, because now frequently is to be found a

Conversation-Partner in the Convers.

Page 48.

In the CONVERS-Mode the following Commands stand for selection (the

Commands can be shortened by the Usage of the Upper-Case Letter(s)):

/Away [Text]             Marks you as absent.

/ALl[Text]               Text to all Users of your Channel.

/Beep                    Beep-Mode on/off.

/Channel [n]             Combines you additionally with  the Channel n.

/CHARacter               Sets various Character exchanges.

/Destinations            Lists attainable Ping-Pong Hosts.

/Help [Command]          Gives Help-Information.

/EXClusiv User Text      Sends Text to all on your Channel except User.

/Filter [Calls]          Sets Calls, whose Text should be filtered.

/Invite User             Invites User onto your Channel.

/Links [args]            Lists or sets (Sysops) conversd-Partner.

/LISt                    Lists all Channels and their Themes.

/LEave [Channel]         Leaves Channel or current Channel.

/MsgUser Text            Sends Text to User or.

/Msg#Channel             Sends Text to the given Channel.

/ME Text                 Sends an Action Text.

/Mode [Channel Options   Sets Channel Options.

/NOtify [Calls]          Sets Calls, whose appearance should be

reported.

/Personal [Text]         Sets personal Desciption.

/PRompt abcd             Prompt sets (a=Query; b=Normal; c=Ctrl-g;

d=Ctrl-h).

/Quit                    Leave Convers.

/QUEry [User]            Starts/Ends Private Conversation.

/Topic [#Channel] [Text] Sets Theme of the Channel. Theme=@ removes.

/UPtime                  How long has this conversd run already ?

/Verbose                 Verbose-Modes on/off.

/VERSion                 Shows Info for this Version.

/Who[*; A; L; Q]         Shows Users and their Channels

(*=owner; A=absent; L=length; Q=short).

/WIdth [Value]           Sets/Shows Line Length.

The individual Explanations:

/ALL

----

When you are in the /query Mode, Text will be with prefixed /all

behaves as if you had worked without /query.

/AWAY <text>

------------

/away sets the Absence Text, which the others can read. With the Call

without Argument, the Text will be erased and can be validated again

as present.

/BEEP

-----

With this the Bell Character (CTRL-G) which can be sent before every

Communication, switched on or off. This Command is actually a Sub-Set

of the /prompt Command, see there.

Page 49.

/CHAN

-----

Additionally combines you with the selected Channel. Contrary to older

conversd-Implementations you still remain in the previous Channel, then

it will support a Multiple-Channel-Connection. In order to leave, you

must use "/leave". Without Input of a Channel the Info will be output

on which Channel you yourself are to be found.

/CHAR

-----

With this Command you can inform the Convers, which Character-Method-

Change you must have. The Syntax is: /char [In-Type [Out-Type]] if you

e.g. work with an Atari, you can Input "/char pc atari". If you use a

PC and choose to write Umlauts in the TeX-Stil, input "/char tex pc".

"/char ?" lists the possible installations.

The Installation will be saved with "/pers" (see there).

The thanks for this nice Function goes to Thommy, <dl9sau@,thynet.sub.

org> (Internet mail) <dl9sau@db0sao.ampr.org> (AmPR-Net mail).

Recommendations should be forwarded to him.

Possible Installations with /char:

iso-8859-1, ansi, 8bit dumb, ascii, none, us tex

ibm7bit, 7bit, commodore, c64, digicom

roman8

ibmpc, pc, at, xt

atari,

binary, image.

/DEST

-----

All Ping-Pong-Hosts, which are combined one with the other, will be

listed. The Numbers show the Answer-Times in Seconds.

/DEST <call>

------------

Requests the Path to the call> and with that shows the transfer time.

/EXCL

-----

This Commando is the opposite to the /msg-Command. With this you send

Text to all Users of this Channel except to the one input as the first

Parameter. Because the Text will be sent out internally as Private

to the others, the Links will be loaded somewhat more.

/FILT

-----

If you do not want to read the Text of a particular User, with this

you can insert in the List. Then all Texts will be filtered out, with

personal Text ("/msg") a return Report will be sent to the Sender. The

Set/Erase occurs as with "/notify", therefore e.g. "/filter + dc1ik -

db4ut" sets dc1ik and erases db4ut out of the List.

Page 50.

/HELP

-----

The Help Command can be followed by additional Parameters. Here the

Backslash may not stand before the problematical Command, e.g.: /Help

Invi. ALL Help Texts can be read as a complete overview, outside the

Convers-Mode with "Help conversd".

/INVI

-----

It will send an invitation to a known User. This invitation will be

conducted through the entire Net. If anyone is on another Channel and

your Channel is installed as Private, he can change onto your Private

Channel. If he is in the Command Interpreter of a Node, he receives

the Invitation, but then he cannot come directly onto your Channel, on

account of which he is invited once again.

/JOIN

-----

Additionally connects you with the required Channel. Contrary to older

conversd-Implementations, you remain in the previous Channel, then it

will support a Multi-Channel-Connection. In order to leave a Channel,

you must use "/leave". Without input of a Channel, Info will be output

to the Channel used by you.

/LEAV

-----

With this Command you can either leave the current or the Channel that

was input. If this is the last, the conversd will be left.

/LINK

-----

The immediate Link-Status will be displayed. Normally these are

Hostname, Link-Status, Runtimes, Version-Codes and Status-Time,

followed by the Time of the next Connect-Request and Number of the

Requests (on Disconnects or in the Construction of to be found Links),

with existing Connection the Queue-Length and Byte-Statistics will be

displayed. When you are the Sysop, you can set or erase Connections.

Then they will be displayed also, still in Brackets.

Syntax: /l [[-] Host [Port [via]]]

/LIST

-----

All Channels, their Themes, Options and Users will be displayed. The

Bracket Values mean:

(@) = Channel Operator,

(G) = With AWAY reports absence,

(!) = User is in the Sysop-Mode.

/ME

---

This Command serves for the purpose, of displaying the Users' activity

on your Channel. If you input e.g. "/me leaves", all Users of this

Channel receive the following:

Page 51.

*** dc6iq leaves

/MODE

-----

The Mode-Command is one of the most complicated. It will be used as

follows:

"/mo [<Kanal>] <+ ; -> <t ; i ; s ; m ; p ; l ; o <User>>".

The Options mean as follows:

t = The Theme of the Channel ONLY allows changes form Channel-Sysops.

i = The Channel will hide Users of other Channels.

s = The Channel is Secret, the Channel Number will be displayed no

longer.

m = The Channel is modified, only Channel-Sysops may write.

p = the Channel is Private, an invitation is required to Login.

l = The Channel is Local, Texts will not be distributed further.

o <User> = Makes <User> to be Channel-Sysop (no - possible).

The Plus sets an Option, Minus erases it. Combinations are allowed, so

e.g. "/mode 69 -s+todc6iq" would cause the following: Channel 69 is no

longer secret, but the Themes may be set only by the Channel-Sysop.

Additionally, dc6iq will be a Channel-Sysop.

/MSG

----

Sends a Text to a special User or to a connected Channel. If the Text

should go to one Channel, the following must be input:

"/msg #<Channel> <text>".

When the Target is a User, he can recognise the Text by the additional

asterisks. e.g. when dc6iq sends a Message to dc1ik with "/m dc1ik This

is a Test", dc1ik obtains the following: "<*dc6iq*>: This is a Test".

/NOTI

-----

In Convers, you will be informed when a certain Person appears in the

Persons-List. e.g. insert "/notify + dc1ik" dc1ik in the List, "/notify

- db4ut" remove db4ut out of the List. The Insertion/Removal Command of

more Calls in one Command is possible, e.g: "/notify + dc1ik db4ut -

dc6iq dh2paf + dg3kcr", causes that dc1ik, db4ut and dg3kcr will be

inserted as well as dc6iq and dh2paf being removed. The removal of

Calls, which do not stand in the List, will be ignored.

/PERS

-----

A short description about yourself can be set, which the other Users

can see with "/who". e.g.: "/pers Fred, Buechig, JN49fb".

Without Text, the description will be erased. This Implementation can

be up to 118 Characters for the Description and then places this

automatically with the Login (The "/char" and "/width" will be

Installed and then alo saved and set with the Login).

Page 52.

/PROM

-----

The prompt-Command takes four Arguments in a sequential Character-

Chain.

"/prompt abcd" sets:

a = As "/query"-Prompt.

b = For the normal Prompt.

d = Is a Character, in order to remove the Prompt (normally Backspace)

c = Is a Character, which will be sent before every Text that you

receive, (normally therefore CTRL-G).

/QUER

-----

The User that was input is in future the individual Receiver for all

Texts, which you issue. Then this will be sent as Private Text to the

User, as with "/m". For the switching off, Call without Argument, after

that all goes again to the Channel as accustomed. So as to say: a

Private-Mode.

/QUIT

When you input that, you leave this wonderful Ping-Pong-Convers. I hope

it pleases you.

/TOPI

-----

With this, a Theme can be set for the Channel. The other Users can see

this, when they input "/who quick" or "/list". If no Channel Number is

input, the Theme of the Active Channel will be set. If a Number is

given, you must be logged in on this Channel. In order to erase the

Theme a "@" is installed as Theme.

/UPTI

-----

This Command displays, how long conversd has been active already.

/VERB

-----

Switches the Verbose-Option on/off. Then you receive a lot of

Information about Actions of the User (Login/Logout/Text-Setting/...),

also when these are not on your Channel.

/VERS

-----

Shows some Text about this Version (in English).

/WHO

----

This Command has 4 Options.

a = Shows all Users and their Default Texts, if set.

l = Generates a LONG List with Personal-Descriptions, Default Texts and

Source Information.

q = Outputs a short Listing.

Page 53.

* = Shows all Users of your Channel.

When you require Information about a certain User, you can use the

"/who u Userlist" Variant.

/WIDT

Makes known your Screen Width to conversd (Characters/Lines). Then the

Reports of the others will be brought to the width. 80 is preinstalled.

The Installation is saved with "/pers" (see there).

(!) <TheNetNode-Commands>:

--------------------------

With a preceding ! it is possible, to call the TheNetNode-Commands from

the Convers.

(CQ) <text>

-----------

Through the Input of CQ a CQ-Call can be started via every TheNet-Node.

CQ Text... (Text up to 75 Characters), however no additional

Digipeaters possible. TheNetNode can manage more CQ-Calls, however only

one Uplink/ Circuit. How do I start a CQ-Call ?

Assumed, DB2OS in Hannover wishes to lay down a general CQ-Call with

the Nodes in Brunswick. First of all he connects to BS:DB0FC and then

inputs there, the CQ-Command. cq cq de DB2OS HANNOVER JO42VG/EM60G VIA

BS -- PSE CONNECT DB2OS-15

IMPORTANT: With a following Command or a RETURN the CQ will be aborted!

VARIANT A:

----------

OM Karl, DK7AL, is connected at the same time with the Node BS:DB0FC

and now with the Input of the USER-Command sees the following List:

BS:DB0FC> TheNetNode 1.70 (731)

Uplink (DF3AV)                <-->      Circuit (BS77:DB0FC-8  DF3AV)

Uplink (DF2AU)                <..>      Downlink (DF2AU-15  DK4EG-1)

Circuit (H:DB0FD  DB2OS)      <..>      CQ (DB2OS-15)

Uplink (DK7AL)               "<..> CQ(DB2OS-15)"

displays, that DB2OS (arriving from Node H:DB0FD) seeks a Connection in

the area BS and has presented the CQ-Command. DK7AL must input to this

place only "C DB2OS-15" and is IMMEDIATELY connected with  DB2OS!!! The

troublesome retracing of the Connection of the Connection Path not

completely applicable / is not necessary.

VARIANT B:

----------

OM Wolfgang, DB3AN, monitors the Frequency and suddenly sees the

following Packet on the Screen (NORD><LINK Firmware):

fm DB2OS-15 to CQ ctl UI^

Page 54.

CQ de DB2OS HANNOVER JO42VG via BS -- PSE CONNECT DB2OS-15 !!!

+----------------------CQ CQ CQ...if necessary. with Text...

+------------------------------Sender, OM Peter, DB2OS

This Packet has been radiated by BS:DB0FC as UI-Packet, immediately

after the receipt pf the CQ-Command (with Text behind). In order to

answer this CQ-Call, Wolfgang must not connect first to the Node BS,

but it gives its individual TNC the Command for the Construction of a

Connection with DB2OS-15 (Connect DB2OS-15, observe the SSID !), as if

DB2OS-15 lives directly in the Neighbourhood. After BS:DB0FC has

received the SABM-Packet from DB3AN, IMMEDIATELY after the Connection

is made with DB2OS, and with DB2OS shows the Report:

"BS:DB0FC> Connected to DB3AN".

As can be seen, the Connection Construction can be conducted on the

User Level or direct after the "Sighting" of the CQ-Call.

(D)EST

------

Displays the Entries of the NODES-List in which with the RMNCs is

customarily known.

Destinations (5):

DB0BID  0-0    DB0EAM  3-3    DB0EAM  4-4    DB0NHM  0-0    DB0NHM  4-4

1)               2)

1.)  Attainable Targets (Destinations),

2.)  SSID-Range of the Calls.

(D)EST <*>

----------

Lists as above, however additionally with Run Times.

(D)EST <call*>

--------------

With this, the Call must not be complete. The Input of (D)est HB9* to

the number of all HB9.. Destinations. They will be displayed with Call,

SSID-Range, Runtime and Port, on which they are attainable.

(D)EST <call>

-------------

Displays the List as with (N)odes <call> .... only for the benefit of

the Flexnet lover.

(D)EST < <Neighbour Call>

Displays the Nodes / Destinations, which reports this Neighbour with

the best Runtime.

(H)ELP

------

Output of a Help Text.

Page 55.

(L)INKS

-------

Shows the incorporated Call Signs, to which the Links are ordered.

PORT-Type-ALIAS-CALL------via Digipeater---------

4  L+  KSDXC DB0EAM-4

5  L+  KSBOX DB0EAM-3

6  N    GOE7 DB0GOE

9  F     BID DB0BID

9  N   SARTG HB9AK     via DB0BID

Type:

L  = Local Entry

L+ = Local Entry will be measured

F  = FlexNet-Neighbour

N  = NetRom-Neighbour

(M)ailbox

---------

If a local Mailbox has been incorporated, the input of an <m> passes a

Connect to this Mailbox. It is the sense of this Command, that it must

not always seek on a Node of the next lying Mailbox.

(MH)EARD <expansion>

Outputs an actual Call-Sign-List of the last 10 heard Calls with Date,

Time, Port-Name, RX-Byte and TX-Byte. Since Version TNN14919, the MH-

List will secure the Statistics on the current Disk-Drive. Now it

serves for the purpose, to connect a User on the Port, on which it has

been heard last.  The SSID will be observed here. It is possible

therefore, to be QRV with the Call DB0XY-1 on the Port 0 and with the

Call DB0XY-2 on another Port QRV. Who is seldom QRV, after some time or

other falls out the List.

<expansion>

-----------

As an expansion a "+" can be input. Then an expanded User-Statistic

will be output. It exists from the User received Rej = (r) sent to the

User Rej = (t) as well as the number of the DAMA-Offences = (d).

KS:DB0EAM> MHEARD (500/1728)

19.03.94 13:34 P 9 [   927365,  1932892] DG9FU      126r   119t     0d

19.03.94 13:33 P 0 [    59149,  40744�5] DG2ACM      45r    28t     0d

19.03.94 13:33 P 0 [    24821,   856196] DK7VW       42r    23t     0d

19.03.94 13:33 P 0 [    40804,  23170�4] DG2ACM-1    49r    44t     0d

19.03.94 13:33 P 0 [      880,    43047] DK3ZL        4r     0t     0d

DG9FU de DB0EAM >

MHEARD (<number>/<number>)

--------------------------

Both Numbers in the first Line indicate:

Page 56.

1. <number> = Length of the heard MH-List,

2. <number> = Number of the free Buffers.

(MH)EARD <number> <extension>

-----------------------------

Outputs an actual Call Sign List of the last <number> Calls with Date,

Time, RX-Bytes and TX-Bytes.

(MH)EARD <call> <extension>

---------------------------

Lists when and under which SSID the User with the <call> has last used

the Node. Furthermore displayed with the RX-Bytes and TX-Bytes (out of

the sight of the Node), which have accumulated since the last erasure

of the MHEARD-List or  alteration of the number of List Entries.

In the <call> also Wild Cards can be used. With that "*" stands for an

optional number (or none) Characters and "?" stands for only 1

Character.

(MH)EARD df6ln = Entries with the Call Sign DF6LN

(MH)EARD df = Entries of DF-Stations

(MH)EARD *b? = Stations with "B" as the penultimate Letter

(MH)EARD *b* = Stations with "B" in the Call Sign

(N)ODES

-------

Displays all immediate known Nodes, which use the NET/ROM- / TheNet-L3-

Protocol. The List Output as a rule will be contained at the latest

Level via so called Broadcasts of the TheNet-Neighbour-Nodes. The List

alters itself therefore, when Links fall over or new Links arrive.

Furthermore alters for itself the Runtimes of the individual Entries

in dependence of the Link-Loading and with bad Propagation Conditions.

Example:

1    2             3    4

/    /             /    /

KS:DB0EAM > Nodes (139/1009):

SH9600:DB0AZ

HUSUM: DB0HES

HHWEST:DB0HHW

HL:    DB0MAR

KIELMB:DB0OQ

SL:    DB0SUE

SYF7:  OZ3DIJ-7

/      /

5      6

1.) Alias of this NetNode.

2.) Call Sign of the NetNode.

3.) Number of the known Node-Entries.

Page 57.

4.) Maximum possible number of Entries.

5.) Alias of the End-Node.

6.) Call Sign of the End-Node.

With the ALIAS is a maximum of a 6-position abbreviation of the best

geographical arrangement of the Node-Location. Normally in our area the

customary Car Registration (:- translator's explanation :- B = Berlin,

HH = Hanseatic Hamburg, H = Hannover etc.) will be used as ALIAS or

short place names written (as e.g. with KS:DB0EAM  or KIEL:DB0IL). As

a rule with the Connect-Command, a Connection can be made to an End-

Node in the List. End-Nodes, whose ALIAS end with "BOX" or "MB" are

recognised as Mailbox; DX-Cluster are with "DX" or "DXC" at the End of

the ALIAS. The number of the known Net-Nodes will displayed in brackets

behind "Nodes". The Nodes-Command can be abbreviated to the first

Letter and also written optionally Upper / Lower Case. The Nodes-

Command can be called with Parameters, in order to obtain Information

of an individual End-Node or Groups of End-Nodes. With the Command:

(N)ODES <call> or (N)ODES <alias>

is obtained a List out of the known Paths to them with <call> / <alias>

of the given End-Node. So is obtained e.g. with "(N)odes DB0FC". An

additional "*" behind <call> displays all Paths.

1   2

Routes to  BS:DB0FC

--Qua---RxQ---TxQ---Mode-Timeout--RTT---Po-Route---------------------

3110  3110  1690  VC   0        60    9  DB0BID

3577  2685  6938  DG   55       892   9  HB9AK via DB0BID

6938  3255  3577  DG   60       3683  9  DB0LBG-7 via DB0BID

> 1630  1630  3480  VC   0        47    10 DB0NHM

^   /     /     /   /   /         /     /    /

3  4     5     6   7   8         9     10   11

1.) ALIAS-Recognition of the requested End-Node.

2.) Call-Sign of the requested End-Node.

3.) > = the Path at this time will be used for a Connection to the End-

Node.

+ = Displays an alternative Route, via which the Data Transfer takes

place.

- = This Route is withdrawn and will be used no longer.

* = Obs will no longer be executed, because the Neighbour supports

different Broadcasts already.

4.) Runtime in in *10 ms of the respective Path.

5.) RxQ Runtime which has reported from the Neighbour via the Target

DB0FC resp. when it is a direct Neighbour, the ascertained Runtime.

6.) Runtime with which the Target will be reported further. (Calculates

itself from RxQ ad measured Runtime to the Neighbour-Node).

7.) Transfer-Mode on this Link DG = Data Gram;  VC = Virtual Circuit.

8.) Timeout for ????????

9.) Runtime to the Neighbour-Node ?????????

10.) Port via which the Connection will be routed.

11.) Call-Sign of the Neighbour-Node for the respective Path.

Page 58.

To obtain expanded Information about a Group of End-Nodes, input the

Command:

(N)ODES <runtime> <name> ?????? gives the still / again.

For <runtime> will be input, the Value for the minimum Runtime of the

End-Node. If it should not be a minimum, but a maximum Runtime input,

after the Runtime a Minus will be written. For <name> a Call-Sign or an

ALIAS will be input, with the "*" and "?" can be used as place holder

With that "*" stands for optionally many (or none) Characters and "?"

stands for only 1 Character. Furthermore the Request of the ALIAS-Part

or of the Call-Sign-Part of the Node-Entry can be abbreviated, in which

sets a ":" before or after <name>. With "<name>:" the request is

limited to the ALIAS-Part, and with ":<name>" the request is limited to

the Call-Sign-Part of the Node-Entry. If only a <runtime is input, all

Nodes with acceptable Runtime will be displayed. If only a <name> is

input, the Runtime will not be considered, the Runtime by choice not

considered. The output List yo the End-Node appears e.g. so:

KS:DB0EAM > Nodes: (7)

HHS:DB0HBS  176/11 HHSBOX:DB0HBS-1  69/11

/     /    /  /

1     2    3  4

1.) ALIAS of the End-Node.

2.) Call-Sign of the End-Node.

3.) Runtime in 10 ms steps for the best known Path.

4.) Port for Neighbour-Node.

Example:

Command        ! Answer

==============!========================================================

NODES 200     ! All Entries with Runtime minimum 200.

NODES -60     ! All Entries with Runtime maximum. 60.

NODES 100 d*  ! Call-Sign or ALIAS begins with "D", Runtime min. 100.

NODES -100 *d ! Call-Sign or ALIAS ends with "D", Runtime max. 100.

NODES *d*     ! Call-Sign or ALIAS contains at ein "D", Runtime

! optional.

NODES ?????   ! Call-Sign or ALIAS is only 5 Characters in Length.

NODES :d*     ! Call-Sign begins with "D".

NODES :?b*    ! Call-Sign with "B" as 2nd. Letter (DB- & HB9-Stations)

NODES :hb9*   ! HB9-Call-Signs.

NODES :*l     ! Call-Sign ends with "L".

NODES k*:     ! ALIAS begins with "K".

NODES *box*:  ! ALIAS contains "BOX".

NODES *dx:    ! ALIAS ends with "DX".

NODES *u?:    ! ALIAS with "U" as penultimate Letter.

NODES *       ! Displays the entire List.

(N)ODES < <Neighbour Call>

--------------------------

Displays the Nodes / Destinations, which report with the best Runtime

from this Neighbour.

Page 59.

(P)ARAMETER

-----------

Output of the Parameter-List.

Common Parameters for all Ports

-------------------------------

KS:DB0EAM> Parms:

01: unused      0        02: Min-Quality 59       03: unused      0

04: Infocast    600      05: Broadcast   300      06: Lifetime    30

07: T-Window    10       08: NoAckBuf    10       09: Timeout     3600

10: SaveConfig  6        11: Paclen      236      12: DAMA-Speedf 0

13: unused      0        14: DAMA-MaxPri 10       15: DAMA-MaxPol 10

16: DAMA-Tout   5        17: unused      0        18: unused      0

19: CommandLog  0

01: unused

02: Min-Quality

Minimum Quality for automatic Update of the End-Node-List. Only Nodes

with this minimum Quality will be entered in the End-Node-List.

Automatically the End-Node-List will always be at the latest position.

In order to include in this list only sensible Paths with certain

limiting Quality, Paths with a Limited Quality (= this Parameter) will

be included exclusively. With 0 the automatic Update is switched off

completely.

03: unused.

04: Infocast  ????????????

--------------------------

Minimum Counter Setting of the Out of Date Counter, with that the

Stretches to the End-Node will be conveyed automatically of other

End-Nodes. Only attainable End-Nodes with an Out of Date Counter

Position greater/same as this Parameter will be conveyed with the

automatic Update. This Parameter prevents the conveyance of Out of

Date Stretches to other Nodes. Should never be greater than Parameter

(03: Obs - Init ), otherwise only the individual Node will be conveyed

to the others.

05: Broadcast

-------------

End-Lists-Auto-Update Broadcast Interval in Seconds, 0 = switched off.

At these Intervals, the individual known End-Nodes will conveyed to

others and the Out of Date Counter reduced for all known End-Nodes.

With 0, no conveyance occurs, to others of the attainable End-Nodes.

06: Lifetime

------------

Counter starting position of the Network Packet-Lifetime-Counter. All

Network Layer-3/4 Packets have a Lifetime-Field, in which will be

given, how long this Packet has to live. It will indicate, how often

Page 60.

this Packet may be forwarded through a Node. With each forwarding

through a Node the Field will reduced by 1, when it reaches 0 the

Packet will be forwarded no longer to the next Node, but certainly

still to the individual Level 4, in the case defined for that. With one

of one's own Node generated Packets, the Lifetime-Counter will be set

on this Parameter. The Counter should prevent, that a Packet will be

forwarded eternally through Loops in the Network. It should be set a

little bit higher than the number of the Nodes on the longest known

Path.

07: T-Window

------------

Transport Window Size in number of Frames. This Parameter displays the

maximum number of Frames, which may be unacknowledged with a Level-3/4

Connection. The actual used Number corrects itself from the lowest

installed T-Window-Value of the associated Nodes.

08: NoAckBuf

------------

Overfill-Limitation-Value in number of Frames. Number of Packets, that

will be saved intermediately on the Transport-Layer-level, up to a

Choke-Message being sent to the intermediate Node. At the same time the

number of Frames, which will be intermediately saved in the Link-Layer,

before the Link-Layer goes into the Busy-State. This Limiting-Value **

prevents the overrun of a TheNet-Node in the case where too many

Packets enter via the Transport-Layer, of the case where a Station in a

Link will send to many Packets at once.

09: Timeout

-----------

No-Activity-Timeout in seconds, (0 = switch off. On a Layer-2-Link or

Layer-4-Circuit as long as no Information-Exchange occurs, the Link

will be broken off.

10: SaveConfig

--------------

Time-Space in 10 min-steps, in which TNN174.STA and MHEARD.TAB will be

saved on Diskette or HD, with the actual Statistical-Data.

11: Paclen

----------

Paclen only acts on Packets, which will be self generated from the

Node. e.g.: Texts in the Node. Installed not larger than 236!

12: Dama-Speedf

---------------

TNN-Digis with Multi-Baud (e.g. 1200 and 9600 Baud) state likewise a

further wish, to create an opportunity event for 9k6-Operation. In the

normal case 1200- and 9k6-Users handled simultaneously, and one can

prefer exclusively via MAXFRAME the 9k6-Operation somewhat.

Conventionally a on 9K6 a MAXFRAME 7 should be used and not more than

MAXFRAME 2 on 1200 Baud. But for some Sysops this was still not enough.

What the new DAMA-Speed-Factor effects, should be explained now by the

following Example. Assumed, we have a 1200-Baud- and a 9600-Baud-Entry,

Page 61.

and we want to give distinct preference to the 9600 Users. If DAMA-

Speedf now stands on 9600, when the User on 1200-Baud will be served

first in every 8th. cycle. At first sight this appears too slow, but

with 9K6 the Frames are certainly so short, that therefore it does NOT

go similarly slowly at Factor 8. If there are mainly no 9k6-Users, then

all is as before.

Important: The correct Baud-Rate must be incorporated correctly in the

Speed-Parameter for each respective Port.

Die "Retardation" calculates itself so:

DAMA-Speedf     9600

Retardation  = ------------ = ----- =   8

Speed[Port]     1200    ===

With the 9k6-Port it establishes itself correspondingly on a Factor of

1. If at the same time there is a 2k4-Entry, then the retardation there

is 9600/2400 = 4. If the 1200 Entry is not made similarly to Factor 8,

naturally an optional Value between 1200-9600 (or more!) can be entered

with Parameter 19. A Value smaller than the lowest Baud-Rate

respectively a Value of 0, switches off this Function.

13: unused

----------

14: DAMA-MaxPri

---------------

Maximum Counter Setting, which can be achieve with low Activity of a

User. Then this User will be requested less seldom whether it has Data

to send to favour the other Users,

15: DAMA-MaxPol

---------------

0 = switched off. x = after x Offences a throwing out follows from the

Dama-Master.

The TNN-Software offers for the SYSOP a disabling DAMA-Control. All

Users, who do not observe the DAMA-Protocol, with each DAMA-Offence

receive an included corresponding WARNING. If, the number set by the

Sysop is overstepped, then follows a throwing out from the Digi. New

is, that the "Beefing Report" will be sent out no longer as a UI-Frame

(Beacon), but now will be appended in the middle of the existing QSO.

The warning displays the number of the actual Offence as well as the

quantity of the Offences, up to the Disconnect of the DAMA-Master (with

corresponding Report).

16:DAMA-Tout

------------

Time in 10 ms, which will be awaited, if no acknowledgement from a User

arrives, up to sending to the next User in the sequence.

Page 62.

17: unused

----------

18: unused

----------

19: CommandLog

--------------

Now creates the possibility, to create a Log-File COMMAND.LOG in the

Directory TNN. All Inputs in the Node will be saved in the File with

CALL, DATE, TIME and COMMAND. There are the following possible

Installations:

0 = No Protocol.

1 = Only Sysop-Input will be Protocolled.

2 = All Inputs will be Protocolled (Caution, the Log-File grows fast!).

Parameters for the respective Port

Port #   MTU  TXDelay  P-Per  Slot   IRTT  MaxFr L2Retry T2Timer

-----------0-------1-------2-----3------4----5-------6-------7--

20:  0   256      28     255    10    700    2      10    1000

30:  1   256      10     255     0    200    7      30      50

40:  2   256      10     255     0    200    7      30      50

50:  3   256      18     128     5    400    5      30     100

Port:

-----

The 10 Positions of the Parameter-Column.

The First-Position begins to count with 0.

The MTU of Port 0 is to be altered therefore, with 'P 20 0' on to the

Value 256.

#: The number of the respective Port.

MTU: Maximum Transmission Unit (64-952).

Maximum Info-Field size of the Frames on this Port. At the moment, for

AX25 the Info-Field-Size is limited to 256 Bytes. If the

Node sends to an AX 25-receiver, an Info-Frame with more than 256

Bytes, this Frame will be answered with a Rej. More than 256 Bytes at

the moment will be worked only from TCPIP-Stations. This Parameter

effects only TCPIP-Connections.

TXDelay:

--------

Pre-Send wait time in Milli-seconds *10, after the keying of the PTT up

to the sending of the Data-Packet.

P-Per:

------

Page 63.

P-Persistence-Value (0-255).

----------------------------

Slot:

-----

Time-Slot-Interval in 10ms. This Parameter displays the duration of the

Time-Slot for the P-Persistence-Control. Everytime, when the TNC will

be radiating a Packet and which lays under Slot-Time, executes the

described chance number outside of the P-Persistence-Range, then will

be awaited for the duration of the Time-Slot and finally renews the P-

Persistence-Procedure.

IRTT:

-----

Means Initial-Round-Trip-Time and is the Start-Value for the RTT-

Calculation, therefore will be the value for the first calculation

employed by SRTT, if still no Measurement could succeed.

MaxFrame:

---------

User-Link MaxFrame in Number of the Frames. Number of the Info-Packets

on Layer2-Level, which without contents of an Acknowledgment may be

sent out following one another.

L2Retry:

--------

User-Link RETRY, 0 for unending. Determines the number of the Requests,

in order to receive a Contact to another Station on the Layer2-Lever

(Answer to Command and Poll). After this number of requests the Link

will be reported as defective, in the case where no Answer follows). 0

installed on a 24h-QRV-Station would lead to Lynch justice, it is warned

therefore expressly about that!

T2-Timer:

---------

User-Link T2 in Milli-Seconds * 10. This Timer determines the Waite-

Time, after which a detailed Information Packet will be acknowledged

with an RR/REJ/RNR-Packet. On the one hand this delay is for the

increase in Throughput, because it gives others in this Interval to

Send, on the other hand, the Send-Layer will given the Chance, to pack

an Acknowledgement in a to be sent Info-Packet and consequently to

save a Link-Layer-Packet. (the result is to be read in the Statistics)

(PI)NG <idappr>

Forwards a Runtime-Request to the IP-Number.

(PO)RT

------

Outputs a List with the actual Port installation

:KS:DB0EAM> Link-Interface Ports:

Page 64.

Port--#-Name------Type---Speed/Mode----Flags----------------Hardware--------

00  00 1200_B     it/s   User  1200   CTEXT  DAMA -------- Vanessa (V 4.04E)

01  01 Goettingen Link   9600d        -----  ---- L2-Digi  Vanessa (V 4.04E)

02  02 Paderborn  Link   9600d        -----  ---- -------- Vanessa (V 4.04E)

03  03 Northeim   Link   9600         CTEXT  ---- L2-Digi  Vanessa (V 4.04E)

04  04 Knuell     Link   9600d        -----  ---- L2-Digi  Tokenring

05  05 Lemgo      Link  28000de       CTEXT  ---- L2-Digi  Tokenring

06  06 9600_Bit/s User   9600         CTEXT  DAMA  ------- Tokenring

07  07 Mailbox    Link  38400         -----  ----  ------- IPX_PD

08  08 Cluster    Link  19200         -----  ----  ------- Kiss1

Port--#:

Indicates the RX-Port and # the TX-Port of the Node. It could be

imagined that it can send to 2 Link Partners via a Port (Radio), but

will be received on separated Ports.

Name:

For an easier overview.

Type: Displays the usage of the Ports whether User, Link or Sysop.

Speed/Mode:

Speed Displays the installed Baud Rate as well as additional set Flags.

e :

External Clock with VANESSA,

c :

SMACK only with KISSLINK,

d :

Full Duplex,

m :

Dual speed on a Vanessa Card,

r :

RMNC-CRC only with KISSLINK.

Flags Ctext:

Whether, and on which Ports will be sent.

Flags Dama:

Whether, and on which Ports the DAMA-Operation Method is activated.

Flags L2-Digi:

Whether the L2-Digipeating is allowed on the corresponding Port.

Hardware Tokenring:

Serial transfer on the Tokenring.

Hardware Vanessa:

Parallel transfer to the Front end Computer Vanessa, with the Software

Version.

Page 65.

Hardware Kiss1:

Transfer via the installed Serial Port.

Hardware IPX_PD:

Here is an Ethernet-Card in Use with an IPXPD-Driver.

(Q)UIT

------

Connection will be released from the Node (DISCONNECT). By that it

enables the possibility, for the previous connected Node to be

reconnected (bonded back). The Text QUIT.TXT will be sent before the

release of the Connection.

(R)OUTES

--------

Displays the existing Routes to the TheNet-Neighbour-Node.

Routes of KS:DB0EAM (18/32)

Node----Type-Delay-Po-Dst--ForwD-RevD----BroInt---State--Route--------------

DB0EAM-3  L+ 17     5   1     17/0                conn.

DB0VFK    L  400    3   1      0/0

DB0GOE    N+ 62     6 126     40/85      488/600  conn.

DB0BID    F  63     9 544     45/82        4/+    conn.

DB0II     N+ 0      9  86      0/5846    292/600  conn.  DB0BID

DB0KH     N+ 631    9 102    759/503     455/600  conn.  DB0BID

DB0KH     N+ 57    11 121     43/71      498/600  conn.

Routes of KS:DB0EAM (18/32)

The Route List is shortened in the essentials! The (18/32) implies,

that in the Link List 18 from a 32 possible Entries are used. The

length of the Link List if necessary can be adjusted with SET

TNNCFG=xxxx,xx. (see: START.BAT)

NODE:

-----

Displays the Call Sign of the adjacent TheNet Node. LOCAL is a fixed

installed Level2-Connection, which sends out no BROADCAST itself. This

can be e.g. a Mailbox or a Cluster. Which combinations are installed as

LOCAL can be requested with the (LO)CAL-Command). LOCAL-Connections are

always unused, because only Level2 takes place there.

Type:

-----

Input, with this Entry, Type with which it deals. Possibilities:

Type L:

-------

Local Call and Alias will be incorporated with a Runtime 4000 ms. There

will be no implemented Runtime Checking.

Page 66.

If a L+ is input, the Target will be connected every 5 Minutes. If

the Target is QRV, this will be incorporated in the Route List with the

Runtime and reported further.

Type F:

FlexNet Port works with FlexNet Protocol.

Type N:

NetRom-Neighbour can return the older Level-3/4-Protocol Method,

however the Measurement Frames is still unprotocolled.

Type N+:

NetRom-Neighbour employs the new Protocol, unattainable Targets will be

reported immediately. The detection and Measuring now occurs

protocolled in the Level-3/4.

Delay:

Runtime to the Neighbour. Average Value from ForwD and RevD.

Po :

Port, via which the Connection is running.

Dst :

Shows the Targets from the Neighbour which will be reported.

ForwD :

Ascertained Runtime from this Node.

RevD :

Ascertained Runtime from the Opposite side.

BroInt :

488/600

488 ms before the next Broadcast. 600 ms Broadcast Interval.

State setup :

-------------

A request to construct a Connection to the Neighbour.

State conn. :

-------------

The Connection to the Neighbour has been constructed successfully and

remains in existence from now on. ONLY when this Level2 Connection

exists, the NODES / DESTINATIONS will be accepted from this Neighbour.

With that, Target Nodes will be removed out of the Net which are not

certain to be reached or to which the Link only exists in one

Direction.

Route :

Digipeater, which does not use the TheNet-Protocol to the incorporated

TheNet-Node.

Page 67.

(R)OUTES (V)ersion

------------------

Additionally, displays the Software level of the Neighbour Node and

what type of a Broadcast will be made.

info-broadcast = Old TheNet Broadcast,

UI-broadcast = Broadcast will be secured in the Level 3 transfer,

differential-broadcast = Only the alterations will be transferred.

(S)TAT

------

Outputs the Statistics, which will be executed in the Nodes.

System Statistics: 15.02.97 09:27:44

Startup: 14.02.97 16:29:29?

Uptime:        0/16:58

(min)  (now)  (max)

Rounds/sec:     469   1526   2489

Tokens/sec:      23     33     51

Free Buffers:  6500   6863   7950

Overall Throughput:         21856  27680 Baud

Active L2-Links:            36     63

Active Circuits:            12     24

CPU load: 39%

TOKENRING load: 36%

Buffer usage: 14%

Network Heap: 649832 Byte

Free RAM (Fmem): 8982528 Bytes

Token ring speed: 38400 Bit/s.

Token-Recoveries: 3    15.02.97 09:00:41

Invalid Frames: ZC: 1  V1: 0  V2: 0  I: 0    15.02.97 09:27:37

Bad Frames: 29    15.02.97 09:00:40

Port 0:<Portname>: RX =     1907248 Bytes, TX =     5889122 Bytes.

Port 3:<Portname>: RX =     5851605 Bytes, TX =     9747530 Bytes.

Port 4:<Portname>: RX =    12238752 Bytes, TX =    22324486 Bytes.

Port 7:<Portname>: RX =    55577546 Bytes, TX =     7490000 Bytes.

Port 8:<Portname>: RX =     3304795 Bytes, TX =     1281052 Bytes.

Total =      229961 KBytes.

Bytes total          I        RR       REJ       RNR   %RR  %REJ  %RNR

Link to DB0GOE           06.03.94 06:38:27 - 06.03.94 21:03:18

RX:    4373648      44196     23645      2154       188    30     2     0

TX:   14631652      78768     27066       238       220    61     0     0

Page 72.

Link to DB0AX            06.03.94 06:38:27 - 06.03.94 21:03:17

Page 68.

RX:    1295328      16120     14472       636         0    42     1     0

TX:    6824718      33662     11907         6         0    73     0     0

Link to DB0NHM           06.03.94 06:38:27 - 06.03.94 21:03:15

RX:     931191       5796      9626       663      1124    61     4     7

TX:    2915931      15564      6895       407         3   118     7     0

IP-Gateway-Statistics:

ipForwarding:      1      ipDefaultTTL:      10     ipInReceives:      0

ipInHdrErrors:     2      ipInAddrErrors:    0      ipForwDatagrams:   2

ipInUnknownProtos: 0      ipInDiscards:      0      ipInDelivers:      0

ipOutRequests:     1      ipOutDiscards:     0      ipOutNoRoutes:     2

ipReasmTimeout:    30     ipReasmReqds:      0      ipReasmOKs:        0

ipReasmFails:      0      ipFragOKs:         0      ipFragFails:       0

ipFragCreates:     0

System Statistics: 15.02.97 09:27:44

Displays Read out Date and Time. Important with automatic requests of

the Statistics for Diagrams.

Startup: 14.02.97 16:29:29

Last New Start of the Program.

Uptime = 0/16:58

Runtime of the Program in Days/Hours:Minutes.

The following Inputs are respectively the: minimum Value, actual Value,

maximum Value Rounds/sec:

<number> <number> <number>

Displays the main Loop throughput of the Program.

Tokens/sec: <number> <number> <number>

Displays the activities of the Tokenring.

Free Buffers : <number> <number> <number>

Displays the free Buffers of the Node.

A Buffer is 64 Info-Bytes and 8 List Pointer-Bytes, therefore, 72 Bytes

altogether. Each intermediately saved Information occupies Buffers, the

List of other known Nodes occupies Buffers. The sense of this number of

free Buffers communicated to the User, is that it has no purpose, with

a very mean number of free Buffers (< approx. 300) to probe Connects.

Then when TheNetNode has no more free Buffers for selection, a Reset

will be released, which deletes all existing Connections with a total

loss of Information. But in normal Operation this case is near to

exclusion, because with the normally set TheNetNode-Parameters,

actually too few Buffers cannot be available (one cannot optionally

unload many Packets one behind the other in the Nodes. CPU load: 70%

Behaves between minimum and instantaneous Processor loading in %.

Page 69.

Overall Throughput : <number> <number> <number>

Displays the Data throughput of the Node. With this it handles pure

Info Data without Protocol- and Overhead Bytes!

Active L2-Links :

<number> <number>

Displays the number of the active and maximum simultaneous Level-2-

Connections.

Active Circuits : <number> <number>

Displays the number of the active and maximum simultaneous Circuit-

Connections. Naturally only the Connections which begin or end on this

Node will be displayed. Certainly all other Connections will be

forwarded through the Net and do not require to be managed on every

Node.

CPU load : 39%

Displays the Loading of the CPU.

TOKENRING load : 36%

Displays the loading of the Tokenring.

Buffer usage : 14%

Ratio between occupied and existing available Buffers in %.

Network Heap: <number>

Free RAM (Fmem) : <number>

Free Main Memory for DOS. With the DPMI-Version of the total free

Memory (up to 16 MB).

Tokenring speed: 38400 Bit/s.

Display of the installed Baud Rate of the Tokenring.

Token-Recoveries: <number> <date>

Requests to the TNC, which are not returned back to the Computer. They

will be displayed with Number, Date and Time.

Invalid Frames: ZC: ..  V1: .. V2: ..  I: .. <number> <date>

Incomplete Frames, whose Headers are not intact. They will be displayed

with Number, Date and Time.

They are split up into various Groups.

ZC: = End Call defective,

V1: = To many "VIA" Calls,

V2: = "VIA" Call defective,

I: = Infofield defective.

Bad Frames: <number> <date>

Defective or false Token. They will be displayed with Number, Date and

Time.

Only the INFO-Byte will be included in the Statistics!

Page 70.

Port 0:<Portname>:RX = <number> Bytes,  TX = <number> Bytes.

On Port 0:<Portname>: RX received Bytes, TX sent Bytes OF ALL

Connections on this Port.

Total = <number> Bytes.

Total of all received and sent Bytes.

Bytes total    I       RR       REJ       RNR   %RR  %REJ   %RNR

Inscribed Line of the Link Statistic Count RX / TX:

RX / TX:

<number> Bytes total

Received Bytes,

<number> I

Received Info Frames,

<number> RR

received Acknowledgement-Frames,

<number> REJ

received Reject-Frames,

<number> %RR

Received RR Frames/sent I Frames in %,

<number> %REJ

Received REJ Frames/sent I Frames in %,

<number> %RNR

Received RNR Frames/sent I Frames in %.

%RR:

It is not every individual I Frame that has to be acknowledged but it

is possible to acknowledge multi-I-Frames with one RR-Frame. The T2-

Timer conditions this. But on the other hand the opposite station may

not poll too fast. This Poll-Frame will be acknowledged again with an

RR-Frame.

%REJ:

Gives Information about the Quality of an Interlink.

%RNR:

Gives Information about the handling speed of the Opposite Station. But

RNRs also indicates, when the Opposite Station has too little Memory.

%:

Currently ALL Frames running on the Interlink will be evaluated. If the

Level-2-Traffic is released, naturally therefore REJ, RNR etc.will be

entered from the Level-2-Connections. This difference can be read from

the Statistics of DB0EAM. On the Interlink to DB0AX contrary to the

Interlink to DB0GOE the Level-2-Traffic is not allowed. To DB0AX grasps

the Control Functions of the Level-3/4 Protocol. Only very seldom does

it come to an RNR- Frame. Here on the other hand, the consequent HF-

Page 71.

Technical Construction can be read (no kind of mutual influence of the

InterLink) of DB0EAM. If as a general rule, 2.4% of the Packets from

DB0AX have been rejected, there is almost 0% in the opposite direction.

On the Interlink to DB0KH only the blocking of the Level-2 could be

detected. If the Statistics of DB0NHM and DB0AX are compared, (indeed

there are only 23 Hours, but tend to be constant), the aggravating

drawback of a Level2-System such as FLEXNET can be seen clearly. The

large number of RNRs will be carried over ALL hops, via which the

Level-2 has been constructed. On the Interlink to DB0AX Level-2-

Digipeating is barred, and with that controls the Level-3/4 Data Flow.

Just disadvantages the behaviour of the acknowledge condition of

FLEXNET-Neighbour.

Link to <call> <date> - <date>.

This Call-Sign-Statistic displays, when the Statistics were erased

last, and when the last Byte was received.

(S)TAT (L)ink

Only outputs the Link-Statistics.

(S)TAT (R)ing

Only outputs the Statistics of the switched on Ports (formerly Ring-

Addresses).

(S)TAT (x)

x=other Characters. Displays only the first six Lines of the

Statistics.

(TA)LK

------

<call>

Switches a TALK-Connection to the User with the <call>.

As an acknowledgement of the receipt when the User is connected with

the Command Interpreter:

You are now talking with DL0EAM. Leave this mode with /q.

All further inputs are received by the User as:

Msg from dl0eam:

text............ However this is only a one sided Connection.

(TA)LK <call> <text>

With TALK <text> a User can send a Text Line to a User, when this is

connected with the Command Interpreter of the Node. If no Connection

exists to the User from the Command Interpreter, the Text Line will be

raised to a Reconnect If the Connection is released through a

Disconnect from the User, the Text Line will be treated  .....

professionally and without penalty.

(TI)ME

------

Outputs the System-Date and Time.

(U)SER

------

Page 72.

After the Input of U (for USER) appears e.g. :

KS:DB0EAM> TheNetNode (PC) V1.70

Uplink(DK3NZ-5)                      <-->  Downlink(DK3NZ-10 DB0EAM-3)

Uplink Northeim via DB0NHM

Circuit(LOH:DB0VFK DL2FAI-1)         <..>  Downlink(DL2FAI-14 DB0EAM-3)

Uplink(DB0OVA-15)                    <-->  Downlink(DB0OVA DB0EAM-4)

Uplink Knuell via DB0KH DB0SHI

Uplink(DL8FAJ)                       <-->  Downlink(DL8FAJ-15 DB0EAM-4)

Uplink(DD1SI)                        <-->  Downlink(DD1SI-15 DK5ZK)

Uplink Knuell via DB0KH DB0SHI DB0DQ

Downlink 9600_Bit/s

Convers(DB0EAM-1) Host               <-->  Circuit(GOE7:DB0GOE DB0EAM-1)

Host(DB0EAM)

KS

Is the "ALIAS" of the Node, a type Pseudonym to which this ALIAS should

offer Geographical Information, which enables simple ordering of known

Calls to an area. For the others enabled contrary to the IDENT (Call

Sign), is the ALIAS a multiple Connect of a Node, then can be e.g. :

KS, KS-1, KS-2, etc.

Simultaneous connects, but only once DB0EAM.

The other possibility, Multi-connect to forward to a TheNet-Node is the

simultaneous operation of the individual Call Sign with different

SSIDs. But currently that cannot be done with every Software. In DL the

Auto Identification of the Main-Entry-Domains as ALIAS will be used,

short Place-Names can be used advantageously, through which the

Recognition value can be increased further. Other Alternatives such as

e.g. Post Codes or WW-Locators are very impressive.

The employment of the ALIAS, instead of the Call Sign, is NO abuse of

the Call Sign, because the ALIAS certainly has done nothing to the

International Norm for Amateur Radio Characters corresponding

construction (have should...). Just as little as a Call to CQ or Test

is no Call Sign abuse or the 4-Character Call by Amtor. The required

Call-Sign-Recognition will be transmitted every 10 Minutes from the

Net-Node with a Beacon transmission to ID, in which the employed ALIAS

will be imparted. Furthermore, the employment of the ALIAS in the AX25-

Address Field remains limited on End User, between the Nodes themselves

the official IDENT (Call Sign) will be used. Also it allows

identification of every TheNet-Node available the INFO-, USER- and

NODES-Commands. A united usage of Auto-Identification in Node Lists

Page 73.

elevates the transparency of the Net substantially.

DB0EAM

Is the official IDENT (Call Sign) of the TheNet-Node.

Uplink <call>

Displays at the Node, that the User with the Call Sign <call> has

entered in the Net. Available Digipeaters will be displayed in the next

Line.

Downlink <call>

Displays, that the User with the Call Sign <call> has left the Net.

Available Digipeater will be displayed in the next Line.

Circuit <alias:ident>

Means that on the left Side, the user of the Node with <id> has

connected from this Node and is connected from there with the Call Sign

<call>.

Circuit <alias:ident>

Means that on the right side, that the Connection to the Node with the

<id> has switched and the User there uses the Call Sign <call>.

Host

Means a Connection to the Operating Terminal of the Node.

CQ <call>

Can only appear on the right side and shows, that the User calls "CQ".

In order to construct a Connection to it, the <call> with the SSID must

be input in the Connect-Command.

<-->

Displays an existing Connection.

<..>

Displays an existing in construction Connection.

An Entry without "right Side" means, that the Connection ends at the

moment and the User is connected with the Command Interpreter of the

Node.

Uplink @ DB0FD

Displays the Entry-Node of the User, insofar as it has not already gone

out of the Circuit-Line. Then this is always the case, when multiple

Level4-Connections(Circuits) are constructed one after each other.

Downlink Port_5 via DB0XYZ or Uplink Port_5 via DB0XYZ

Displays the Downlink-Path, which the User has taken. In this case

>Port_5< stands for the Port-Names of the corresponding Port (see

(PO)rt Command and is a Level2-Interlink.

(U)ser h

As (U)ser, additionally displays the Convers-Host-Connections.

(U)ser + displays the Level2-Connections in Table Form......

Page 74.

L2 -User:

PO SrcCall   DstCall  LS  Rx Tx Tr SRTT   RxB      TxB  Baud  ConTime Pri

-------------------------------------------------------------------------

1 DB0EAM    DB0GOE   IXF  0  9  0  79 2109928 22685912 1413  38:59:23  -

3 DG9ABF-15 DB0EAM-3 IXF  0  0  0 755    4583        9  253   0:02:25  -

3 DL1DAQ-13 DB0EAM-3 DBS 10  0  0 674  259713      188  200   2:52:29  -

4 DB0RDX-15 DB0EAM-4 DBS  8  0  0 172   50776     6325   15   8:06:15  -

5 DB0EAM    DB0AX    IXF  0  0  0  62 2273953 21069538 1330  38:59:17  -

6 DB0EAM    DG9XYZ   IXF  0  4  0 370  451216   189109  377   3:46:25  0

6 DB0EAM    DG9XYZ-1 IXF  0  0  0  96  104530     2955   33   7:11:00 20

/    /         /      /   /  /  /   /     /         /     /      /     /

1)  2)        3)     4)  5) 6) 7)  8)    9)       10)   11)    12)   13)

1) User Port for the simple overview.

2) Source Call Signs of the L2-QSOs.

3) Target Call Sign of the L2-QSOs.

4) L2-Link-Status:

SET= Link-Setup,

FMR= Frame Reject,

DRQ= Disconnect Request,

HTH= Waits for Connect to a Digipeating-Target,

IXF= Info Transfer,

REJ= REJ Send,

WAK= Waiting Acknowledge,

DBS= Device Busy,

RBS= Remote Busy,

BBS= Both Busy,

WDB= Waiting Ack And Device Busy,

WRB= Waiting Ack And Remote Busy,

WBB= Waiting Ack And Both Busy,

RDB= REJ Send and Device Busy,

RRB= REJ Send and Remote Busy,

RBB= REJ Send and Both Busy.

5) Number of the received Frames in the Wait Queue for this Link.

6) Number of the still to be sent Frames in Wait Queue for this Link.

If a Link to a Neighbour collapses, simply this will have been broken

off with more than 100 outstanding Packets in the L2.

7) Number of Retries.

8) Position of the 'Smoothed Round Trip Timers' (in * 10 ms).

9) Number of received Bytes from the existence of the Link.

10) Number of Bytes sent from the existence of the Link.

11) Actual effective Baud Rate for this Link.

12) Connect Time in HH:MM:SS and with over 99:59:59 Hours then only as

DD:HH.

Page 75.

13) With DAMA-Net Entries:

Actual Priority of the User (0: = highest Priority).

.......and also the Level4-Connections.

L4 - User:

Call      Node    Route(s) S   Rx  Tx Tr Win   RxB      TxB   Baud   ConTime

-------------------------------------------------------------------------

DB0RDX    DB0EA   DB0KH    I.  0  10  0  10    6336   102292   29   8:06:24

DJ3WY     DB0FD   DB0GOE   I   0   1  0  10       0       32    0   0:06:29

DOBJ-15 DB0HAN +DB0GOE  .I   0  10  0  10      42  1255996 1123   2:29:05

/       /       /    /   /   /   /  /       /    /       /     /       /

1)      2)      2a)   3)  4)  5)  6) 7)     8)   9)     10)   11)     12)

1) Call Sign of the User.

2) Call of the Node with which the User is connected.

2a) There exists an alternative Path. BOTH Paths will be used for the

Data Transfer!

3) Call of the Neighbour-Node via which the L4 goes.

4) L4-Circuit-Status:

S= Circuit-Setup,

I= Individual Node stands in Choke (Data Transfer arrested),

I.= The Target Node stands in Choke,

I = Normal Info-Transfer,

D= Disconnect-Request.

5) Number of the received Frames in the Wait Queue for this Circuit.

6) Number of the still to be sent Frames in the Wait Queue for this

Circuit.

7) Number of Transport-Retries.

8) Transport Window Size.

9) Number of received Bytes since the existence of the Circuit.

10) Number of sent Bytes since the existence of the Circuits.

11) Actual effective Baud Rate for this Circuit.

12) Connect Time in HH:MM:SS and with over 99:59:59 Hours then only as

DD:HH

(U)ser (L)inks

Displays in Table Form ONLY the Level2-Connections.

(U)ser (C)ircuit

Displays in Table Form ONLY the Level4- (Circuit -) Connections.

Page 76.

(U)ser <Port No.>

Displays in Table Form ONLY the Level2-Connections of the input Port.

(V)ERSION

Outputs the actual Version Number of the Software.

(V)ERSION +

-----------

Displays the Version and the Compiler switches.

At the moment, the possibilities are:

DIPROUTE

IP-Router incorporated

DPACSAT

Pacsat Server incorporated

DSYSPORT

Port Mode Sysop is released.

EXTERNAL Command for all Users

------------------------------

Now in Principle, the File Name can be freely selected. Text Name and

Program Names however vary MORE than by the Extension. A Program

MSG.EXE and a Text file MSG.TXT are not possible, in this case only the

EXE-File will be found.

Now under external Commands all Texts, which are to be found in the

PATH TEXTCMD. Through the Editing of the CTEXT.*** the Users now can

be offered further Texts. For the standardisation the following

designation should be maintained please:

(A)CTUAL (external Command)

Outputs actual Information.

(HA)RDWARE (external Command)

Outputs a Hardware description of the Node.

(I)NFO (external Command)

Output of an Info-Text.

(MA)P (external Command)

Shows a small Card of the direct environment of the Node.

(SO)FTWARE (external Command)

Outputs a Software description.

EXTERNAL Program for all Users

------------------------------

At the moment the Program stands here:

Page 77.

Digimail:

(MSG)

-----

(MSG) S <ENDCALL/GROUP> #<lifetime> <text>

For the creation of a Digimail:

The Message-Command enables, a User of the Node to write a short (!)

Message in the CTEXT. Then with the next Connect this Line will appear

in the CTEXT. Also, instead of the End Call an End Group can be input.

This is a type of Distribution List. Every Member of the Target Group

receives a copy of the Message.

The Lifetime can be input, but need not. It can be input from #1 = one

Day to #99 = ninety nine Days. If no Lifetime is input, the Default will

be set on the default = 14 Days. The lowering of the Lifetime occurs by

way of MSY C. See "External Program for the SYSOP".

Dependent on that, is whether the System works with a Hard Disk or a

RAMDISK, if the Message will be lost with a System Crash or Reset.

Example: MSG S DL9GYA #10 Roland, please Connect to me, when you are

back!

MSG S SYSOP Link after ..... defect?!

Finally the Message to DL1KWS and DL9GYA has been forwarded. The

Definition of a new Group occurs only from the Sysops. Please ask

accordingly.

(MSG) R

Reads out an individual Message.

(MSG) R <call>

Reads out the to <call> Message.

(MSG) R <call> 1 or 1-3

Additionally a Numbering can be input. Therefore reads the Messages 1

or  1-3.

(MSG) L

Lists the individual Header.

(MSG) E

The MSG E-Command erases all individual Digimail-Messages in the

individual CTEXT.

(MSG) G

Indicates all available Distribution Groups with Call.

(MSG) G <group>

Displays only the Call in this <group>.

New Groups can be created only by the Sysop!

Page 78.

(MSG) V

Outputs the Version Number and - Date.

Locator Calculation:

(QTH)

-----

(QTH) <locator> or (QTH) <locator> <locator>

QTH calculates Distance and Direction between two QTH-Locations. If

only one QTH-Location is input, the Distance to the Node will be

calculated ! Furthermore QTH ascertains all valid inputs for the

locations, e.g. for the investigation of the new Worldwide QTH-Locator

from the geographical Coordinates. QTH accepts both the new worldwide

as well as the old QTH-Locator. Furthermore Longitude and Latitude

can be in Decimal Form or also be input in Degrees:Minutes:Seconds.

(QTH) <locator>

Calculations of the distance and direction between the Location of the

Node and <QTH>.

(QTH) <locator1> <locator2>

Calculations of the Distance and Direction between <locator1> and

<locator>.

Valid Input Formats:

Old QTH-Locator :  FM04C or FM04C/2 (Without Input valid Field Frame

/2)

New QTH-Locator  :  JO52JW

Geogr. Coordinates:  DDDeg:MM:SS/DegDeg:MM:SS e.g. 10:47:30/52:56:15

(easterly Longitude/northerly Latitude in Deg:Min:Sec)

Geogr. Coordinates:  Deg.Deg.Deg/DegDeg.DegDegDeg

e.g. 10.792/52.937 (easterly.Longitude/northerly Latitude in Decimal

Form)

It is to be observed, that within a geographical Coordinate it will be

worked only in Deg:Minutes:Seconds or with Floating point numbers

(Real Numbers). A mix of both Formats is not allowed.

SATELLITES-Location-Calculation:

(SAT)

-----

Displays the current 40 possible Satellites as well as a short  Help.

Page 79.

(SAT) <no>

Calculates the Satellite Position at the time point of the request,

taken from the Location of the Node. Outputs will be not only the

Positional Data, but also the required Antenna-Installation.

(SAT) <no> <locator> or (SAT) <n> <coordinates>

However as before,  will not be the Locator of the Node, but the input

used.

Valid Input format:

Geogr. Coordinate:  DDD.DDD DD.DDD  e.g. 10.792 52.937

(easterly Longitude/ northerly Latitude in Decimal Form)

User Statistics:

----------------

(TOP)

-----

This call evaluates the MHEARD.TAB and creates a "Hit List". The

evaluation is not always the latest position. The File MHEARD.TAB will

be created dependent on the Parameter 11. The actual position can be

read out at the Time at the head of the Table.

(TOP) <number>

Outputs a List with a Length up to a Maximum of <number>.

(TOP) -<port>

Outputs a List of 10 Calls, which are known on this <port>.

(TOP) <number> -<port>

Outputs a List with a length up to a maximum <number>, which are known

on the <port>.

(TOP) <call>

Outputs only the Call Signs, but with the different registered SSIDs

(TOP)-v

Displays the Version Level.

Page 80.

MHEARD-Evaluation from 04.03.95 00:09 UTC to 31.03.95 22:53 UTC

No. Call   Port           Rx         Tx      Total  % RxRej TxRej Dama Via

1  DB0CL  cl3      41031482  261394098  302425580 52  3388  2151    0

2 DB0WHV  whv      92135818   31672170  123807988 21   489   132    0

3 DG3BAF  70cm      7867967    5666931   13534898  2   134     0    0  *

4 DG4BAQ  70cm      9751173      82280    9833453  1    14     1    0  .

5 DG9BIA  70cm      8083105      43419    8126524  1    80    57    1  .

.  Has received Call -----!          !          !  !     !     !    !  !

.  Has sent Call --- ----------------!          !  !     !     !    !  !

.  Has transferred Call via the Digi -----------!  !     !     !    !  !

.  Has Call part loaded to Digi -------------------!     !     !    !  !

.  Has sent Call, because heard badly -------------------!     !    !  !

.  Has received Call, because Digi heard badly ----------------!    !  !

.  Has Call transgressed against DAMA ------------------------------!  !

.      Call has   * = worked ------------------------------------------!

.      Call is    . = is QRV with different SSID on Ports -------------!

142 DB0BHV 70cm            2          1          3  0     0     0    0

143 DL4XL  70cm            1          0          1  0     0     0    0

144 DH2BAJ 70cm            1          0          1  0     0     0    0

145 DD4BN  cl3             0          1          1  0     0     0    0

0 DB3OU  70cm            0          0          0  0     0     0    0

0 DC5OH  70cm            0          0          0  0     0     0    0

0 DA0CL  cl3             0          0          0  0     0     0    0

0 DB0W|V whv             0          0          0  0     0     0    0

The listed Users have transferred 573.669.016 Bytes = 100 %.

In total 573.669.016 Bytes have been moved from 142 Users.

If anyone wonders about the Case of the difference between Places and

Users: it arises when a User is active on more Ports. e.g.: 70cm und

23cm. Or so the... fact is, it will have counted only the Call Sign.

Who has not transferred, does not acquire a Place.

Example for an individual Output:

Date           Port           Rx         Tx RxRej TxRej D Call      Via

31.03.95 22:53 70cm      7528028    1917008  120    0   0 DG3BAF

31.03.95 22:32 70cm       314818    3684301   13    0   0 DG3BAF-2

30.03.95 10:08 70cm            0          5    0    0   0 DG3BAF-15 DL1BKL

25.03.95 09:49 70cm        23946      65179    1    0   0 DG3BAF-1

03.03.95 10:07 70cm         1175        438    0    0   0 DG3BAF-4?

The List explains itself.

Under D, the DAMA-Offences, and under Via means that, it has stood as

Digipeater for selection.

Visiting Card:

--------------

Page 81.

(SAV)ecall

----------

(SAV)ecall <field recognition> <text>

The Fields can be filled or saved with (SAV)ecall.

SAVECALL /N .... . . . . . ..... 30 Characters for the Name

SAVECALL /Q .... . . . . . ..... 30 Characters of the Location

SAVECALL /L ......                6 Characters of the World-Locators

SAVECALL /D .......               7 Characters fo Input of the DOK

SAVECALL /V .........             9 Characters QRV on ... Digi

SAVECALL /M .................... 25 Characters Mybbs of the Mailbox

SAVECALL /T .... . . . . . ..... 40 Characters for free Text

( a * as Text erases this Entry )

SAVECALL /*     ==> Caution !! erases the entire Entry !

DOC Please enter so:   F73 or  F73/Z25

.....otherwise it fails with the hunt afterwards.

(SH)owcall

----------

(SH)owcall <call>

Is a small Program, which reads out and indicates the Visiting Cards of

the Users, which they themselves have created with SAVECALL.

Muster:

== ShowCall ====================================== 01.09.95 ===

=  Call   :  ......     Name : .............................. =

=  Loc    :  ......     QTH  : .............................. =

=  QRV on :  ......... MyBBS : ....................           =

=  Dok    :  .......  Update : YYMMDDHH CALL                  =

=  Free MSG    :  ........................................    =

================================================== de DG3AAH ==

The List exists from the interest of the Users. For this reason only

those may read and search who themselves have entered at least the

Field Name, Locator and QTH. Also the Connection <call> can be called

with the Place Holders "*" and "?". With that "*"  stands for an

optional number of Characters and "?" stands for only 1 Character.

There will be a List of Calls read out of existing : - Call, Name, Home

Place and Locator.

The first 3 Characters however must be input, and with that will be

read out a not unending Lists.

Online Help:

(H)elp

------

Replaces the old Help Function 'H'. Only in the File will be sought the

input Command. Displays all possible Commands.

Page 82.

(H)elp <command>

Displays the first Page (20 Lines) of Help for the <Command>.

(H)elp <command> 2

Displays the second Page of the Help for the <command>.

(H)elp <command> *

Displays all Pages of the Help for the <command>.

APPENDIX A:

-----------

The various Text Files and their meaning.

-----------------------------------------

In order to handle most simply the formation of the Info - or Help

Text, we have selected the Path, to place these Texts as individual

Files on Diskette, RAM-Disk or Hard-Disk. Each individual Text can be

edited by the SYSOP with the EDIT-Command and be read with the READ-

Command. Furthermore almost all Texts are accessible also other Users

via the Individual Command.

The Following Texts should be available:

AKTUELL.TXT (in the SUB-DIR: TEXTCMD)

Actual Information about the Node, performance, etc.

(Old AKTUELL.TXT may be removed.)

CTEXT.TXT (in the SUB-DIR: TEXTHLP)

Connect-Text (will be sent only, when released).

In the CTEXT.TXT the Advice about the available AKTUELL.TXT can be

available. If its Date is automatically input, so this is to be output

with a %A. Example for a CTEXT:

(H)elp (A)ctual  (from the %A)

CTEXT.n (in the SUB-DIR: TEXTHLP)

Port-dependent Ctext. CTEXT.n will be sent additionally to the

CTEXT.TXT on the respective Port.

HARDWARE.TXT (in the SUB-DIR: TEXTCMD)

Information about the Node Computer, etc. (HARDWARE-Command).

INFO.TXT (in the SUB-DIR: TEXTCMD)

Information about Link Stretches, etc. (INFO-Command).

MAP.TXT (in the SUB-DIR: TEXTCMD)

Small Card of the Neighbour Node (MAP-Command).

POLLDISC.TXT (ins SUB-DIR: TEXTHLP)

Report, which will be sent, when the User uses neither DAMA nor Pseudo-

DAMA. After Poll-UI-Count (PARAMETER ..) UI-MSGs will be sent. An

Page 83.

available '%n' in the Text will be exchanged in the Frack Time.

QUIT.TXT (in the SUB-DIR: TEXTHLP)

Will be output with the QUIT-Command if available.

SOFTWARE.TXT (in the SUB-DIR: TEXTCMD)

Information for the Software (SOFTWARE-Command.

SUSPEND.TXT (in the SUB-DIR: TEXTHLP)

See the Suspend Command!

An overview about all available Texts is received with the 'HELP INDEX-

Command. This Command lists all available Texts in the Queue, in which

it stands on the Diskette.

APPENDIX B:

-----------

(TNC2c)

-------

TNC2c and TheNetNode

--------------------

Clock Rate of the SIO-SyncB-Clock

If the TNC is operated with 2.4576 MHz, then without question the

Bridge BR1 is broken through and place SyncB (SIO Pin 29) at Pin 1 of

the 74HC4060. Also this alteration is described in the TNC2c-Guide. If

the TNC should be operated with 4.9152 MHz, then the following

alterations are necessary:

Change the Crystal (... clear)

Bridge BR1 scratched through and SyncB (SIO Pin 29) - that is the

middle connection from BR1 - place at Pin 2 (Pin 2!!) of the HC4060.

Pin 2 is not to be connect to BR1, but the alteration is necessary,

with that the SyncB-Clock runs at 600 Hz.

The EPROM must have an access time at highest 200ns!

If the TNC should be operated with 9600Bd on the HDLC-Side, a Clock

Frequency of at least 4.9152 MHz is necessary, 9.8304 MHz is better

(With 9.8304 MHz is absolutely necessary). For the conversion to 9.8304

MHz a few ICs are required, which do not lie in the 'Come in Handy

Box'. Standard ICs are adequate for a User-TNC, which will be operated

beyond their Specification. The Net Node however should run in a

lengthy operation, mostly to a non optional fast attainable Location.

For this reason no Risks should be entertained and the Components only

operated within their Specification. Which type of Program Crash or

also only Program inaccuracy appears with non specified Operation, is

not to be  foreseen. e.g. Runtime Info with a TheNet-Net-Node is false.

For the CPU and the SIO 10 MHz-ICs are to be used (e.g. Zilog Z84C0010

and Z84C4010).

Page 84.

The RAM requires an access time of 80ns or less.

The EPROM requires an access time of 100ns or less.

The ICs 74HC132 and 74HC139 are to be exchanged against 74AC132 and

74AC139!

From the 74AC139 (5), when the TNC is to be operated without a Battery,

a direct Connection can be installed to the 62256 (20). Through that

the Runtime is not applicable via both gates of the 74HC132.

The Bridge BR1 will be connected so that the SyncB-Connection of the

SIO (Pin 29) is connected with the 74HC4060 Pin 3.

KISS-Software for the TNC?

CONTENTS "KISS-Software for the TNC" \f C \l "2"

In the Tokenring and Watchdog :

On the Diskette is KISS17.BIN.

It contains the Eprom Contents for the TNC.

Content of Address Cells

Description (All Values in HEX):

5C

00..08..0F

Ring address for Programming 8 ... 16 Channels,

5D 00..FF

TX-Delay in 10 ms Steps HEX 14 = 200 ms,

5E 00..FF

PERSIST,

5F?00..FF

SLOTTIME,

60 00..FF

TXTAIL,

61 00..01 00 = Simplex; 01 = Full Duplex,

62 00..FF PTT Halt Time (Low-Byte),

63 00..FF PTT Halt Time (High-Byte),

64 00..01 00 = DAMA off; 01 = DAMA on.

Page 85.

The Addresses 5C and 64 must be adjusted with the burning of the Eprom.

The Addresses 62 and 63 are relevant with a FULL DUPLEX-Link. The other

Addresses will be installed from TNN and serve here only for the

completeness of the Information.

Address. 5C:

Every TNC in the Ring must have its own Address, which must stand in

the EPROM.

Address 5D..61:

These Memory Cells will be altered by NN from a New Start of the TNC

accordingly to the installed Parameters.

Addresses 62 + 63:

The PTT Halt Time is active only with the Installation of FULL DUPLEX.

If the TX is inactive, it will be keyed with the installed TX-DELAY.

After the end of the Keying the Timer runs out. If a further Packet is

to be sent in this time, it will be sent with TX-DELAY = 0 ms and the

timer reset. Through this a lot of Send Time will be saved, which now

stands available for the Data Transfer. If this Function should be

exploited, the 22 F Capacitor at PIN 11 (SIO RTS NOT) of the Modem-

Disconnect on the TNC2c PVB must be bridged. Naturally, the "Watch Dog-

Switching" for the Send Keying is then out of operation.

To be observed is, that the TNC2 must always be fitted with 32 KByte

RAM, and it should be installed only on TNCs used for 9600 Baud-Full

Duplex-Cases, which work with a CPU- Clock of > 6 MHz !

See a Construction Guide under APPENDIX B.

Example:

Address. 62 = Hex 50 ; Address. 63 = Hex 46

Overall:

Hex 4650 = 18000 ms = 3 Minutes Halt Time.

Address. 64:

If the DAMA-Bit is set, the TNC communicates to the Computer when the

Packet has been sent. With that the DAMA-Tout (22) Timer will be

started. It is to be programmed, for easing the burden of the Tokenring

and of the Computer only by the TNC, which should be installed as DAMA-

TNC.

Watchdog

--------

Version with MAX 691:

So long as the TNC works, it outputs a change between + 10 Volts and -

Page 86.

10 Volts to Pin 5 of the RS232. This change will be conducted to a MAX

691, which it monitors. If the change fails, first of all the Ring will

be closed again by a relay. Now the TNC will be restarted by the MAX

691 via Pin 8 of the RS232, which is connected with the Reset-

Transistor of the TNC. If another Impulse arrives from Pin 5 of the

RS232, the TNC will be entered again by the opening of the Relay

Contact in the Ring. First of all, in order to monitor the Max 232, I

have converted the Level on to a Level from 0 Volt and + 5 Volts by a

Resistance-Diodes-Zener Diodes-Resistance-Combination because

unfortunately the MAX 691 tolerates only +5 Volts.

Watchdog, Version with 4060:

The transfer of Data to the TNC and to the Computer will be monitored.

This occurs with two cheap IC 4060s. the IC is actually a divider. It

will be governed, to count continuously. If the Counter attains a

position such that Pin 3 (Q14) becomes active, a Relay will be

activated, that either resets the Computer or the TNC extracts the

Voltage. The divider counts further, and when Pin 13 (Q9) becomes

active, then the 4060 receives a Reset-Impulse, which will be fed to

Pin 12. Also at this Pin the Signal of the RS232 will be fed, which on

the arrival of Data, will reset the 4060 continuously. As a rule 20-40

Tokens per Second runs over the Ring. That may be reached.

Watchdog, Version with VANESSA:

With PIN X38 the Vanessa Card offers a RESET-Entrance. This Reset will

be activated when after 10 Minutes NO Frame has been sent from the Card

(Both Ports). If the Card is installed on an Interlink, every 3 Minutes

a Beacon will be sent from there and consequently the Timer reset.

The substantial advantage lies therein, that the Watchdog is

essentially more of a bargain (ca. 10.-DM) than the old Version, which

per TNC/Computer necessitated an expense of ca. 25.-DM. The old

Watchdog mostly should not have a reaction, because the Toggling of the

RTS-Pin was available, although the TNC has dependency.

A disadvantage that has to be evaluated is, one must drive to the

Node, when a TNC actually fails, because then the whole Node and not

only a Link is concerned. But also, Murphy always ensures that it is

always the one which should never fail. The entirety is likely a pipe

dream, but the Money that is saved, can be used much better in a Link.

If one still foresees a "Not-Reset" for the entire Net-Node e.g. per

DTMF-Control, also one can conduct an aimed Reset and so spare a few

Excursions to the Node.

And after the New Start...

After the New Start of the TNC, it appends its New Start Report to the

next Token. On the strength of this, it receives its intended

Parameters: TX-Delay etc. as Half- Full Duplex communicated from the

Computer. Simultaneously the incident will be entered in the Statistics

of the Program, in order to arrive at such mischief of the dodges.

Page 87.

APPENDIX C:

-----------

V A N E S S A

-------------

Installation Guidance:

----------------------

Contents:

---------

1.    General.

2.    Specification.

3.    Installation and Configuration.

4.    Problems and Error Finding.

1.    General:

SEPRAN is an Acronym and stands for:

SWISS EXPERIMENTAL PACKET RADIO AMATEUR NETWORK.

Various activities fall under this Project designation, which will be

conducted within the framework of the SWISS-ARTG (Swiss Amateur Radio

Teleprinter Group). In order to be able to realise a faster PR System,

the work comprises activities on various Levels such as Computer,

Software, Antennae or High Frequency-Construction-Groups .

1.1    Concept PR-Nodes.

There are many Ideas, as to what appears to be potential Hardware.

Especially on the Market, one can find for himself established X.25-

Products (Multi-PAD, Multiplexer). The characteristics of a good

intermediate System are both the high Data Rate as also the large Data

throughput. The critical point of the System will always be achieved,

when very many short Data Packets are to be handled. An apportionment

of the assignments in Routing on the one side and maintenance of the

Links on the other side, will be considered as necessary, in order to

attain the Targets.

Ideally a Multi-Processor-Concept appears, in which a Processor which

intercepts I/O-Last (FP VANESSA) and a Central Processor (CP) which

establishes the identity of higher functions for Multi-Channels.

These considerations lead to the following Concept:

Front-End-Processor VANESSA-2 with two independent AX.25 Channels, the

necessary Peripheral ICs and a Z80-280 CPU from ZILOG.

IBM PC (XT or AT compatible) as Routing Processor (CP) for more Front-

End-Processors.

DUAL-PORT-RAM as common Interface between the CP and FP.

Why a PC as Routing Processor

Page 88.

The usage of a PC as Routing Processor offers many advantages. If we

consider a Node of medium size, at least a Housing and a Current supply

must be provided for the Digital Part (5 VDC). For Testing, a temporary

application entry and a Screen is necessary.

Besides that, a comfortable Node Help Text should be saved for the

USER, Parameters, Configuration Table etc. Also with a Power failure,

this Data should not be lost, so that every time the SYSOP need not

load new entries of the respective Data. To that end, a PC with a

Floppy Diskdrive offers just that. With the Start, the Data will be

read from the Floppy into the RAM and then stands at the disposal of

the USER.

If we consider the Cost Side, so the expense for obtaining a (Used) PC

is very modest, for a medium Node a 286 type, a small Node needs an XT

or AT running at 10 MHz. A Hard Disk is not necessary, a Floppy of 720

KByte suffices perfectly.

2.    Specification:

The Front-End-Processor is constructed on an IBM-PC-Plug-in-Card and

supports two Data Channels with high speed.

CPU Type ZILOG Z80 280,

Address Space, 64 KByte out of 16 MByte, Run-Mode, System / User, Cache

256 Byte, MMU built-in, DMA, 4 Channels, Clock 9,8 MHz, 19,6 MHz

optional, Timer 3 separated Timers each with 16 Bits.

Memory RAM

32 KByte, 128 KByte optional, EPROM 32 KByte, 128 KByte optional, DP-RAM

8 KByte.

Interface

AX.25 Data ZILOG SCC 85C30, with DMA,?RS-232C

Diagnose Terminal, Option.

Displays 5 LEDs, Modem 2 parts. AM-7911 1200bps, External Modems, RS-

422 Driver, Connection Sub-D 37 Poll Plug.

Data Rate internal Modem 4 x 1200 bps Full Duplex, external Modem 4 x >

38400 bps Full Duplex.

2.1   Processor:

Processor: The ZILOG Processor Z80-280 will be installed. It is Binary

Compatible to the familiar Z80 Processor, however through its expanded

Command it allows an efficient Memory Management and saves DMA and UART

through the integrated Timer a few external ICs.

The most important characteristics are:

Separated Stack - and Memory area for SYSTEM, USER, DATA and PROGRAM

CODE, Virtual Memory area of 16 MBytes,

Page 89.

On-Chip-Peripherals: four DMA Channels, three 16 Bit Counters, one UART

as well as Cache Memory.

2.2   Memory:

The usable Memory area comprises 16 MBytes. The inbuilt MMU (Memory

Management Unit) enables up to 4 x 64 KByte memory to be Programmed in

Direct access. The minimum Configuration comprises 32 KByte EPROM and

RAM for Code respective Data. RAM and EPROM can be inbuilt up to 128

KBytes. For future Memory expansion a 32 pin Byte-Wide-Socket has been

foreseen. Additionally the usage of 8 KBytes Dual-Port-RAM [DP-RAM] is

foreseen. An Arbiter prevents Access Conflicts. With simultaneous

Access both Processors on the DP-RAM will block the one Bus with a Wait

State. The maximum delay is given through the Cyclic Time of the CPU,

which executes an access, and amounts to a maximum of 70 ns.

2.3   Memory

Decoder:

The Memory-Decoder separates out simply, because through the large

Memory Domain the upper Address management for Decoder purposes can be

extracted. The Programming of the MMU (Memory Management Unit) allows

an optional arrangement of the physical Memory Blocks within the CPU

Address Segments and consequently the complete filling of the Address

space in Direct Access with Memory.

2.4   Watchdog Timer:

A Watchdog Timer should raise the reliability against Hardware

Problems. With that, in no case should occur inadmissible out

maneuvering of the Software. The Watchdog will be retriggered within

the Timer-Interrupt-Routine, so that an inadmissible Program Cycle

releases a Restart of the Z80-280 CPU. In the case where VANESSA should

be installed in a Satellite on Inter-Galactic Flights, it is calming to

know, that an Error recognised by the Watchdog-Timer will lead to the

System Restart.

2.5   Status Displays:

For the Display of the System Condition five LED Indicators are

available.

2.6   Modem:

The AM-7911 is included as a Modem. This Single-Chip Modem allows a

Baud Rate of 1200 bps according to BEL-202 Norm. For larger Baud Rates

all necessary Signals will be conducted via RS-422 Driver to an

external Modem.

2.6   RS-422 Driver:

The Signals to and from the Modem will be conducted exclusively via

RS-422 Management Driver. On the one hand, through that will be a

practical optional conduction length, on the other hand through the few

drilled conductors, induction and earth loops will be prevented.

2.7   I/O Functions:

Two Output-Ports have been realised for the Control of Peripherals, .

Page 90.

2.8   RS-232 Serial Port:

For the implementation of a PR-Node, Hardware Tests as well as further

development of Hard - and Software is available at the Data plug of a

Serial Port RS-232 for a Diagnostic Terminal. Via this Terminal, an

internal Test Program can be called as well as Operational Parameters

monitored and altered. For Normal Operation no Terminal is required.

2.9   Comments about the Hardware Concept:

Through the Parallel Data handling on the Bus of the Computer, it has

given a mighty push in the Data Throughput. The working speed of the

Node now will be determined, through its Links (Baud Rate and the

transfer method (Semi - or Full-Duplex)). The Run Times within the Node

through the Parallel working of the Data have become so mean, that also

as a consequence with a 64 Kbit/s Full-Duplex-Link it does not fail.

Observations of the Node DB0NHM (which is accommodated already from

DB0EAM by way of a Vanessa Card shows distinctly, that the Answer Time,

which the RMNC-Node ascertains, is only approx. ca 50 % of its RMNC-

Neighbour DB0NID (both Radio Stretches 9600 Bit/s Semi-Duplex).

DUAL PORT RAM (DP-RAM):

With conventional solutions (TheNet, TheNetNode) the entire Data

Traffic goes from the Host-Processor to the TNC and returns via the

serial RS232-Lines. Now if more Channels are connected, it concentrates

entirely on one or two Ports. In between Bit Rates of 9600 bps per Link

are no longer seldom, so that this Port will be the original "Bottle

Neck" in the System. For this reason the serial Port Cards to the  PC -

Bus become heavily loaded. As an alternative the VANESSA occupies a

DUAL PORT RAM. With that is guaranteed, a faster and more simple Data

Transfer to and from the PC. The PC and the VANESSA CPU can access

quasi simultaneously on the memory and the transfers follow blockwise.

RS 422 Conducting Driver:

At a Node complex Earth relationships are present. Delays of creeping

streams lead inversely to the spoiling of the Data Transfer. Adjacent

to the Antenna Earths (Lightning Protection), Earth Protection (Net

Part) and Ground of the Digital Signals are still to be associated,

without that the Earth Loops act. Modem switching for fast Links must

be arranged possibly dense with the Modulator / Demodulator, because

otherwise Induction and Conductor Capacities of it make a noticeable

presence. That means, that on the one side the construction of the

Modem is possibly the most dense with the TRX (max. 30 cm Conductor

length), and on the other side the ready made construction will demand

to be in a Rack. For the decoupling to the VANESSA all Signals to or

from the Modem will be via the RS 422 Conduction Driver and a few

drilled connections. With that, Conductor Length  of more that 500 m

with Signal Cable possible, the System is decoupled.

2.10  VANESSA Firmware:

KISS MODE (Keep lt. Simple and Stupid):

For the Operation of the VANESSA as a TNC-Substitute with the Node, a

KISS-Mode-Firmware has been developed. With that the VANESSA takes

the Data Packetwise from DP RAM and forwards it onwards to the Modems

Page 91.

via the  SCC 85C30 (HDLC Controller). The Send Controls (Slottime,

DCD/Persistence Algorithms, TX Delay etc.) follow independently from

the Host Computer via the VANESSA. Received Data will be forwarded

after a Plausibility Test to the Host Computer via the DP RAM for

further working. With that the Host Computer is unloaded from the Time

critical assignments. Against the actual KISS Routines more various

Test Routines for Send alignment and Hardware Test (VANESSA

Peripherals, Memory, LEDs etc.) are available, with by choice can be

called from an external Terminal.

HOST MODE:

On the Contrary to the KISS-Mode, in the Host-Mode the Front-End-

Processor VANESSA undertakes the complete AX.25-Layer2-Protocol-

Control. Between Host and VANESSA will be transferred only the Info

Data as well as the Status of the Links (Connected, Info, Busy,

Disconnected etc.). The Host Computer occupies the Master Function,

i.e. the Front-End-Processor will be invited, to send Data or Status to

the CP.

With that, the usage of the VANESSA is possible as a High-Performance-

TNC for DieBox or Terminal Programs such as SP, GP or THP. By choice up

to 30 Channels per VANESSA will be supported.

The further Development for PR or employed Operation Types have (still)

set no Limits. The reserve Computer Times are not yet scooped out,

Memory Space likewise is sufficiently available. Thanks to the

universal RS 422 Port every optional Modem can be adapted internally or

an external Clock adapted.

3     Installation:

The Construction of the VANESSA Hardware has been advanced

intentionally  universal, in order to enable individual Configurations.

The individual Functions are evident from the Circuit, here the most

important will be explained in brief.

3.1   Basic Configuration Jumpers:

2 x 1200 bps, BEL 202, Port 0/1.   (Vanessa Card held so that the

SUB-D 37 is on the right.)

X1  left          X2  left          X23 left          X21 left

X18 up            X17 down          X16 down          X15 up

X14 below         X9  below         X10 below         X11 up

X12 up            X13 up            X8  down          X7  up

X6  up            X5  down          X4  down

X24, X25, X28, X29, X27 and X26 remain closed.

DIP Switch SW 1 all Switches set to ON.

IC 13 and IC 14 fitted (Modem AM-7911).

IC 33 to 36 empty (RS422 Port).

3.2   Adjustment of the Port-Address:

The Port Address is defined with the DIP Switch SW 1. With that the

Page 92.

Address of the DUAL PORT RAM fixed.

x don't care

x  ON  ON  ON           Port No. 0/1            Card 1

x  OFF ON  ON           Port No. 2/3            Card 2

x  ON  OFF ON           Port No. 4/5            Card 3

x  OFF OFF ON           Port No. 6/7            Card 4

x  ON  ON  OFF    *     Port No. 8/9            Card 5

x  OFF ON  OFF    *     Port No. 10/11          Card 6

x  ON  OFF OFF    *     Port No. 12/13          Card 7

x  OFF OFF OFF    *     Port No. 14/15          Card 8

Comment:

Vanessa Port 8-15: Because it gave Address conflicts for addressing of

the LED Address with certain HD-Controllers, the Addresses have been

removed. Unfortunately, through that, a Hardware alteration must be

made on the Vanessa Card. On IC29 Pin 16 (with Port 0-7 on +5 Volt)

must be set on GND. This can be done most simply with a connecting

socket and a Wire Bridge,

e.g. from Pin 18.

3.3  Employment of Modem by G3RUH:

With the usage of Modems by G3RUH remove IC13 (Port A) and IC14 (Port

B). The Jumpers X24 and X28 (Port A) respectively. X25 and X29 (Port B)

are open. Employing the internal Modem IC AM-7911 the corresponding

Line Drivers IC33 to IC36 must be installed.

Warning: CTSa (b) and RTXCa (b) (IC35/36 Pin 11 and 13) will not be

supported from the G3RUH Modem, and MAY NOT be placed in the Socket !

Configuration of Jumpers:

Port A      Port B            X24   X25   X28   X28

G3RUH       1200 bps          OFF   ON    OFF   ON

1200 bps    G3RUH             ON    OFF   ON    OFF

G3RUH       G3RUH             OFF   OFF   OFF   OFF

The Vanessa Card offers for selection at its Output TRXCa (b), the 32-

divider Clock of the installed Baud Rate. For the G3RUH-Original Modem

however, only the 16-divider clock will be required. For this reason,

this must still be  divided by 2, e.g. with a 74HC74, which has a good

edge quality.

3.4   Modem with own Clock Generator:

Contrary to the Original-G3RUH-Modem the DF9IC-Modem offers outlets for

the RX - Clock and the TX - Clock. i.e. the Modem generates the

corresponding Clocks internally.

For a better understanding, the VANESSA - Plans, Page 5 and Page 2

should be to Hand.

The description covers only Port A of the VANESSA, the Port B will be

wired similarly.

Page 93.

The RX - Clock will be placed on IC35 Pin 14/15 (RS422-Side) and

reached via Pin 13 then on the  - RTXCa.

For the TX - Clock a 26LS32 must be misused as an Entry, it offers for

Port A IC35 Pin 9/10. Pin 11 from IC35 may not be placed any more in

the Socket, but will be placed with a small wire bridge on the

Connection  - TRXCa to IC34. The corresponding Driver on IC34 should be

taken out of Operation by bending out the  IC - Leg (otherwise consumes

only current. If on both Vanessa - Ports DF9IC-Modems are to be

installed, IC34 can be saved completely.

Because the Connection - CTSa now lies open, this must be connected to

Ground Potential !

For the conclusion the Clock divider IC27 Port A / IC32 Port must still

be removed from the Vanessa. Because the VANESSA can be operated by an

external Clock - Mode NUR NRZ - Coding, the Modems with NRZ -Gal must

be in place. (Must be ordered extra!)

Now a Warning to finish:

Without question, if the RS422-Port is used without the Driver, already

it gives considerable Problems with 20 cm Cable Length at 38k4 Baud and

the SCC will be endangered by Potentially - unrequired Differences !

Parameter Installation:

As an external Clock will be attached, then an "e" must be as a

Parameter in the Mode. For a 38k4 Full-Duplex - Link the correct Entry

would be "mode=38400de". Then the Baud Rate declaration is indeed

unimportant, because certainly the SCC will be clocked on the VANESSA

from the Modem, but the correct Baud Rate is more informative.

3.5   Test of the Configuration:

For the Functional Test of the VANESSA a terminal is connected at the

37-Sub-D-Plug. The RS232-Connection is operated with 4800 Baud, 8 bit,

No Parity.

A description of the FIRMWARE-Test Steps is contained in the VANESSA

USER Manual. With the Start of the PC, on the RS 232 Port a Logon

Menu will be output, this extinguishes both lower LEDs.

Now the Interface of VANESSA to the PC can be tested with the Help

Program VANUTIL.EXE. Following a short explanation the Test Steps:

1. Show Configuration

Displays the installed Address of the DP-RAM and the FIRMWARE Version.

2. PC I/O Port Test

Test of the upper LEDs to the Plug.

Page 94.

3. Dual Port RAM Test

Static Test of the Dual-Port-Ram. With that a Test Pattern will be read

in and compared.

4. Vanessa Reset

Test of the RESET-Connection to the VANESSA, likewise the lower LEDs

will be activated.

5. Dynamic RAM Test

This is the most important Test ! With that, Data will be written in

the Dual-Port-Ram, through which Vanessa moves to another Address and

read out by the PC and compared. With that will be checked, the Timing

on the PC-Bus as well as the correct functioning of the Dual-Port-Ram.

If Errors intrude here, the danger exists that Data will be transferred

corruptly. Advice for hunting down Errors follows in the next chapter.

3.6   Dual Port Mode:

With the Help of the Program "VANCONF.EXE" the furbishment of easy

Multi-Baud-User-Entries are enabled. However, both of the associated

Ports  must lie on the same Card. Under Dual-Port-Mode mutually, the

PTT and DCD of both Ports must be moderately barred by Software. With

the help of the BAT-File, the  Dual-Port-Mode must be fixed already

with the Starting of the Computer. If the Port 0 + 1 is to be used, the

Program "VANCONF 0 D" should be called. 0 = Port 0 + 1 D = Dual-Port-

Mode. The Baud Rates do not require to be set, because they will be

installed by TNN.

4.    Problems, Error Search:

VANUTIL.EXE recognises no Vanessa-Error in the Dual-Port-RAM or

Addressing Logic to the PC-Bus. VANESSA-Firmware, CPU und RAM control

with DEBUG and Terminal to the RS232. Extinguishes both Upper LEDs with

the Start.

Control of the Address Modification by Port Number 8 and larger.

Likewise errors in the Dual-Port-RAM Area can be verified with

the DEBUG-Program. The Address of the Dual-Port-Ram can be calculated

as follows: Start Address: 0xD000:0000 + (n * 0x1000), i.e.0xD000:0

with first Port, D100:000 with the second Port etc. In this area

(0xD000:0 - 0xD000:fff can be input with DEBUG Data and re-read again

(n = Port No. 0 ...15).

Data Loss:

With a faster Computer mutilated Frames or entire missing Data have

been observed. With an XT-Board no Problems were to be observed, but

from ca. 16 MHz these disturbances appeared. The Dual-Port-Ram on the

Vanessa must be distinctly faster than 100 ns, with 70ns-Rams, 33 MHz

Computers no problems observed. With 386s or 486s the Main-Board-Setup

should be set on the following Values, because the AT-Bus-Clock with 8

MHz specified is:

Page 95.

- DRAM READ   1 WAIT STATE,

- DRAM WRITE  1 WS,

- EXTRA AT CYCLE WS 1,

- AT BUS CLOCK CLK / x (so that the Bus-Clock is ca. 8 MHz.

Experiments on the DB0EAM-Computer have produced, that the Vanessa

Cards display no kind of Error with VANUTIL8.EXE. when used with a 12

MHz Bus-Clock

Alternatively the components of the VANESSA can be altered. With the

usage of ALS-Types in place of the HC- or HCT-Types in the Area of the

Dual-Port-RAM no Error Functions could be detected with the faster Bus-

Clock .

In place of the HC/HCT-Types exchange against ALS-Types:

IC 20, 21, 22, 23 24, 25 and IC 28.

LEDs

Always arriving are questions about the LEDs. The green LEDs sit at the

Sub-D-37-Shell. They will be controlled from the Z280 Processor and

display the sending of a Frames (not to be confused with the PTT, with

which the Duplex-Operation certainly remains keyed for 3 Minutes after

the last Frame). For this reason, these LEDs will not be controlled by

the VANUTIL-TEST. If an external Clock is installed with the Vanessa,

but NO Modem is connected, these LEDs can glow, because then the SCC

awaits a Clock.

The yellow LEDs lying above, display the Reception of Data via the

Dual-Port-RAM, therefore via the Computer Bus. The Function of the LEDs

allows checking with the VANUTIL-Test. The individual red LED displays

the Function of the individual Card Processor. They glow, when the

Processor stands (when actually nothing  may be happening..).

Both left LEDs (yellow and green) are for the Port A condition of the

Card and consequently the right pair for the Port B.

The upper LEDs can be controlled with the Debug-Program via the PC-

Port. The Address calculation is as follows:

Left LED Address 0x306 + (n * 0x8), also 0x306, 0x30E ...

Right LED 0x307 + (n * 0x8), also 0x307, 0x30F etc.

From that it produces the following Address List:

Page 96.

Port  0 :  o 306 00 --> LED glows (upper left)   o 306 01 --> LED aus

Port  1 :  o 307 00 --> LED glows (upper right ...)

Port  2 :  o 30e 00

Port  3 :  o 30f 00

Port  4 :  o 316 00

Port  5 :  o 317 00

Port  6 :  o 31e 00

Port  7 :  o 31f 00

Port  8 :  o 126 00 ..If the Address is not altered:

o 326 00

Port  9 :  o 127 00

o 327 00

Port 10 :  o 12e 00

o 32e 00

Port 11 :  o 12f 00

o 32f.00

Port 12 :  o 136 00

o 336.00

Port 13 :  o 137 00

o 337 00

Port 14 :  o 13e 00

o 33e 00

Port 15 :  o 13f 00

o 33f 00

Both of the lower LEDs allow just addressing under the Addresses:

Port  0 +  1  o 300 00 --> LED Glows   o 300 01 LED off

Port  2 +  3  o 308 00

Port  4 +  5  o 310 00

Port  6 +  7  o 318 00

Port  8 +  9  o 120 00 ..Address not altered:

o 320 00

Port 10 + 11  o 128 00

o 328.00

Port 12 + 13  o 130 00

o 330 00

Port 14 + 15  o 138 00

o 338 00

Open Gate on the Vanessa Card: Open C-MOS Entries to Gates bow to the

oscillation. For this reason it should be better on a logical Level

Affected are the ICs:

(Bracket Values = IC-Entry)

IC 9 (13); IC20 (9-10) and (12-13); IC 25 (9-10) and (12-13;

IC 26 (10) and (12) and IC12 (4-5), (9-10) and (12-13).

SWISS ARTG

Swiss Amateur Radio Teleprinter Group

Geschaeftsstelle Arturo,

HB9MIR,

Blaueweg 8

CH-4335

Page 97.

Author: HB9PAE @ HN9AJ.

APPENDIX D:

-----------

(Ethernet) ...What must I do ?

------------------------------

Ethernet gives 2 different Cables.

For the first, the Operation is possible on our very well known RG58.

With this it MUST BE OBSERVED, that BOTH ends of the Cable must be

terminated with 50 Ohm Resistor.

The alternative to the RG58 is a Twisted-Pair. It consists of a double

conductor and requires no conductor termination.

How is progress made from here, when one is still unsure with Ethernet?

1.)

First of all, a free IQ must be found on the Computer, then the

Ethernet-Card can be allocated.

2.)

Then the Address domain of the Ethernet Card is fixed in the area

0x2e0 - 0x320. Here it can be overlapped with the Vanessa Cards IF CARE

IS NOT TAKEN, please observe.

The Vanessa Cards use the Addresses 0x306...0x33f (one for each of the

Number of the Vanessa Cards and employed Ports.) Furthermore the

default of Address Domain D000..Dxxx will be used. This also must be

remembered with the simultaneous Usage of Vanessa Cards and Ethernet

Cards.

3.)

After installing in the Computer the Ethernet Card can be checked for

availability with the Program TESTDRVR.EXE. For this purpose the

corresponding Card Driver must still be called. e.g.: NE2000.COM with

the input of the Soft/Hardware IRQ and Address Domain of the Card.

NE2000 0x60 0x300.

4.)

Now if, TESTDRVR reports no Problems on the Computer, so SEEPKT.EXE

can be started on the Computer (this is an Ethernet-Monitor) and on the

other Computer SENDPKT.EXE (this is a Beacon, which sends fixed Data).

The Data that SENDPKT.EXE generates, should be able to be read with

SEEPKT.EXE. Then exists in actual fact a Connection of the 2 Computers

via Ethernet no longer in the Paths, in so far that functions in both

Directions.

But first the Testing of the Port Cards:

TESTDRVR - A Utility for the Test of FTP-Packet Drivers:

--------------------------------------------------------

Page 98.

Forward:

Unfortunately not everything that the Handbook promises, is contained

with the Network Cards supplied Driver. This may lie in part for the

ambiguousness of the Specifications, but partially it lies with

security of false understanding protection for "Data Espionage" in the

Net. Certainly we can book also on the account "Carelessness".

Now the Driver can be tested with the User Software. Often there are

inconveniences and the economical Error Reports or nothing said -

which mainly arrives. Here TESTDRVR should care for any comfort.

If with the Driver delivered with the Card nothing works correctly, the

Driver from out of the FTP-Packet can be used with the Standard Cards.

For NE2000 and WD (SMC) compatible Cards, Drivers are contained in this

Package.

FTP-Packet Drivers are the unique Drivers, which permit simultaneous

various Network-Protocols on a Card. In addition, outwardly the Port is

documented extensively and the entire Software is freely available in

the Source Text .

In this Guide I will not write much about Network basics. That should

occur in a separate Documentation.

Prerequisites:

--------------

Hardware

--------

Today, Network Cards are "NE2000" compatible for less than DM 50.-.

customary is/are "Cheapernet" = BNC Connection (10Base2) and/or UTP

(RJ45) Connection (Twisted-Pair). The BNC connection has the advantage,

that without further Hardware they can be coupled to several Computers.

If it is required to "solo" test a Computer, please observe, that the

Network Connection to both sides must be terminated with 50 Ohms.

Otherwise  most cards disallow the Sending. Therefore install a T-Piece

with two 50 Ohm Terminations.

Modern Cards are longer configured with Jumpers, but by Software. This

can lead to Problems, if the chosen Defaults from the Card manufacturer

Defaults are allocated already in the Computer. Then usually one must

convert other Cards first, in order to be able reconfigure the Network

Card. A NE2000 compatible Card requires a free Interrupt and a free

IO-Address. WARNING: If Vanessa Cards are to be installed, so the IO-

Addresses for each Port between 0300-0338 are occupied already !!

Software

--------

If you have already got a Network Connection to the Computer, please

bear in mind, that a Network Card can not serve two Masters at the same

time. Therefore the available Net Software (Novell, WfW etc.) Must lie

dead, in order to be able to experiment with FTP-Packet Drivers. Then

subsequently it can be attempted, to install the "old" Software on the

base of the Packet-Driver. It only works in this direction, nothing

else coming back. A Packet-Driver requires no more than a Call for the

Page 99.

Installation (Example "EP3000PD 0x60"). Here PROTMAN.SYS and similar

Programs definitively are false.

The Input of a Software-Interrupt for the Driver must be input

with the Installation. This Interrupt can lie by Definition in the

Range 0x60 to 0x80. There is nothing against using 0x60.

Test Execution

--------------

At first the Signature of the Driver will be sought and so the employed

Software-Interrupt found. Naturally, the Software-Interrupt can be

input in the Call. But because nearly all Programs search for the

Driver by its Signature, it makes sense already to precede.

Then we must inform the Driver, what we demand from it - the Network

type, which type of Packets etc. Furthermore we must say to it, where

it should deliver the Packets, which are entered "for us". As a result,

the Driver delivers to us a "Handle", that in the future designates

with the communication between the Driver and us the Letterbox. Without

a valid Handle will not be worked.

Now we ask the Driver, which Command Set it understands. It gives three

expansion steps: Basic Service, Expanded Service and High Performance

Functions. We require the expanded Service. The first Versions of

TFNETR and TNN also delivered with the Basic Service. In order to be

compatible with PE1CHL and G8BPQ, but once again we have altered the

Protocol.

After that, we request the Hardware Address of the Card. This Address

is unequivocal, every Card has another Address. We should never alter

it without compelling grounds. With the request we experience the

beauty of the Segmentation of the 80X86 Processors. If the Programmer

of the Card Drive was not careful, the Address arrives as

00:00:00:00:00:00. Then we are fairly sure, that the same muck up is

also in the Receive Routine. Then with that, Crashes are pre-

Programmed.

The Communication between two Network Cards can follow on more Methods:

-In Packet will be given, the Hardware Addresses of the partaking Cards.

That is complicated. Then I must know the Address of my Partner before

the first Contact input.

-The Packets will be sent "to All" (Broadcast). For these purposes the

FF:FF:FF:FF:FF:FF is defined. This method disturbs very much in large

Nets, because then we over feed the entire Net and every one of our

Packets is heard and must be analysed.

-The Packets will be sent to a Group (Multicast). Only the Group hears

these Packets. This is the method we use. Now we control, whether our

Driver understands this method on the receive side.

Page 100.

The next Test follows, in order to see, whether the Program NETWATCH

will run. NETWATCH is a universal Network Monitor. Unfortunately many

Programmers have nailed onto this necessary "promiscuous mode" for this

reason. But it certainly gives the FTP-Driver.

Now we set the Driver on the Send side in the Multicast-Mode. For it,

we define the "BPQ"-Address.

As a last Test we send a Packet (with optional contents) to our

Group. If the terminating resistance is not in place, then here certain

Cards hang. Others give an Error Report.

For the finish, we release the Handle again. If the test has passed up

to here, the hope exists, that also the Driver will run with the User

Software.

To end; a last piece of advice: a Computer Crash with receiving or

sending of a Packet in connection with a WD (SMC) Card is mostly due to

an error in the Configuration of the Memory Manager to respond. This

Card uses namely parts of the Main Memory for communication with the

Driver.

Error Reports:

Certain Error Messages should not appear. They are marked with a ???.

Nevertheless if they appear, so it points to a capital problem within

the Driver.

No Error

The Driver has reported an Error and is explained by request, it should

not have occurred.

Invalid Handle

The Driver does not remember its own Handle.

Invalid Class

The Driver does not support the Class "ETHERNET".

Invalid Type

We have demanded "any old" Type.

Invalid Number

We have demanded "the first Card".

Invalid Packet Type

We have demanded "all available" Types.

Multicast illegal

"Our" Mode will not support, Driver unavailable.

TERMINATE not permitted

We do not demand, that the Driver is removed.

Not permitted RX-Mode

"Our" Mode will not be supported, Driver unavailable.

Page 101.

Insufficient Memory

Card is attempting to demand too much, after a RESET of the Computer

once again. Is other Network Software still active.

Type is occupied

Other Software has left the Card undefined.

Illegal Command

A Standard Command will not be recognised, Driver unavailable.

(Hardware) Error with the Send

Termination Resistance not in position or false Connection.

Cannot set Hardware Address

We fumble because nothing there.

False Hardware Address Format

The Driver is defective.

RESET not possible

That has been a Command misunderstanding.

Now on the TNN-Side, the following is required:

In the Start.Bat an Ethernet-Card-Driver must be invoked. Simply call

the Driver once, then most communicate, what Parameters it has. For

this on the Diskette is a DIR TESTDRVR with a few Help Programs, to

assist with this situation.

tnl1.exe -v

nwpd 0x68 10 0x340

ipxpd.exe /i=0x68

TNNGNU.exe

tnl1.exe -u

nwpd -u

APPENDIX E:

-----------

THE PACSAT-BROADCAST-SERVER from TNN

------------------------------------

New from TNN V1.70:

-------------------

-Parameters complete in the BROADCAST-Server,

-DIRECTORY-BROADCAST for WiSp,

-New Message Management,

-Max. Number of saved Mail installable.

Execution:

----------

TNN performs simple DieBox-S&F with a Mailbox and saves the Messages on

the HD and then makes its Broadcast with the Data. The Command "BOX"

switches TNN into the S&F-Mode. The messages will be saved in ascending

Number order as they arrive via the S&F. With the Start of TNN the

1st. and the last number will be ascertained.

Page 102.

The Broadcast:

--------------

The Protocol is documented in PACSAT.TXT, it is 100% PACSAT compatible.

The PACSAT-Server uses optional and extended header and transfers in

the RLM Message Format. Also with that e.g. WISP (to obtain AMSAT)

should arrive clearly and the User gets the Messages sorted

automatically in Rubriks. One integrated Header Generator from PE1HCL

is now called PFHADD.EXE (V3.3 from 2.6.95), it must stand in the

PACSAT-Directory. With that WiSp returns with; TNN-PACSAT-BROADCAST,

DIR-Entry will be sent out before the actual File (from TNN V1.70).

Operation:

- Message Entry by DieBox-1.8-Compatible S&F (No BIN-Mail !).

- User & Sysop-Port (Command PACSERV, PAC*), stands there for the

selection: For USER accessibility:

H- Help,

I- Information about the Server,

Q- Quit,

P- Parameter Request.

For SYSOPs:

+ Port Dest Call via DestRoute

Broadcast for a Port activation.

Example: + 1 QST-1 via DB0LWL

DB0LWL

DestCall should always be QST-1. Broadcasts can be transmitted VIA L2.

But this should be done with caution, because as a rule, a 9600-Baud-

Broadcast via a Neighbour Node leads to a Crash, because this cannot

transport the huge Data pile fast enough. This is valid especially for

RMNC/FlexNet/BayCom-Neighbour.

Port

Broadcast deactivation.

R

Reload, the File System after an external alteration newly read in.

M

PacSat-Boxcall sets, M call, should be the Call Sign of the Node or a

Fantasy-Call, but it must agree with the Call Sign with which the Box

is incorporated. This Call Sign will never be sent HF-Moderately.

Actually it will be required, only because with the DieBox all Partners

must have different Calls. DB0HRO-5 (Box) makes S&F, with DB0HRO-7

(Cluster) S&F, consequently no longer can DB0HRO (Node) be added as

S&F-Partner, also the Node can be entered there as BR0CST (what is also

always) and places M on M BR0CST. Then if it should crash. P for the

installation of various Parameters

Page 103.

BROADCAST-Parameter :

01: PacSatTimer : 200

02: PacSatFrms  :  15

03: PacMaxMail  : 1000

P 01 installs at what Intervals the Node sends the Pacsat-Frames, this

Parameter must be so installed, that the TNC will not overrun.

P 02 installs the number of Packets which will be sent at the same

time. With 9k6 that should be 20 or so, (pi * aprox. 5 Frames per

Second), otherwise there is no reward.

P 03 installs the maximum number of Mails in the PACSAT-Server. If with

S&F more Mails in the Server than are input than by

P 03, so the oldest Mail will be erased.

Via the Timer Control an optional Channel loading can be created, which

enables everything from individual Send on a User-Entry to the duration

of Keying, with that two Tactics will be supported:

- Only Pacsat-Outlets:

Duration of Keying, for ISM-Operation. Then TNN makes a rumpus with a

continuous Carrier 24-Hours a Day, 365 Days in the Year. For this

purpose, a DAMA-Eprom is used, on the Port where PacSat will be

operated, but DAMA for the PORT SWITCHED OFF. The DAMA-Acknowledgement

will be used in order to guarantee the continuous keying.

P 02 determines, how many Frames will be sent to the TNC, which must

always have an adequate supply. With that it does not release the PTT,

but that may not be too many, otherwise it overruns. After each Ring-

Loading this Value must be, so that the TNC also without, that it holds

Data. With 19k2 Tokenring I can have the Parameter P 02 on 14, with

38k4 can be less, with very heavy loading Tokenring more.

P 01 inputs, as long how long as will be waited, when NO DAMA-

Acknowledgement arrives, this Parameter should stand on 1000, but is

actually irrelevant (as long as the Acknowledgement arrives).

- Mixed In/Outlets:

Here, either DAMA must be switched on for the Port and a DAMA-Eprom

employed or NO Dama-Eprom used and no DAMA switched on, DAMA-Eprom and

no DAMA does NOT work. (that makes the Node Continuous Keying)

Continuous-Keying-Outlets are absolutely preferable, they convert 82MB

of Data a Day, that is 10-times a normal 9k6-Entry. Furthermore every

User which was on that, not only Continuous Sucker XYZ.  A Broadcast-

Outlet with is not sensible and should not be operated. But it must be

cared for, in as much as the Sender is fit also for continuous keying.

The Tnet-Mini in the Original is not suitable!

Execution of the Broadcasts:

----------------------------

Page 104.

TNN has a Priority switching, which repeats the last 200 incoming

Messages in the 10-15 Minute Clock. With that, Users are required to be

switched on for only a short period of Time and immediately have the

most important Messages. For the Rest of the Time the other Messages

will be sent out.

Broadcast Format:

-----------------

Is 100% compatible to what our artificial Earth satellites make,

inc.DIRECTORY-BROADCAST.

Installation:

-------------

The PACSATPATH must be attached in TNN174.PAS, it should stand in

C:\TNN\PACSAT\. All Messages will be saved there. The File "PFHADD.COM"

must be copied in this Directory also, it will be required internally.

Then the Sysop of the next Mailbox must be appealed to, that with it

enters the S&F. When it has connected to the Node, the Box must

present"BOX", in order to arrive in the S&F-Mode. As soon as the 1st.

Message arrives, the Broadcast is released, it can be broadcast also on

more Ports (also it works on one with 9k6, on the  other with 19k2,

i.e. the sequences are independent, so that one Port does not wait for

the other one). The Node requires (naturally) a Hard Disk, It should

have 80MB or so already, more is also not bad.

MISCELLANEOUS:

--------------

The User:

---------

WISP is a Program, that automatically decodes the Mail and sorts into

Rubriks.

Maintenance effort for the Sysop:

---------------------------------

Little. When the Broadcast is first installed, it runs by itself and

requires no maintenance. If a Read Error intrudes on the HD, the

Sysop can have a free journey, also that is with Boxes, unfortunately

it is not able to be altered.

Important:

----------

TNN must read approximately 1,5kb per Second from the HD, for this

reason it is recommended that SMARTDRV is installed with READ-ONLY

CACHE, it respects the HD. The PRE-READ-MEMORY should be placed from

the Standard Value of 8kb on 32kb (experiment with that).

Vanessa is preferred for Tokenring, the Tokenring with 9k6 Broadcast

must have 19k2, 38k4 is better. On full Digis (many Links, e.g. FC) it

can happen, that continuous keying does not work, that cannot be

eliminated. With Vanessa it should not occur.

Doubled Messages:

-----------------

Will not be checked. Users, who can choose via the Box-Software

Page 105.

(WinGT), the S&F, also can operate in the Server, then that also

outputs on the outlet. The Sysop should check that a little, so that

with that the people cannot make mischief.

Request:

--------

(Also demand of error Parts through UI-Frames) is not sensible for our

usage, also it will perform first time.

Special:

--------

In certain districts a PR-BROADCAST-Outlet in the ISM-Area is not

possible by reason of existing ATV-Allocation. On these grounds will be

e.g. in Colgne BROADCAST made on the Input (!) of an FM-Repeater. For

this two Frames will be sent, in order to check, if the Repeater has

been opened. If the Repeater is still closed, 2 Frames will be sent

again. But if the Repeater has been opened, the PTT will be barred by a

small switch. Naturally the throughput is less than with a CONTINUOUS-

TX-Outlet, but I think that it is a good alternative. For this purpose

I have developed a small switch. With requests please  apply to:

(DG1KWA / Andreas @ B0GSO).

APPENDIX F:

-----------

Construction of the Layer 3/4 Frames

------------------------------------

With TheNet, TheNetNode and TheBoxware.

---------------------------------------

Because for the 3 Programs, TheNet, TheNetNode and TheBoxware the same

Layer 3/4-Protocol is valid, only TheNet will be discussed in the

following, but all 3 Programs are intended (when not expressly

expressed otherwise). Here, only the Frames with expanded AX.25-Format

will be handled, Identification-Frames etc. remain invisible.

All special Layer 3/4 Frames of TheNet will be sent out with the PID

"CF", whereby a distinction of normal Layer 2 Frames is ensured (with

is the PID "F0"). This is valid both for I-Frames as also for  UI-

Frames.

Basically, the extended Protocol in the Info field of the AX.25-I-

Frames consists of 3 Blocks: the Network-Control-Block (Layer 3)

occupies 15 Bytes, which upon them the following Transport-Control-

Block (Layer 4) exists a further 5 Bytes, and eventually remains the

Information Part, for which a Length of from 0 to 236 Bytes is

defaulted.

Network Layer Frames.

---------------------

The Router of TheNet defines, on which Path, Messages from an End Node

will be transferred to others. The Routing follows automatically on the

assumed most favourable Path. That employed Path can be altered during

Page 106.

a Connection, if e.g. a NetNode fails; then automatically an

alternative Path will be employed (if available) - the End User notices

nothing of the diversion.

For the Routing by TheNet, first of all, Information will be exchanged

via the attainable End Nodes with a Broadcast-Transmission. With

TheNetNode the Runtime will be measured to the Neighbour-Node.

The higher Transport-Layer is constructed on the Network-Layer, so that

all of TheNet-I-Frames contain a Network-Control Block, which is always

constructed the same. It exists from a total of 15 Bytes altogether.

The first is the Call Sign of the Sending Node (7 Bytes), followed by the

Call Sign of the Receiving Node (7 Bytes). Eventually the Lifetime of

this Frame will be transferred (1 Byte). The Lifetime will be decreased

by 1 with each Repetition to a Neighbour-Node until either the target

Node is attained, or the Value 0. In the case that 0 is reached, the

Frame is to be destroyed, to prevent a Frame from circling through a

resulting accidental endlessly long Loop of the Net and by that blocks

the Net.

Runtime Measurement by TheNetNode.

----------------------------------

In order to measure the Runtime for Layer 3 Frames to a Neighbour Node

and back, Measurement-Frames will be sent periodically from TheNetNode

to all connected Neighbour Nodes. With that it deals with I-Frames with

the PID "CF". In the Network-Control Block is incorporated as Sender,

the Call Sign of the Node, which undertakes the Runtime-Measurement. As

End-Node the Phantom-Call-Sign L3RTT is used (as "Layer 3 Round Trip

Time"). The Transport-Control Block is occupied by 5 Bytes "00". In

the Information Part more Data is contained such as e.g. the System

Time with the creation of the Frame. Because the Data will be evaluated

only by the Sending Node, the precise Data Format is not important.

Furthermore this part of the Protocol is still in the Development

Stage, so that alterations are not yet finalised.

Automatic Routing (old method).

-------------------------------

This method of the Routing is retained ready for Nodes, which still do

not support the Protocol of ON5ZS.

For the automatic Routing, as a rule the Information will be sent out

from the Net-Node via the Content of the Target-Node-List. This follows

through UI-Frames with the TheNet-PID "CF". The Sender Call-Sign of

this Routing Frame (Broadcast) is the Call-Sign of the sending Net-

Node, and as Target-Call-Sign Phantom-Call Sign "NODES" will be used.

For TheNetNode the End-Call-Sign is no longer defaulted from the

Program, in order to be able to route via special Layer 2 Digipeaters

(e.g. Flexnet).

In the Info-Field of the Broadcast-Frames, as recognition a Byte"FF"

will be sent first, followed by the Ident of the Sender-Node (6 Bytes).

Eventually up to 11 Entries of the Target-Node-List will follow.

Page 107.

Every End-Node-Entry occupies 4 Data Blocks. The first Data Block

contains the Call-sign of the End-Node (7 Bytes). The second Data Block

contains the Ident of the End-Node (6 Bytes). In the third Data Block

follows the Neighbour-Node, via which the End-Node is known to have the

best Runtime (7 Bytes). Eventually in the fourth Data Block the

Runtime of the best Path to the End-Node will be indicated (1 Byte).

All indicated Idents will be transferred as normal ASCII-Characters,

during which the AX.25-Format (ASCII 1 Bit moved to the left) will be

used for all.

With newer TheNet-Versions the Sending out of the End-Node-List is

expanded to the Layer 3 Lifetime of the individual End-Node. With that,

this expansion remains compatible with the older Versions, the

additional part has been attached to the earlier Sending. As a

recognition of the beginning of the Lifetime Block, a Byte "00" will be

sent. This recognition prevents simultaneously, a further evaluation of

the Frame by older Programs, because otherwise the Lifetimes of a

further End Node must follow, beginning with a Call-Sign. But Call-

Signs cannot begin with "00".

Following that, is the individual Layer 3 Lifetime of the Sender-Node.

Affiliated will be the Lifetimes which will be sent out in the Frame

contained End-Node in the Queue, in which previously the End-Node has

been sent out.

For the Lifetime in each case, 1 Byte will be sent and indeed those

Lifetimes, which were incorporated also in a Frame of the corresponding

End-Node. If the Lifetime is unknown, "00" is sent.

New Routing in TheNetNode of ON5ZS:

-----------------------------------

Router alterations:

-------------------

Now the BROADCAST will be transferred in the secure Level 3. So as a

Node disappears, its Quality will be set on "1". The Neighbour will be

informed through the BROADCAST. This immediately informs its other

Neighbour and so further. With that the entire Net will be appraised of

this fact. Length of the expansion: between 0 and 4 Seconds per Hop.

A new System:

1 Stretch and 2 Paths. The System is now so far clear.

/---------------------------\

user -- on4ulg                             on5vl --- on5vl-5(bbs)

\---on1apd-----on4aic-------/

|          |

|          |

on1xx -----on1yy

The solution was fairly simple...

Page 108.

With that on5vl takes the Path on5vl, on4aic, on1apd, wi th on4aic the

best Path to on4ulg via on1apd are incorporated. Through the

Calculation Method of the Quality, the Quality of on4aic with on5vl is

similar to the Quality of on5vl with on4aic, on5vl can calculate simply

the Quality of the Stretches from on4aic (and determine, whether on4ulg

will be addressed directly, or not).

a)

When is possible, (no ping-pong -> calculation of the Qualities of the

Neighbour).

b)

The alternative Path will be chosen, when the transfer time of a Frame

on the alternative Stretch is shorter.

c)

When a Frame from the Level 4 arrives (on4aic -> on1yy will never be

required).

The alternative Stretch will lay claim from the instant, when a

definite number (*) of Frames is available in the tx_buffer level2 ( ->

to send ) with on5vl -> on4ulg.

(*)

This number adjusts itself to the Quality difference between Main-

Stretch and alternative Stretches in the Method, as the Frames by

chance arrive in the correct sequence with on4ulg (TNN forwards in the

correct Sequence).

There is no "Selective retransmission request" from on4ulg.

Transport Layer Frames.

A special transfer protocol will be used between the End Nodes of a

Circuit, in order to enable a  transparent, error free Data transfer.

This additional Protocol will be transferred in the Transport-Control-

Block and in the Information Part of the Frame.

The Transport-Control-Block consists of 5 Bytes. The meaning of the

first 4 Bytes is dependent on the 5th. Byte, the Control Byte. The

Control-Byte itself exists consists of the Opcode as well as 3 Flags.

The Bits 0-2 in the Control Byte build the Opcode, derived out of

the Frame Type. At the moment only 6 different possibilities will be

used from the 8 possible Opcodes :

1 = Connect-Request,

2 = Connect-Acknowledge,

3 = Disconnect-Request,

4 = Disconnect-Acknowledge,

5 = Information-Transfer,

6 = Information-Acknowledge.

At this moment, the Bits 3 and 4 will (still) not be used, they will be

set, always on 0.

Page 109.

Bit 5 is the More-Follows-Flag. This Flag displays, that the

Information Part has be divided into multiple Frames. The Frames will

be divided, in order to transfer transparently, Frames of the End Users

containing more than 236 Bytes of the Information Part. In the first

Part will be transferred the maximum possible 236 Bytes of Data, the

rest follow in the next Frame. With older Versions of TheNet,

Information about the Node itself (User List, Node List) will be

transferred partially within the Net also with a substantially larger

Number of Data Bytes. Then instead, correspondingly more Frames with

set More-Follows-Flag will be sent one behind the other. Then with the

End-Node, the sub-Division in AX.25 equitable 256-Byte-Frames will be

undertaken.

Bit 6 is called NAK-Flag. This Bit will be used and displayed only with

the Frame-Types: Information-Transfer and Information-Acknowledge, for

a Frame to be repeated.

Bit 7 is the Choke-Flag. This Flag will be used with the Frame types:

Information-Transfer, Information-Acknowledge and Connect-Acknowledge.

The various meanings of these Flags is explained with the corresponding

Frame Types.

Connect-Request.

----------------

A new Circuit will be demanded with a Connect-Request-Frame. The first

two Bytes of the Transport-Control-Block specifies the Circuit of the

Sending-Node. With that the first Byte indicates an Index in the

Circuit-Table (Circuit-Index). The 2nd. Byte will be used as additional

recognition for the respective Circuit, in order to prevent

ambiguousness (Circuit-ID). The following 2 Bytes will not be used and

set on "00". The Flags in the Control Byte will not be used and also

set on 0.

First in the Info-Part of the Connect-Request-Frames will be

transferred, the recommended Window Size for the Transport-Layer

(1 Byte). The Call-Sign of the User follows, with which the new Circuit

wishes to construct. Eventually follows the Call-Sign of the  Sender-

Node. Altogether there are the 15 Bytes in the Info-Part of the

Connect-Request-Frame.

Additionally with newer Versions of TheNet will be transferred, which

Net Node the User has entered into the Net, and via which Digipeater

Path it is to be attained. At first the Call-Sign of the Net-Node at

which the User has entered in the Net, will be transferred. After that

follows the possible available Digipeater List with the Uplink, also a

maximum of 8 further Call-Signs. For the completion follows 1 Byte

"00". For this expansion also will be transferred, a minimum of 8 and a

maximum of 64 Bytes additionally in the Information-Part.

For all with the Connect-Request transferred Call-Signs, the 1 Bit

shifted to the left AX.25-Call-Sign-Format will be used.

Connect-Acknowledge.

--------------------

Page 110.

A Connect-Request-Frame will be answered with a Connect-Acknowledge-

Frame. The first two Bytes of the Transport-Control Block indicate the

Circuit of the Node which the Connect-Request has sent. They will be

undertaken directly from the Connect-Request. The following two Bytes

designate the Circuit of the Node, which sends the Connect-Acknowledge-

Frame, again in the sequence Circuit-Index - Circuit-ID, as with the

Connect-Request-Frame.

When the Connect-Request has been accepted, all Flags are erased in the

Control Byte. In the case where no Circuit can be constructed, the

Choke-Flag (BIT 7) is set.

A new Circuit will be demanded with a Connect-Request-Frame. The first

two Bytes of the Transport-Control Blocks specify the Circuit of the

Sender-Node. With that, the first Byte indicates an Index in the

Circuit-Table (Circuit-Index). The 2nd. Byte will be used as additional

recognition for the respective Circuit, in order to prevent

ambiguousness (Circuit-ID). The following 2 Bytes will not be used and

set on "00". Also the Flags in the Control Byte will not be used and

also set on 0. In the Info-Part of the Connect-Request-Frames, the

recommended Window Size for the Transport-Layer will be transferred

first (1 Byte). After that the Call-Sign of the Users follows, which

the new Circuit wishes to construct. Eventually the Call-Sign of the

Sending-Node follows. Altogether there are also 15 Bytes in the Info-

Part of the Connect-Request-Frames.

In the Info-Part of the Connect-Acknowledge-Frame 1 Byte will be

transferred to be used in the Layer 4 for the Window Size.

Disconnect-Request.

-------------------

When an Information-Transfer-Frame is received and no Information for

the Opposite-Station exists, the Receipt will be acknowledged with an

Information-Acknowledge-Frame. The first two Bytes of the Transport-

Control-Block again designate the Circuit of the Node to which the

Information-Acknowledge-Frame will be sent, in the sequential Circuit-

Index - Circuit-ID. The third Byte of the Information-Acknowledge-Frame

is not used and will be set on "00". The 4th. Byte of the Transport-

Control-Block is the Receive-Sequence-Number and displays that as the

next awaited Send-Sequence-Number of the Opposite Station.

A Disconnect-Request-Frame contains no Info-Part.

Disconnect-Acknowledge.

-----------------------

As an Answer to a Disconnect-Request-Frame, a Disconnect-Acknowledge-

Frame will be sent. The first two Bytes designate the Circuit of the

Node, to which the Disconnect-Acknowledge-Frame will be sent, in the

sequence Circuit-Index - Circuit-ID. The following two Bytes will not

be used and set on "00". The Flags in the Control-Byte will not be used

and therefore erased.

A Disconnect-Acknowledge-Frame contains no Info-Part.

Page 111.

Information-Transfer.

---------------------

An Information-Transfer-Frame transfers the Information of the User.

The first two Bytes designate the Circuit of the Node, to which the

Information-Transfer-Frame will be sent, in the sequence Circuit-Index

- Circuit-ID. After that a Byte follows for the Send-Sequence-Number of

this Frame. The 4th. Byte of the Transport-Control-Blocks is the

Receive-Sequence-Number and displays that as the next awaited Sequence-

Number of the Opposite Station. Simultaneously with the Information

Transfer, the receipt of the Frame from the Opposite Station will be

acknowledged .

If the More-Follows-Flag is set, this means, that the complete

Information of a User-Frames could not be transferred all at once,

because in the Information-Transfer-Frame the most that can be

contained is 236 Bytes of Info (the remaining 20 Bytes will be required

for Network - and Transport-Control-Block). In the next Frame is the

continuation of the User-Information.

The NAK-Flag will be set, when a received Information-Transfer-Frame

did not have the expected Send-Sequence-Number. With that the Repeat

of the missing Frame will be demanded.

If the Choke-Flag is set, this means that first of all no further

Information for this Circuit can be accepted.

In the Info-Part of the Information-Transfer-Frame, the User-

Information will be transferred (max. 236 Bytes each Frame).

Information-Acknowledge.

When an Information-Transfer-Frame is received and no Information for

the Opposite Station is included, the Receipt will be acknowledged with

an Information-Acknowledge-Frame. The first two Bytes of the Transport-

Control-Block indicates the Circuit of the Node, to which the

Information-Acknowledge-Frame will be sent, in the Sequence-Circuit-

Index - Circuit-ID. the third Byte of the Information-Acknowledge-Frame

is not used and hence will be set on "00". The 4th. Byte of the

Transport-Control-Block is the Receive-Sequence-Number and indicates

the next awaited Sequence-Number from the Opposite Station.

The NAK-Flag will be set, when a received Information-Transfer-Frame

did not have the awaited Send-Sequence-Number. With that the Repeat of

the missing Frame will be demanded.

If the Choke-Flag is set, it means that at first no further Information

can be accepted for this Circuit. The More-Follows-Flag will not be

used and is erased. An Information-Acknowledge-Frame contains no Info-

Part.

DF6LN  (23.7.93); extension to the ON5ZS-Router ON5ZS/DG9FU (9.9.96)

Flexnet-Interface:

------------------

General:

--------

Page 112.

The submitted description, documents the Implementation of the FlexNet-

Gateway-Routines in TNN. With the acquiring of the foundation no

intention exists to copy, to improve or to take over the FlexNet-

Router. The target was and is, to attain a conjunction of the TNN-

Digipeaters and FlexNet-Digipeaters on the System-Level and on the

other side to exchange Network-Information.

Up to now the FlexNet - and Nord-Link Groups could not work out

common Interlink-Protocol. On these grounds we have attempted, the

Interlink-Communication of the FlexNet-Port partially to emulate and

with that to realise a simplified Gateway-Port.

Limitations of the Implementation:

Because the FlexNet-Port is not (openly) documented, we undertake in no

way a Function guarantee. On the above mentioned grounds and what is

more, the intention of a limited Emulation. My choice (and continuing

Information) the Port can be extended at a later date.

The following Functions have bee realised:

- Initialisation,

- Runtime measurement to the Neighbour,

- Answering of the Runtime-Measurement of the Neighbour,

- Evaluation of the Destination-Information.

Not realised are the following Functions:

- Return report to known Destinations,

- Find Destinations,

- Header Compression.

At this time this Version can manage a maximum of 2 FlexNet-Neighbours.

Runtime measurement and Destination-Undertaking follow dynamically. The

receiving FlexNet-Targets will be saved in an extra DESTINATION-Table,

with that it offers no intermixing with NETROM-Nodes. Connect is now

already possible. A further broadening of the Destinations to other

TheNetNode-Nodes under the present Router is not significant. For this,

first of all the TheNetNode-Router must be altered. But priorities are

Targets out of the NODE-LIST !

Because the FlexNet-Nodes cannot send their Destinations VIA a TNN-Node,

for which it entertains a Flexnet Connection (to recognise the not

incorporated "@ behind the Call in the Parameter List), but TNN will

enable the Routings one behind the other, for this the SSID -15 has been

released. Now the FlexNet-Node can realise a Flexnet-Combining to the

TNN-Nodes and via the SSID -15 a VIA-Connection to a further connected

FlexNet-Node.

APPENDIX G:

-----------

The GNU-Version from TNN

------------------------

CONTENTS "The GNU-Version from TNN" \f C \l "2"

Page 113.

Introduction:

------------

Previously under MS-DOS we had to fight always with the available

memory being limited on 1 MB (+ HMA). This memory must accommodate all

Programs. With that, Programs such as TNN were squeezed into a Memory

Area.

With the GNU-Version it is now available for the entire memory to be

used. But why still support MS-DOS, when there are such good Operating

Systems as OS/2, LINUX, or WIN 95 ?

All of these Operating Systems have a legal higher Memory demand,

furthermore is inherited for our purposes an unrequired Interface. For

this reason the further support of MS-DOS is continued.

What does DPMI mean ?

---------------------

The first Processors from the Firm INTEL could address 1 MB of Memory,

at that time it was a multitude. With the Boom of the Computer

Technique it has been recognised quickly, that this Limit has been

exploded quickly. For this reason INTEL implemented a CPU-Operating

Method, the Protected Mode. With this Method on a 286 (at that time) an

unbelievable 16 MB can be addressed. Unfortunately this one was not

correct, consequent with the conversion, so the CPU could be switched

without further ado in the PM (Protected Mode), but for the switching

back into the Real Mode, so it was designated 8086 compatible Mode, was

only possible with great effort. First with the 386s this deficiency

has been rectified. Furthermore the Protected-Mode once again has been

expanded, so that up to 4 GByte (32 Bit Selector and 32 Bit Offset) can

be addressed.

Unfortunately this had no advantages for an Operating System such as MS-

DOS. Through the special Method of the PM it has been given no downward

compatibility. Nevertheless, in order to be able to access above 1 MB

in the Memory, crutches such as LIM-EMS or XMS have been implemented.

With this, the 386s  has been exploited a further Mode of the 386s, the

Virtual-8086-Modes, but on this I will not enter further.

In order to eliminate this Problem, the Dos-Protected-Mode-Interface

has been defined. With that it is possible for the Protected-Mode-

Programs to be achieved under MS-DOS, without being forced to implement

a similar new Operating System.

Then the DPMI-Server switches the Processor (from 386s) always back

into the Real-Mode, when e.g. a Dos-Routine or a BIOS-Call takes place.

After that, the CPU will be reset into the PM. Furthermore the DPMI-

Server places a certain number of Functions for disposal, which eases

the Programming in the PM.

Why Protected ?

---------------

To build Addresses.

-------------------

Page 114.

Actually the Protected-Mode is intended for a Multitasking-System.

Through this Mode the CPU Possibilities offer, to protect individual

TASKS one against another, i.e. a Program cannot access into another.

But this is attained only at a certain cost. A Pointer does not point

at a physical Address, but contains a Selector and an Offset. For the

Generation of a physical Address im PM, it will branch for the Selector

Part of the logical Address, as Index in a Descriptor Table. The Entry

in the Descriptor Table contains a 24-Bit Base Address, to which the

Offset-Part of the logical Address will be added, in order to build the

physical Address.

In the DPMI 32-bit this is similar, certainly will be used there 32-

Bit-Offsets (4 GByte !), which can manufacture an occupation of a 4G-

length-Segment. Now if an application attempts to access Data beyond a

Segment Limit, so it attempts a general Protection violation (GBP or

General Protection fault), hence Protected Mode. In DPMI-Version from

TNN the 16-Bit-DPMI will be employed.

What does it change for the User ?

----------------------------------

At first nothing, the DPMI-Version offers the same Functions as the

"normal" Version. Certainly the System Demand is somewhat higher.

Through switching between PM and Real-Mode a certain speed forfeiture

is to be noticed. Also there should be available  minimum of 3-4 MB-

RAM. Then for DOS-Calls certainly there is more than 400 KByte for

selection and the Buffers can be measured somewhat generously.

Occurring Errors (Exceptions) will be perpetuated on the Hard Disk in

the TEXTCMD-Path and should be forwarded to a NORD><LINK-Service-Place

(e.g. DG9FU,DB2OS or DG1KWA), with that we have the facilities, to make

out the Error.

After all the DPMI-Version tolerates  OS/2 or WIN 3.1x / WIN 95.

better.

LIMITS  / PROBLEMS:

-------------------

Restricts through the construction of the DPMI it gives an entire large

hook. As described above already, the CPU must be running close to and

fro switching running between the Protected Mode and the Real Mode.

Naturally this costs time.

For this reason certain matters are to be observed:

KISSLINK

--------

Only with Fifo, has not much sense without it !!

BAYCOM-USCC

-----------

Restricted through the Construction of the Card. Very many Interrupts

will be generated ! Four 9k6-Links correspond to ca. a 38k4 KISSLINK

without Fifo ! The USCC-Cart admittedly enacts also via a small Fifo,

but if it does it is fragile !

Page 115.

VANESSA

-------

Super for DPMI, because the VANESSA generates no Interrupts !

ETHERNET

--------

Naturally creates IRQ ...as this is critical, I can say...first after

Tests.

I would recommend most warmly a 486DX2-66 or higher.

DEBUG.TXT:

---------

With Crashes  / Exceptions:

---------------------------

If ON THE SCREEN a Debug-Output is generated. The Program "SYMIFY-O

DEBUG.TXT TNN.EXE" should be called IMMEDIATELY FROM TNN17x.EXE, this

evaluates the Stack and generates the actual Debug-Report. Please

forward this to a TNN-Service-Place. After that, the Sysop himself is

responsible for a Reset ! This can be either a Hardware-Watchdog or a

File in the START.BAT which stands after the calling of the TNN-EXE and

SYMIFY, e.g. RESET.COM. Unfortunately up to now, another solution was

not possible !

Possible Exceptions:

--------------------

08 = Double Error,

0A = Invalid TSS,

0B = Segment not available,

0C = Stack-Error,

0D = General Protection violation,

0E = Page Error.

Further Advice for the 32bit-Version of TheNetNode for DOS:

-----------------------------------------------------------

- The TNN174.STA/MHEARD.TAB/CONVERS.PRS of the 32bit-Version differs to

the DOS/DPMI-Versions through the DWORD-Alignment of the GNU-

Compiler.

All Functions of the TNN171 for DOS/DPMI should function just as

with the 32bit-Version.

- At the moment, the 32bit-Version has a very primitive Timing-

Behaviour, a better Timing-Routine is being worked.

- The 32bit-Version generates no Resets but abandons the Program, then

in a Batch-File after the TNN.EXE a Program is called, which executes

the Reset. The "SYMIFY" (s.u.) is to be inserted between TNN.EXE and

RESET.EXE.

Compiler:

---------

As a Compiler the DJGPP/GNU arrives for the operation. This Compiler is

freely available.

Page 116.

In order to simplify the transfer of the DOS-Version onto the GNU-

Version and also to enable a reverse possibility, a STARTCNT.EXE has

been developed. This allows the operation with another TNN174.TNB.  The

STARTCNT.EXE for the access of "outdoors" can stand in the SYSEXE

or/and also in the in the TNN-Directory.

STARTCNT <number>

Places the respective number at Starts (Through running the START.BAT).

The number will be decreased by 1, when the STARTCNT.EXE is called

without <number>.

The following Programs also, are necessary :

SYMIFY.EXE

----------

For the reading out of the Memory with a Program Error.

CWSDPMI.EXE

-----------

The required DPMI-Handler icl. CWSDPMI.DOC, CWSDMP0.EXE CWSPARAM.EXE.

GO32-V2.EXE

-----------

I do not yet know also hi.

RESET.EXE

---------

With that the Software will be restarted immediately.

Now the START.BAT could appear :

set tz=UTC0

set conversd=P:1

set tnn32buffers=9000

set tnnpath=c:\tnn\

set tnncfg=c:\tnn\

set msgpath=c:\tnn\msg\

set confpath=c:\tnn\

set digicall=DB0EAM

set satqth=JO41PI

set qth=9:16:12/51:21:10

set tokencom=0

set kiss1=1

c:\b-log.exe c:\tnn\sysop.pro -m

STARTCNT

IF ERRORLEVEL==1 GOTO OKGNU32

ECHO Start old TNN171...

REM !!!!!!!!!!!!!! OLD SOFT STARTS HERE (TNN171.EXE) !!!!!!!!!!!!!!!!!!

COPY TNN174.DOS TNN174.TNB

TNN_171.exe

REM !!!!!!!!!!!!!!! OLD SOFT STARTS HERE (TNN171.EXE) !!!!!!!!!!!!!!!!!

GOTO ENDE

Page 117.

:OKGNU32

ECHO Start new TNNGNU.EXE

REM !!!!!!!!!!!!!! HERE THE NEW SOFTWARE WILL BE STARTED !!!!!!!!!!!!!!

COPY TNN174.DPI TNN174.TNB

TNNGNU.exe

SYMIFY -o DEBUG.ERR TNNGNU.EXE

TYPE DEBUG.ERR >> C:\TNN\TEXTCMD\DEBUG.TXT

REM !!!!!!!!!!!!!! HERE THE NEW SOFTWARE WILL BE STARTED !!!!!!!!!!!!!!

:END

RESET.EXE

---------

Further questions

-----------------

Then is to be recommended:

- PC Intern,

- Die 386/486'er-CPU,

- http://www.delorie.com.

APPENDIX H:

-----------

-------------------------------------------------------------

D A M A - a new Method for Packet-Radio !

CONTENTS "DAMA - a new Method for Packet-Radio !" \f C \l "2"

-------------------------------------------------------------

from DK4EG Detlef J. Schmidt,

Stenbrechstr. 22,

D-38106 Braunschweig

NORD><LINK e.V.

c/o Peter Guelzow, DB2OS

Str. 5

LAATZEN

Germany

Again, always Packet-Radio-Amateurs have Problems with that, they

supposedly do not arrive at 'their' Digipeater or mine, the Digipeater

will not have understood it, because although it can hear the Digi with

good loud Signals. The conclusion that the receiver of the Digi must be

deaf, appears strongly.

Naturally it presents that case also, but should not be considered

here.

It is possible that the Digipeater has not 'nothing heard', but that it

'heard much too much'. Typically it has the 'best' Location of all

interested Stations. That leads to Collisions by the Digi with more

Stations, which themselves also do not hear one with another ('Hidden-

Stations-Problem'). Only when one of the simultaneous received Signals

arrives distinctly stronger than all others at the Digi, it has got

through by Chance. Then for more distant lying Stations it is often

impossible to penetrate to the Digi with their Packets.

Page 118.

Different Requests have been expressed, in order so solve this Problem

on the Amateur Bands. A Solution is for example the usage of Duplex-

Digis (BTMA). But this System also has a number of Disadvantages. The

Hardware expense of a Duplex-Digis is about several times higher than

with a Simplex-Digi. Furthermore it occupies T W O  Channels, but

brings only the throughput OF ONE collision free Channel. A throughput

increase only appears statistically through Collision Reduction.

But it would be more Elegant, if one could lose the Problem e.g.

through a simple EPROM-Exchange or better still only through other

installed Parameters.

But now this Problem of the Packet-Radio-Amateurs is nothing new. It

occurs also with other Radio Services, which have the same, e.g., when

a Communications Satellite should be addressed from Ships on the high

sea.

An established Method that solves this Collision Problem, is e.g. the

DAMA-Method. It is an abbreviation of: Demand Assigned Multiple Access,

therefore also perhaps, Demand-Related-Multiple-Access (on the

transference Channel. In the Literature the pertinent concept CODAC

instead (Carrier Oriented and Demand Assigned Control). But the concept

is certainly by reason of the phonetic confusion possibility with a

Firm Name has been abandoned.

With the execution of this established DAMA it is approximately as

follows: In a Connection oriented Protocol, at first after the Slotted-

Aloha-Method will be requested, to connect to the Satellite. Collisions

are seldom, but will be taken as the Phase in purchase. After that the

Satellite recognises the Station and includes it in its Polling-List.

Therefore, all Stations will be called in succession from the

Satellites (polled), whereby the Call is simultaneously the

acknowledgement of the received Packets.

Once the Station is connected, it sends its Data Blocks (I-Frames) but

only after an invitation through a Poll. It can send more Frames in a

Block. If the End Node (User) does not Answer immediately within a

predetermined Time, it will be assumed, that the Poll has not arrived,

and then in the next Cycle will immediately repeat the Poll.

The Acknowledgement will be sent after Receipt of User-Data-Blocks (I-

Frames) in the next round with the Poll (therefore when all other

Stations of the Connect-List have been worked). If against that, only

an empty Acknowledgement arrives (Receive Ready/Final), so it will be

passed over in the next round.

With increased loading of the transfer channel an immediately not

active Station can be set down still further in the Poll-Priority, but

immediately achieves the highest Priority again, when it answers with

one (or more) I-Frames.

If the previous Protocol Description is read through, it can be

believed almost, that the AX.25-Level-2-Protocol can be recognised in

there. Certainly lying therein, is the Chance and the possibility of

Page 119.

the DAMA-Method for Amateur-Packet-Radio. AX.25-L2 immediately offers

all Elements, which are required for DAMA. There must not be imported

any new Syntax Elements. Many Functions already allow attainment through

adjustment of the Operating Parameters. The Rest at this Time is only a

minimum alteration of the State Table in the Firmware.

How could a DAMA-Version appear for Amateur-Packet-Radio.

Because already all Syntax Elements are available, nothing new must be

imported and defined. We remain therefore, in the hitherto Terminology

of AX.25 for the description.

The various Phases of the Protocol should be considered here

individually.

Connection Construction.

------------------------

If an End-Node is to be connected via a Digi, so the new Station will

be included directly in the Poll-List and polled with SABM-Frames. If

following within a default Number of SABMs, no UA of the requested End-

Node arrives, the Station will be struck out of the Poll-List again. If

the Initiative goes out returned from the End-Nodes, as previously in

the CSMA-Method an SABM to the Net-Node will be sent. In this Phase

Collisions are possible, it is a possible necessity, to send further

SABMs after a defaulted Timeout as previously, up to the Net-Node

answers with a UA.

With that the End-Node in the Polling-List (User-List with TheNet) is

entered and from now on this will be coordinated. The End-Node answers

with a RR count 0 as Signal to the Digi, that its UA has been entered.

Idle

----

So long as no Information-Transfer between Net- and End-Node takes

place (Idle), the Net-Node sends its Polls with RR and the attached

actual Count. If an RR with the attached Count, arrives as an Answer of

the End-Node to the Digi, the Time to the next Poll will be extended to

the same End-Node, in order not to burden the Channel unnecessarily

with Polls. If the RR of the End-Node is not received by the Digi

(because either the Poll or the Answer disappears), then the same End-

Node will be polled again immediately with the next pass. (Therefore

when all other Uplink-Stations have been worked once)

If after a default Number of Polls no answer has been received, the

End-Node can be erased out of the Poll-List. At first sight, this

Method only places more Transfer-Effort than in the earlier Version,

because now also the Opposite Station is routinely polled and some time

or other will be appended. The difference is only, that in the newer

Version, the End-Node will be struck out of the User-List, if it no

longer answers, with a constant number of attempts.

User-Data-Transfer  Net-Node - End-Node

---------------------------------------

Page 120.

This case differentiates itself in now way from the 'normal' CSMA-

Method. Because the Initiative lies moreover with the Digi (Master),

also it can send to the End-Node (Slave) at each time one or more I-

Frames instead of a Poll. The End-Node acknowledges as received with RR

and the attached Count, but also, it can acknowledge directly with I-

Frames and the corresponding Counts. The application of the Poll-

/Final-Bits remain unchanged.

User-Data-Transfer  End-Node - Net-Node

---------------------------------------

As already outlined previously, the Net-Node sends in succession Polls

to all in the User List listed End-Nodes. The End-Node waits therefore,

until it is polled from the Digi with an RR or its one I-Frame and then

answers immediately with one or more I-Frames to the Net-Node. This

waiting for a Poll makes the decisive Aspect of the Collision

prevention contrary to the earlier Method, with the multi End-Nodes can

be sent quasi simultaneously, because they do not hear one another an

not coordinated.

Additionally the Problem will be shifted, that two End-Nodes send,

which they themselves actually could hear, but in the respective

individual Dead Time between the switching  off of the Receiver and to

the transmission of the modulated Carrier which the other does not

'hear'. This case is not so rare, because different send ready Stations

will be synchronised with each other from the Carrier of the

Digi.

The Digi will not acknowledge I-Frames immediately, but first in the

next Round, after all other Stations on the List have been served. Then

this is also at the same time as the next Poll. If no I-Frames are

queued to be sent, again from the End-Node will be sent only an RR with

Count.

Connection Construction

-----------------------

Again it is the same Syntax Elements for the operation. If the Net-Node

(Digi) throws out the End-Node (User), so it sends a DISC-Frame as

before. The End-Node answers it immediately with a UA (Final Bit

set). If the UA gets lost, the End-Node answers with a next arriving

DISC-Frame as previously with a DM-Frame. If in the returned case the

End-Node 'exits', so it awaits the next Poll and consequently sends its

DISC-Frame to the Net-Node. In this Case it is irrelevant, whether the

Digi acknowledges with UA immediately, or first in the next Round.

UI-Frames

---------

A certain special position takes in UI-Frames (Unnumbered Information),

both in the normal CSMA-Method as also in the DAMA-CSMA-Method.

Originally they have been created for, an Information Exchange to be

realised outside of the normal Protocol execution.

If UI-Frames are to be sent from the End-Node, normally this can appear

only outside of the Operation with the Digi. Actually these Cases

Page 121.

should not occur, because in these cases the Direct-QSO should not be

operated on the Digi-Entry-Frequency. UI-Frames of the Digi demonstrate

moreover, again no Problem, because it will be heard by all Stations on

the Frequency.

Miscellaneous Protocol Elements

-------------------------------

Consequently completely described, an idealised Session of the

Connection Construction up to dismantling.

It will not display all AX.25 possible Protocol Elements. But also,

it is not completely necessary in this Context, because similar

Elements retain their logical meaning. DM, RNR, REJ will be used in the

same sense as previously. The difference to the pure CSMA-Version is

exclusively, that the End-Node it sends as the other Protocol Elements

only after a Poll of the Digi.

Net-Node-Side, the only difference that is makes, is that these

corresponding Frames will not be sent immediately or at any time, but

always first, when all other registered Stations (in the same sense)

have been worked once, and which should be addressed exactly in the

Frame addressed End-Node.

Agreement of the Protocol Versions

----------------------------------

A possible importation of the DAMA-CSMA-Method could execute

continuously. Naturally the throughput will be so much larger,

certainly as more Stations enter on the new Method. But some Stations,

which do not enter the new Channel-Access-Method, can contribute

partially to the increase in throughput, in that they install their

Parameters otherwise. For that purpose the Delay between Receipt of one

of its corrected Frames and the 'belonging to' Acknowledgement (T2 or

DWAIT) will be set on a Minimum. (less than 1 sec).

The Time up to the Poll being sent out on an individual outgoing I-

Frame (in order to 'paint on' an Acknowledgement) will be set on a

relatively high Value, the distinctly larger than normal is the

chronological distance apart between two Polls of the Net-Node.

Both sides of the new Protocol Version must operate up to the full

possibilities of the Method being ladled out. For this purpose the End-

Node-Side must be informed anyway, that now it has to install itself on

the new Protocol-Variant. More Methods offered:

- Importation of an UPLINK-Command additional to the CONNECT-Command;

- Importation of a Channel Specific Parameters in the TNC, of the

Protocol-Variant control;

- Automatic recognition of the Protocol-Variant by way of Protocol-

Identifier-Bytes from the Net-Node;

- Limited Usage of the Method through alteration of the TNC-Timer (as

previously outlined);

- Naturally beside that also is still possible, to display the

importation of a new Protocol-Elements, with which the respective

Variant will be controlled. That e.g. as with X.25 the Function of an

Page 122.

SABM-Frame. Furthermore, with that Compatibility will be ensured

to the earlier Protocol, because the 'old' Protocol-Versions should

not react on unknown Frames or with FRMR.

Summary

The outlined Method can raise the throughput distinctly on an AX.25-

Channel. It has the decisive advantages, that it gives no Net crashes

through over loading of the Channel. The transfer capacity of the

Channel rises continually up to the Maximum, it gives no tip over

point as with pure CSMA, by which the Net-Transfer-Rate again accepts

above a certain threshold (ca. 65 to 85 % with pure CSMA).

Only with minor Channel Loading more transfers will be required with

DAMA (Overhead). But this does not disturb any more, so long as the

Channel Capacity will not be scooped out. With increased Channel

loading the Overhead-Part then goes further back up to the minimum

Value of the ideal CSMA-Method without Collisions.

Also as determined 'social' Components give DAMA distant Stations the

possibility, to work with stability via the Digi, without being 'howled

down' from the near lying Stations.

It is not required, that all participating Stations change

simultaneously on to the new Protocol-Variant. All Protocol-Elements

retain their original Meaning, so that the End-Node can use the old and

the new Variants in parallel. With that the Channel throughput will be

better, certainly the more Stations partake of the new Variant.

Literature

div.          : Normblatt X.25 CCITT.

div.          : Transaction on communication IEEE.

Fox, T.       : AX.25 level 2 protocol specification AMRAD.

Kauffels, F.J.: Lokale Netze . R.-Mueller-Verlag.

Schmidt, D.J. : Synchrone DFSS-Protokolle mit 6809-Micro-Computern in

heterogenen Sternnetzen. TU-Script BS'81.

Tanenbaum, A. : Computer Networks. Prentice Hall Verlag.

Schmidt,D.J.  : DAMA, ein neues Verfahren fuer Packet-Radio cq-DL, 4/89

Glossary

DM   : Disconnect Mode

DISC : Disconnect Frame

FRMR : Frame Reject

I    : Information Frame

REJ  : Reject Frame

RNR  : Receive Not Ready

RR   : Receive Ready

SABM : Set Asynchronous Balanced Mode

SARM : Set Asynchronous Response Mode

UA   : Unnumbered Acknowledge

UI   : Unnumbered Information Frame

BTMA : Busy Tone Multiple Access

CSMA : Carrier Sense Multiple Access

Connection oriented protocol:

Page 123.

All Nodes on the route know all Stations which the route uses, at

least short time. Contrary to this, stands the Protocol without

connection, by which all Data-Packets will simply reach further (also

e.g. simple  Level-2-Digipeats with PR). Connection oriented Protocols

are typically more expensive in the Realisation and require more

Computer accomplishment in the Net-Node, but instead they are

considered special on Transfer Channels with less Capacity, because not

in every Frame the entire Routing-Information must be transported, but

only once during the Connection Construction.

Definition of the DAMA-Slave from DG2FEF

----------------------------------------

In the following here at the sharp end the Definition of the DAMA-

Slave, as it is implemented at the time in TF2.7. This Definition

in future should be obligatory for the Programming of Layer-2-Software,

in order to create a uniform Standard, on which Basis the Operating

Type DAMA and OPTIMA (DAMA fo Duplex-Digis) can be developed further.

1.)

As Master-Poll (resp. DAMA-Poll) is addressed to the Slave Frame to

understand, in whose Address-Field at the same time as the DAMA-Bit,

the Poll/Final bit and the Command/Response-Bit is set.

2.)

In the Operation Condition "DAMA-Slave" a Station should change, when

it receives a Frame addressed to itself with set DAMA-Bit. As DAMA-

Slave it should run a Global Timer, which is installed with a suitable

Value (the recommended Minimum Value amounts to 4 Minutes). The Timer

should be initialised with each of its adjusted Frame with set DAMA-Bit

again on the Start Value. The Station should leave the Operation

Condition "DAMA-Slave", when either the global DAMA-Timeout runs out,

or it has disintegrated Connections on the Channel of the DAMA-Master.

3.)

The Slave may never radiate a Frame for a QSO with or via the Master,

at the same time as the Poll/Final- and the Command/Response-Bit are

set. This is valid for so called "Unproto-Polls".

4.)

It may give no motivating Actions from the Slave. Every Action of the

Slave may be only as a Reaction to the Master-Poll and which its

transmitted Information follows. A Reaction on the T3 is admittedly,

also in this connection the sending out of a Poll, however basically

the T3 should be installed on a Time not smaller than 3 Minutes. Also

this Parameter should not be configurable for the User.

An exception builds the Construction or Destruction of a new Link,

here naturally, is to be found instead one from a Slave motivated

Action. The T1 in this Case may be used as Criteria for the creation

of an SABM or DISC, the sending out must follow inside the Reaction to

a Master-Poll.

Page 124.

5.)

The Reaction of the Slave to a Master-Poll has to follow immediately

and without consideration of the DCD.

6.)

The Reaction of the Slave to a Master-Poll must exist from an

individual connected Send. A "Flicker" of the Sender may not occur

under any circumstance.

7.)

With Multi-Connect-Connections, the Slave with a DAMA-Poll sends out on

an arbitrary Link, all queued, unsent Frames of all existing Links.

Already once sent Frames of a definite Link will be repeated only

against that, when this Link is explicitly polled.

APPENDIX I:

-----------

TCP/IP and TNN

--------------

A Guidance for the adjustment of the Routes from TheNetNode Version V

1.73

It has indicated, that many of TNN Digis previously had do do nothing

or little with TCP/IP. On these grounds, here I attempt to write a

little Guidance with Examples.

There should be no grounds for the TCP/IP Protocol to intercede, but it

should only serve for the purpose to configure a TCP/IP modestly

correct. The TCP/IP basics are to be read at another place.

IP Address:

-----------

At first the Digi must receive an IP Address. IP Addresses are issued

worldwide, i.e. an IP Address is only available ONCE in the World,

in order to be unique. Therefore you cannot simply choose an address,

but must obtain one from an IP Address Supplier. Brian Kantor (Call

Sign not immediately to hand) creates worldwide. Furthermore, there are

more IP Address Suppliers in every Country. In DL it is Ralf Kloth

dl4ta who has again, further Address Suppliers, who supply the

individual Regions. For the Hamburg Area it is e.g. Eddi dk3uz.

Otherwise Brian Kantor has the entire 44 Net and must pay for that ! I

do not know how many, but such Nets are restricted; that only for short

Info.

It makes an exception:

44.128.0.0 - 44.128.255.255 are Addresses which may be employed for

testing, then these Addresses will not be shared officially. How does

an IP Address appear ? Example from db0hht.ampr.org:

44.130.0.100

|   | |  +---- the 100 is the Address of db0hht.ampr.org

|   | +------- the 0 stands for the Net 0

|   +--------- the 130 stands for DL

+------------- the 44 stands for the Domian ampr.org

Page 125.

In this Example it differentiates the Addresses on one Point, i.e. in

one there stands the 0 for Net 0 and in one the 150 for Net 150!

The IP Address will be installed now by TNN with the Command:

IPA <Address>

Installation of the tcp/ip Routes:

As we now obtain the IP Packets of e.g. db0nds will be routed out

correctly in the Net 0.

For the Net 0 db0hht.ampr.org is the Gateway, i.e. all Packets should

be sent via db0hht.ampr.org and admittedly the Addresses 44.130.0.0 to

44.130.0.255, also the 255 Addresses for the corresponding Net. Then

DB0HHT.ampr.org forwards these Addresses correctly onto the Entry.

Also, we must also install a so called Cluster Routing and then we can

e.g. make with db0nds:

At first the Route to db0hht.ampr,org will be installed: (44.130.0.100)

IPR 44.130.0.100 + NET/ROM

Then all Addresses of the Net 0 via db0hht.ampr.org will be routed:

IPR 44.130.0.0/24 + NET/ROM 44.130.0.100

Now all Computers in the Net 0 can be reached from db0nds via

db0hht.ampr.org. When there would be nothing more.

From where does db0nds know that db0hht can be reached with AX.25?

For that the ARP Entry is planned.

ARP 44.130.0.100 + NET/ROM db0hht

With that the Software will be informed that TCP/IP Packets, which

have the Address 44.130.0.100, are sent to the AX25 Address db0hht per

Netrom!

Namely, here we must differentiate accurately between "Hostname" and

"AX525-Addressee"  ! That is an entirely important Point, the

pleasureable confusion established:

it can e.g. be the following:

Hostname : dk3hg.ampr.org

AX25-Call: dk3hg

Username : dk3hg

Here, always appears this "dk3hg", but all three Names, which admittedly

are called the same, but have a different meanings !!

Page 126.

Now a Word for the "routes":

----------------------------

It makes little sense to route individual Hosts, without employing the

Cluster-Routing, for that is achieved only this ONE Hostname and

nothing more. But that is not the meaning of a Router, as TNN173

displayed it.

Furthermore the Sysops themselves should agree with each other "who,

how, what routes, with that it does not arrive at confusion. A good

beginning strictly speaking, is to apply always to the Sysop of the

original TCP/IP Digi, when is comes to such a question.

An entirely special advantage I wish to expose. Previously, before

everything it was for the Northern (Netromland) to make very

problematic TCP/IP in the South (Flexnetland), because the Digi in the

Flexnetland always must have to send its Broadcast via Flexnet to the

North. Now that can be bypassed, if the TNN, is connected to Flexnet,

their routes have been installed. Then it is possible to reach from

e.g. to reach db0hht from all other possible TCP/IP Digis, which ONLY

Flexnet makes.

I myself wish, to thank for these Paths also with Frank dg3aah, who

with much patience with db0nds has installed the routes for us.

If you would like to know which Sub-Net-Coordinator is appropriate for

his Region, can look in the Boxes from "Subnet_Koordinator_Liste".

There, all Coordinators for DL are listed. Also, I will send this List

with pleasure by demand per mail, if it cannot be found in the Boxes.

If you have no Sub-Coordinator in your Region, then you must apply to

Ralf dl4ta, in order obtain to a new Sub-Net. Simultaneously an OM in

this new Region should undertake the administration of this Net. Then

it is advocated to take an IP Address for the TNN Router begins with a

1, as e.g.:

44.130.news_Net.1.

That has the advantage that by choice, this Net can be divided yet

further.

An actual List of the IP Addresses is distributed by db0hht or WAMPES

is to be found by other UNIX Digis, in the Directory  /tcp. actually

from each Sysop more or less :-). Who wishes to have the entire

worldwide List and has an Internet connection, can take a look around

ftp://ftp.ucsd.edu/hamradio.

In Appendix  1 is to be found the apportionment of a Net in further

Sub-Nets. Im Appendix 2 is an incomplete Example as we have installed

with db0nds.

Appendix 1

----------

A Net can apportioned as e.g. 44.130.0.0 to 44.130.0.255 still further

into individual Sub-Nets. Then that appears e.g. so:

Page 127.

Netmask    /Bits       Addresses        Hosts

=======================================================================

44.130.0.0/24           256             254

44.130.0.128/25         128             126

44.130.0.192/26         64              62

44.130.0.224/27         32              30

44.130.0.240/28         16              14

44.130.0.248/29         8               6

44.130.0.252/30         4               2

-----------------------------------------------------------------------

Appendix 2

ipa 44.130.150.100

ipr 44.130.0.100   + netrom

arp 44.130.0.100   + db0hht

ipr 44.130.0.0/24  + netrom    44.130.0.100

ipr 44.130.82.101  + Ruhner-B.

arp 44.130.82.101  + Ruhner-B. VC db0oca-11 db0lwl-2

ipr 44.130.83.0/24 + Ruhner-B. 44.130.82.101

ipr 44.130.20.0/24 + Ruhner-B. 44.130.82.101

ipr 44.130.42.0/24 + Ruhner-B. 44.130.82.101

ipr 44.130.56.0/24 + Ruhner-B. 44.130.82.101

ipr 44.130.60.0/24 + Ruhner-B. 44.130.82.101

ipr 44.130.80.0/24 + Ruhner-B. 44.130.82.101

ipr 44.130.88.0/24 + Ruhner-B. 44.130.82.101

ipr 44.130.91.0/24 + Ruhner-B. 44.130.82.101

Will be expanded still ......, should serve only as inspiration :

---------------------------------------------------------------------------

With Questions, Comments, Improvement recommendations etc. I request by

mail to me. Michael (dk3hg@b0hht)

LICENCE:

--------

General Licence for Amateur Radio Software (ALAS)

-------------------------------------------------

Copyright(C) 1992 Nord><Link e.V.

c/o Lutz Schindler

Bechtsbuetteler Weg 18

38110 Braunschweig.

This Document may be copied and distributed as long as it is not

altered. For inspirations, to improve it, I am thankful.

1.

Forward:

--------

This Licence arose from the General Public Licence of the Free Software

Foundation (GPL). I have attempted, to retain the sense and bring more

clarity to it. Some passages have been substantially removed. But it is

left to each User's option, to employ instead of this Licence, the GPL

conditions.

Page 128.

The sense of this Licence is to protect the Author and the user of the

Software. However there are some necessary restrictions, and also it

brings certain duties for which. those who with this Licence distribute

or alter the incorporated Software.

This will be achieved from that, the Software is protected by Copyright

and the User obtains through this Licence, the Possibility of a near

unrestricted Usage.

2.

Range of Validity

-----------------

This Licence is valid for every Program or Part of a Program Package,

that outputs a Copyright-Notice, which covers itself on the Licence. In

the following, "Program" means either the Program or a Part thereof.

"You" are the Licencee.

3.

Your Rights:

------------

You may use or copy or distribute or alter the Program, so long as with

that, you attach no Commercial intentions.

4.

Your Duties:

------------

4.1.

You may not alter the Copyright Entry and the advice of this Licence,

and with every Start of the Program, it must be visible for the User.

4.2.

To each third party, that you give a Copy of the Program, the

same Rights, which you yourself have received. You must impose on him

the same duties.

4.3.

For any distribution, you may not ask for any money, other than the

cost for the Medium and Postage.

4.4.

You may only distribute the complete Program, as you have received it.

4.5.

If you alter the Program or parts of it for you own work application

- literal or altered

- so the following points are valid for the arising new Program:

4.5.1.

You must clearly indicate:

--------------------------

- You Name and your Address

- and the fact, that you have altered the Program Application or Parts

of the Program.

Page 129.

4.5.2.

You must distribute the Program either with the complete Source Text or

on demand, a Copy of the Source Text against a fee of a maximum of the

Cost of the Medium and the Postage.

4.5.3.

You may declare no Limitations, which exceed this Licence.

5.

Miscellaneous

-------------

You obtain the Program without any Guarantee for Function, Error Free

or Practical Usefulness for a definite purpose. You renounce any Damage

claim, no matter on what Grounds.  With the Usage of the Program you

recognise these Licence conditions without reservation.

Vers. 1.0, 13-OCT-92

--------------------

Ende ALAS

---------

CONCLUSION:

-----------

What must be defined precisely, will be the demonstrated (hopefully)

Questions. This File, I know now already, will be altered frequently

and must be completed.

With friendly Greetings

Nord><Link e.V.

c/o Lutz Schindler

Bechtsbuetteler Weg 18

38110 Braunschweig.

Print out (from the Laser Printer) is obtained against expense

reimbursement of DM 0,08/Side + Postage by:

Henning Happe                Lead by

Sechsaecker 20               Henning DG9FU

34246 Vellmar.

Contributions from

Peter DB2OS   Andreas DB7KG   Georg DF2AU   Nils DF6LN   Klaus DF7BZ

Andreas DG1KWA   Frank DG3AAH   Bernd DG8BR   Odo DL1XAO   Kurt DL2LAY

Reiner L5HCD   Matthias DL9HCJ   Peter HB9PAE   Daniel ON5ZS.

My special thanks go to DF6QQ. Uli has assumed a labourious small job

of the punctuation and also still eliminate certain small writing

Errors!

Thus Issue has been Proof Read by:        (German Version Only !)

Klaus DF7BZ.

TheNetNode 1.74 (c) NORD><LINK e.V.

10. September 1997.